- SCHEDULE  B

- Name of work:Repair/Renovation of Government Boys Primary School in Mevo Laghari.Tal: Khangarih.

| - S:NO: | - Description | - QTY | - RATE | - UNIT | - AMOUNT |
| --- | --- | --- | --- | --- | --- |
| - 01 | - 02 | - 03 | - 04 | - 05 | - 06 |

- 01.Removing cement plaster or lime plaster.(S:NO:53 P:NO:13)

- Total:	=1526.0sft;x	121/=	p%sft	Rs:1846/=

- 02. . Dismentling of C C ratio 1:3:6(S:NO:19b P:NO10)

- Total:	264.0 cft x	1306/80p%cft	Rs:3450/=

- 03 .Excavation in foundation of Building bridges and other                                                                             structure dog belling dressing refilling around structure with                                                                                      excavated earth lead up to one chain and lift up

to 5’(S:I:NO:18bPNO:4 )			Total:	=420.0cftx	3176/25p%ocftRs:1334/=

- 04.Cement concrete brick or stone ballast 11/2”x2” ratio 1:5:10 (S:I:NO:

Total:	=72.0cftx8694/95 p%cft	Rs:6260/=

05. Pacca brick work in foundation with cement sand                                                                                      mortar 1:6(S:I:NO:4e P:NO21)

Total:	=137.0cft x11948/36p%cft	Rs:16369/=

06. .Pacca Brick work in Ground Floor with cement  sand mortor (1:6)

Total:	181.0cft x12674/36 p%cft	Rs:22940/=

07. Applying Floating coat of  1/32” thick.(S:NO:14 P:NO53)

Total:1526.0sft x660/= p%sft		Rs:10072/=

08 Cement plaster (1:4) with cement sand mortor 3/4” thick                                                                              .(S:NO:11c P:NO:52)

Total:1526.0sft x3015/76p%sft	Rs:46020/=

09. Reinforced cement concrete work i/c all labor and                                                                                      material Except the cost of steel for its bending and                                                                                                             binding which will be paid separate . This rate also i/c                                                                                                          all kinds of forms moulds lifting shuttering curing and                                                                                                            rendering the exposed surface .RC work in roof slab                                                                                                          beams columns rafts lintels laid in position complete                                                                                                     in all respects.(S:I:NO:6A P:NO:16)

=15.0cft x337/= pcft		Rs:5055/=

10. Fabrication of mild steel reinforcement concrete i/c cutting                                                                                        bending and laying in position making joints and fastening i/c                                                                                         cost os steel wire also i/c the removal rust from bars.(S:I:NO:8a P:NO:17)

0.535cwt	atRs:5001/70pcwt	Rs:2676/=	 11Prov/Fixing.G:I frame an chowckets of size 7”x2” or                                                                                              4 ½”x3” for door using 20 guage. GI sheet i/c welded                                                                                       hinges and fixing at site with necessary hold fast .filling                                                                                          with cement and sand slurry of ratio 1:6 and repairing the                                                                                           job .The cost also i/c all carriages ,tools and plants used in                                                                                           making and fixing.(SI:NO:29 P:NO:93).

Total:	=85.0rft x228/90prft		Rs:19457/=

12. Prov/Fixing.G:I frame an chowckets of size 7”x2” or 4 ½”x3”               	                                                      fr window using 20 guage. GI sheet i/c welded hinges and fixing                                                                     at site with necessary hold fast .filling with cement and sand slurry                                                                     of ratio 1:6 and repairing the job .The cost also i/c all carriages ,tools                                                                        and plants used in making and fixing.(SI:NO:28 P:NO:93).

Total:	=134.0rf x240/50prft		Rs:32227/=

13. First class deodar wood wrought ,joinery in doors                                                                                         and windows etc fixed in position i/c chowckets hold fasts                                                                                                ,hinges iron tower, bolts ,cleats handles and cards with hooks                                                                                      etc.(only shutters)1x3/4” thick.(S:I:NO:7bP:NO:58)

=184.0 sftx902/93 psft		Rs:166139/=

14. Supplying and fixing Iron steel GRILL ¾”position flate                                                                                      iron of approved design i/c painting3coats etc (weight not less                                                                                       than 3.7lbs of finished griLL(S:I:NO:26P:NO9)

=295.0sftx180/50psft		Rs:53248//=

15. Filling watering and ramming earth under floor                                                                                               With new earth Excavated from out side lead up to                                                                                             one chain and lift up to 5’(S:I:NO:32P:NO:4)

=11475.0cft x3630/=p%ocft	Rs:41654/=

16. .Add Extra Lead for (3) miles.

=11475.0cft x579/41 P%cft	Rs:66505/=

17. Cement Plaster (1:6) up to 12’ height1/2”thick.(S:I:NO:13b P:NO:52)

= 986.0 sftx2206/60p%sft	Rs:21757/=

18. Cement Plaster (1:4) up to 12’ height3/8” (S:I:NO:11c P:NO:52)

=986.0sftx2197/52p%sft 	Rs:21668/=

19. Providing and laying 1” thick topping cement concrete (1:2:4)

Including surface finishing and deviding in to panels (d)3”thick

(SI:NO:16 P:NO:42)

=1559.0sft x4411/82 p%sft	Rs:68780/

20. . Cement Pointing struct joints on wall  1:3 (S:NO:19b P:NO:53)

=516.0sft x1287/44 p%sft	Rs:6643/

21. Reinfor cement concrete concrete spouts including fixing

In position 2’x1/2”x6” (S:I:NO:14 P:NO:18)

=5.0 NOx261/25 			Rs:1306/=

22. Making Notice Board made with cement.(S:I;NO       P:NO:     )

=96.0 sft x58/11 psft			Rs:5579/=

23. White wash 3 coats.(S:I:NO:26cPNO:54)

=1560.0sft x829/95 p%sft		Rs:12947/=

24. Primary Coat of chalk under Distempring.(S:I:NO:23 P:NO:54)

=2306.0sftx442.75 p%sft		Rs:10210/=

25. Distempring of two coats.(S:I:NO:24cP:NO54)

Total:	=4985.0sft x1043.90 p%sft		Rs:52038/=

26. Preparing new surface Painting doors and windows any type

i/c edge.New surface(S:I:NO:4cP:NO:68)

=430.0sft x2116.41p%sft		Rs:9100/=

27. Colour washing two coats.(S:I:NO:25bP:NO:54)

=2036.0sft x859/40 p%sft		Rs:17497/=	            																			           		__________________

Total:	 722651/=

________% above/below on the rates of CSR.

CONTRACTOR

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI