PART “B” WATER SUPPLY & SANITERY FITTING

NAME OF WORK : 	(ECE) INTERVENTION SUKKUR DIVISION ADP NO. 154 (2016-17 PROGRAMME AT GBPS: ALI ABAD (SemisCode.41505482) U/C N.F.JALLALANI TALUKA KOTDIJI DISTRICT KHAIRPUR

| S.NO: | ITEM OF WORK | QTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

1.      S/F squirting type white glazed earth were W C pan with internal fitting and flush papers and making required and making good CC i/c fixing in position etc .(S.I No.(B) – P-1)									2 No  		4802/60	Each 	            9605/=

2.        Providing and fixing 24x18 lav: basin in white glazed earth in were complete with and i/c

the cost of W.I or C.I cantilever brackets 6” builts into walls painted white in two coats

after the primary coat of red lead paint (S.I No 10  P-3).

1 No		4928/-		Each	            4928/=

3.	Providing & fixing steel sinks stainless local make complete with cast iron or wraught iron brackets 6 inches built in wall, 1-1/2" c.p bubber plug chrome plated brass chain, 1-1/2" c.p brass waste, with 1-1/2" P.V.C. waste pipe & making requisite number of holes in wall & plinth & floor for pipe connection & making good in cement concrete 1 : 2: 4.

1 No		5162/30	Each	            5162/=

4.	through walls and roof making good etc painting two coats after clearing the pipe etc.

(S.I No.1 –  P-12)

½ ” dia			70.0		73/21		P.Rft	           5125/=

¾” dia  			70.0		95/79		P.Rft	            6705/=

1” dia  			40.0		128/55		P.Rft	            5142/=

5.      	 S/F long bib cock 1/2” dia of superior quality (S.I No.2ii – P-16)

4 No 		1109/46	P.No	            4438/=

6.      	 S/F C.P cock 1/2” dia of superior quality (S.I No.1– P-16)

4 No 		475/28		P.No	            951/=

7.	Providing & fixing C.I plain dia branch of the required degree including extra painting to match the colour of the building plain tee(S.I No.1– P-16)

2 No 		704/20		P.No	            1408/=

8.	S/F fiber glass tank of approved quality and design and wall thickness specified i/c  nuts and     builts and fixing in plate form of CC 1:6 and making ( 250) gallon  (S.I No.3 a – P-21).

1 No                 21989/0            P.No	            21990/=

9.	RCC pipe with collars class “B “ digging the trench of the required etc and fixing in position i/c cutting and fitting with makes phalate and cement mortor 1:1 and testing with water pressure  (S.I No.  02 – P-  24  ).

20.0		199/25		P.Rft	            3985/=

10.	P/F 6”x2” or 6”x3” C.I floor trap of the approved selt cleaning design with a C.I screwed down grating with or without vent arm complete with & i/c making requisite number of holes in wall plinth for pipe connection & making good cement concrete.(S.I.No 20/P-6)

2.0		2024/43	P.No.	            4049/=

11.     	Providing and fixing in position nylon connection complete with ½ dia brass stop cock with

Pair of brass nuts and lining joints to nylon connection. (S.I No.23 P- 06).

3 No		447/15		Each		1341/=

12.	Providing & fixing  4"  x 4" dia C.I branch of the required degree with access doors, rubber washer 3/8" thick and bolts and nuts and extra painting to match the colour of the building. (S.i.No: 23 P-06)

2 No		702/0		P.No		1404/=

13.	P/F full way gum metal balves with wheels threaded are flabged & with rubber wishers standared patterns (S.I No: 4  P- 17).

2 No		271/90		Each		544/=

14.	P/F of pistom pump local made single phase 220 Volts A.C Motor i/c fixing in position etc (R/A)

1 No		14800/0	Each		14800/=

____________

Total Rs:  86775/=

Amount Total (A)

___________________% Above/Below on the rates of CSR Rs.___________/- Amount to be added / deducted

on the basis of premium quoted			Total (B)

Total (A) = A + B in Words & Figuure___________________________________________________________

__________________________________________________________________________________________

CONTRACTOR								EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

KHAIRPUR