BILL OF QUANTITIES/ SCHEDULE “B”

Description & Rate of items based on Schedule/ Market (offered Rates).

- REPAIR/RENOVATION OF EXISTING BUILDING OF REGIONAL DIRECTOR BUREAU OF STATISTICS SUKKUR OCCUPIED BY NAB OFFICE (PROVIDING INTERNAL ELECTRIFICATION ) ADDITIONAL ROOMS, OFFICE BLOCK, FIRST FLOOR, ADDITIONAL ROOMS OF BUNGALOWS (OFFICES) CAR PORCH SHADE, OFFICERS BUNGALOW 3 NOS , DAY TO DAY COMPLAINTS (STAFF QUARTERS).

| S.NO: | QTY: | . NAME OF ITEM. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  |  | PART “A” NEW SCHEDULE ITEMS 2012 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | 220 | Wiring for light or fan point with 3/.029 PVC insulated wire in 20mm (3/4”) channel patti on surface as required (S.Mo.124 P.15). | 1130 | Per point | 248600/- |
| 2. | 70 | Wiring for plug point with 3/.029 PVC insulated wire in 20mm (3/4”) channel patti on surface as required (S.I.No.126 P-No.15) | 985 | Per point | 68950/- |
| 3. | 28 | Wiring for call bell point with 3/.029 PVC insulated wire in 20mm (3/4”) PVC conduit recessed in the wall or column as required (S.I.No.126 P-No.15) | 1764 | Per point | 49392/- |
| 4. | 50 | Providing & laying (MAIN or SUB MAIN) PVC insulated with size 2-7/.029 copper conductor in ¾” dia PVC conduit on surface (S.I.No.8 P-No.2) | 173 | Per Mtr. | 8650/- |
| 5. | 1800 | Providing & laying (MAIN or SUB MAIN) PVC insulated with size 2-7/.029 copper conductor in ¾” dia PVC conduit on surface (S.I.No.10 P-No.2) | 222 | Per Mtr. | 399600/- |
| 6. | 1500 | Providing & laying (MAIN or SUB MAIN) PVC insulated with size 2-7/.036 copper conductor in ¾” dia PVC conduit on surface (S.I.No.11 P-No.2) | 252 | Per Mtr. | 378000/- |
| 7. | 1200 | Providing & laying (MAIN or SUB MAIN) PVC insulated with size 2-7/.044 (6mm2) copper conductor in ¾” dia PVC conduit recessed in the wall or column as required (S.I.No.10 P-No.2) | 341 | Per Mtr. | 409200/- |
| 8. | 600 | Providing & laying (MAIN or SUB MAIN) PVC insulated with size 2-7/.052 (10mm2) copper conductor in 1” dia PVC conduit on surface (S.I.No.5 P-No.1) | 463 | Per Mtr. | 277800/- |
| 9. | 80 | Providing & fixing circuit breaker 6,10,15,20,30,40,50 & 63amp SP (TB-5S) on prepare board as required (S.I.No.202 P-No.30) | 916 | P.No. | 73280/- |
| 10. | 08 | Providing & fixing circuit breaker 3,5,10,150,20 & 30amp TP (XS-30NS{NB}) on prepare board as required (S.I.No.205 P-No.31) | 5301 | P.No. | 42408/- |
| 11. | 04 | 206 Providing & fixing circuit breaker 15,20,30,40,50 & 60amp TP (XE100CS{CB}) on prepare board as required (S.I.No.206 P-No.31) | 5521 | P.No. | 22084/- |
| 12. | 150 | P/F (MAIN or SUB MAIN) PVC insulated with 4 core copper conductor 600-1000 volts size 35mm2 (S.I.No.104 P-No.12) | 2529 | Each | 379350/- |
| 13. | 04 | Providing & fixing circuit breaker 15,20,30,40,50,60,75 & 10amp TP (XS100NS) on prepare board as required (S.I.No.202 P-No.30) | 9261 | P.No. | 37044/- |
| 14. | 70 | Providing & fixing Bakelite ceiling rose with two terminals (S.I.No.228 P-No.33) | 72 | P.No. | 5040/- |
| 15. | 78 | Providing & fixing Brass Battern holder c(S.I.No.232 P-No.33) | 70 | P.No. | 5460/- |
| 16. | 90 | Providing & fixing (MAIN or SUB MAIN) PVC insulated & PVC sheeted with 4 core conductor 6000/1000 volts size 25mm2 (S.I.No.103 P-12) | 1909 | P.M | 171810/- |

Page No:2.

| S.NO: | QTY: | . NAME OF ITEM. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 17. |  | 02 |  | P/F DP I/C change over switch 500 volts 500amp on a prepare board (C.S.I No.198 P-30) | 40126 |  | Each |  | 80252/- |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. |  | 02 |  | Providing & fixing circuit breaker 225amp TP on a prepare board as required (S.I.No.208 P-No.31) | 25541 |  | P.No. |  | 51082/- |  |
| 19. |  | 01 |  | Providing & fixing circuit breaker 400amp TP seething 250-400amp (XS-400C.I) on prepare board as required (S.I.No.210 P-No.31) | 39401 |  | Each |  | 39401/- |  |
| 20. |  | 03 |  | P/F volt meter size 96/96mm 500volts as required as per instruction of E.I (S.I.No.285 P-No.41) | 999 |  | Each |  | 2997/- |  |
| 21. |  | 03 |  | p/f Ammetre size as required. (S.I.No:284 P-41). | 1054 |  | Each |  | 3161/- |  |
|  |  |  | PART “A” NEW ELECTRIC SCHEDULE ITEMS TOTAL |  |  | RS: |  | 27,53,562/- |  |  |

PART “B” OLD ELECTRIC SCHEDULE ITEMS.

| 1. |  | 40 |  | P/F distribution metal board double shutter to accommodate circuit breakers i/c painting with enamel paint and for other similar job on surface (S.I No.5 P-No.13) | 1426.33 |  | Each |  | 57053/- |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. |  | 95 |  | P/F mild steel bar fan clamp 15.8 mm (5/8”) dia suitable for RCC roof (S.I No.1(c) P-No.14) | 141.01 |  | Each |  | 13396/- |  |
| 3. |  | 120 |  | Providing tube light 1-40 watt complete with 40 watt long rod choke starter and patti fitting with Philips components i/c necessary electric connection and fixing on wall or ceiling etc complete. (S.I.No:1 P-33). | 497 |  | Each |  | 59640/- |  |
| 4. |  | 17 |  | P/F 2-40 watts tube light complete with 40watts 4.0” long rod choke starter and putty with Philips components i/c necessary electric connection and fixing on wall or ceiling etc complete (S.I No.1 P-No.33) | 1677 |  | Each |  | 28509/- |  |
| 5. |  | 05 |  | P/F wall bracket light fancy type superior quality including necessary electric connection and fixing on wall oe ceiling etc complete (S.I No.5 P-No.33) | 497 |  | Each |  | 2485/- |  |
| 6. |  | 80 |  | P/F energy sever superior quality including fixing on existing holder etc complete (S.I No.6 P-No.33) | 497 |  | Each |  | 39760/- |  |
| 7. |  | 03 |  | P/F earthing set with 2x2x1/4” cooper plate buried in ground at a depth of 12” or less if water comes out from the ground level with salt and charcoal etc i/c making the pit 12” deep by excavation of all type of soil Earth plat to be connected with No.8. | 3610.04 |  | Each |  | 10830/- |  |
| 8. |  | 36 |  | P/F A.C ceiling fan regulator on metal board recessed in the walls column and covered with plastic sheet (S.I No.2b P-No.14) | 124.30 |  | Each |  | 4475/- |  |
| 9. |  | 13 |  | P/F A.C or DC electric bell 200/250 volt 70mm (S.I No.21 P-No.11) | 150.70 |  | Each |  | 1959/- |  |
| 10. |  | 24 |  | P/F flush type electric bell bush button (S.I No.23 P-No.12) | 25.19 |  | Each |  | 605/- |  |
| 11. |  | 20 |  | Providing (20mm)3/4” G.I. pipe with 1.13 3/029 PVC wire for down of ceiling fan. | 25.19 |  | P.RFT |  | 504/- |  |
|  |  |  | PART “B” OLD ELECTRIC SCHEDULE ITEMS TOTAL |  |  | RS: |  | 1,09,654/- |  |  |

Page No:3.

| S.NO: | QTY: | . NAME OF ITEM. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “C” NON SCHEDULE ITEMS.

| 1. | 22 | S/F wall bracket fan with regulator, capacitor with ON/OFF swtch (Pak, GFC, SK). |  | Each |  |
| --- | --- | --- | --- | --- | --- |
| 2. | 36 | S/F A.C. ceiling fan 56” size i/c fixing charges and wiring connection (Pak, GFC, SK) company. |  | Each |  |
| 3. | 08 | S/F Exhaust fan 12” with iron body and compete with fan hole. |  | Each |  |
| 4. | 12 | S/F water proof capsule light complete with wiring and connection etc. |  | Each |  |
| 5. | 60 | S/F LED lidhgr 22 Watts squire/ round shape i/c wiring connection. |  | Each |  |
| 6. | 1500 | S/F Delto cable size K-6 for internet 3 mm fitted in PVC duct etc. |  | P.M. |  |
| 7. | 39 | S/F Telephone sheet double T.P. socket i/c board and frame with connection. |  | Each |  |
| 8. | 30 | S/F Internet sheet single i/c board frame with connection etc. |  | Each |  |
| 9. | 350 | S/F Man service telephone cable of (20 pairs) in PVC pipe under ground in trench etc. |  | P.M. |  |
| 10. | 1500 | S/F Cable wire of (2 pairs) for intercom connection in PVC duct on surface. |  | P.M. |  |
| 11. | 10 | S/F Search light of superior quality LED type squire shoe with stand and holes for fixing complete with connection etc 50 watts. |  | Each |  |
| 12. | 40 | S/F extension lead wire. |  | Each |  |
| 13. | 10 | S/F Electric charger light. |  | Each. |  |
| 14. | 70 | S/F 2-pin plug socket 10 Amp i/c fancy switch with plastic sheet and board. |  | Each. |  |
| 15. | 220 | S/F Fancy type switch 10 Amp: i/c fancy plastic sheet and board etc. |  | Each. |  |
| 16. | 01 | S/F Electric busbar 200 Amp: 4- Pole with copper grips and fiber patties i/c iron box etc complete with connection etc complete. |  | Each. |  |
|  |  | PART “C” NON SCHEDULE ITEMS TOTAL RS:- |  |  |  |

PART “A”	___________	% Above / Below New Electric Schedule Items 	Total Rs:-

PART “B”	___________	% Above / Below Old Electric Schedule Items 	Total Rs:-

PART “C”	___________	Non Schedule Items 				Total Rs:-

____________________

G. Total Rs:-

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |