BILL OF QUANTITY

Name of work: -	Construction of Compound Boundary Wall around of Eid Gah in village Choudero UC-Choudero Taluka Nasirabad

Sr # 17

Head of A/C: -	Community Development Programme

for Sustainable Goals 2016-17 (PS-41)

| Qty: | S.No. | Item of work | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

A: -        COMPOUND WALL SIZE (100’x75’).

01. 	Excavation for foundation of building, bridges and other structure i/c deg belling dressing refilling around the structure with excavated earth watering ramming lead up to one chain and lift up to 5.0 ft. in ordinary soil.(CSI No.18(b) page-05 )

2238.00 Cft.		@ Rs.3176/25				P.%0Cft		Rs:  7108/-

02. C.C Brick or Stone Ballast 1 ½ “ to 2”guage. 1:4:8. (CSI No.5 p-16).

559.50 Cft.		@ Rs.9416/28				P%Cft.			Rs. 52684/-

03. Pacca brick work in foundation and plinth in cement san mortar in 1;6  (GSI No:4(e),P-25

1902.97 cft.			@Rs: 11948/83			P%cft.		  Rs:  227374/-

04. Reinforcement cement concrete work i/c all Labour and material accept the cost of steel reinforcement and its Labour for bending and binding which will paid separately. This rate also i/c all kinds of forms moulds, lifting shuttering curing rendering and finishing and exposed surface i/c screening and washing of shingle. R.C.C work in roof, slab, beams, columns, rafts, lintels & other structural members laid in situ or precast laid in position etc complete. Ratio 1:2:4 (GSI No:6(a), P-19)

70.25 Cft.		@ Rs.337/00			P.Cft.				Rs.  23674/-

05. Fabrication of M.S reinforcement for C.C i/c cutting, bending, laying in position making joints fastening i/c cost of binding wire also i/cs removal of rust from bars. (GSI No: 7(ii), P-20)

2.82 cwt.		@ Rs.5001/70			P.Cwt				Rs.  14118/-

06.	Pacca Brick work other than building i/c stricking of joints upto 20’ height in cement sand mortar 1:6. (CSI No.7(i)P-26).

2167.26 Cft.		@ Rs.12346/65				P%Cft.		Rs.  267584/-

07. 	Cement Plaster (1:6) ½ “thick up to 12’ Height.  (CSI No.12(a)  page-58 )

2187.42 Sft.		@ Rs.2206/60				P%Sft	         		Rs. 48268/-

08.	Cement Plaster (1:4) 3/8“thick up to 20’ Height. (CSI No.11(b)  page-53 )

5187.26 sft		@ Rs.2197/52				P%Sft			Rs. 113995/-

09.  Cement pointing struck joints on walls. Ratio 1:3 (CSI No.19 (b) p-59).

4404.00 sft		@ Rs.1213/58				P%Sft			Rs.  53446/-

10.Making and fixing steel gated doors complete with1/16” thick sheeting

Locking arrangement angle iron frame 2”x2”x3/8” and ¾ “ sq: bars @ c/c. (CSI No.23 p-91).

60.00 sft		@ Rs.726/72				P.Sft			Rs. 43603/-

13.  White washing two coats (GSI No.23 P-53).

2591.42 Sft.		@ Rs. 829/95				P% sft			Rs.  21507/-

14.  Colour washing two coats. (CSI No.26 p-59).

2591.42 Sft.		@ Rs. 859/95				P%Sft.			Rs.  22285/-

15. Painting new surface preparing surface & painting of guard bars gates of iron bars grating, railing i/c standard braces etc. (T.similar open work).(CSI No. 4(d) P-76). 2 coats

120.00 sft.		@ Rs. 1489/68			P%Sft.			Rs.  1788/-

Total		Rs. 802567/-

Amount Total (a)

--------------------% Above/ Below on the rates CSR		Amount to be added/ deducted on the

Basis of premium quoted.

Total: (b)

Total (A) = a + b in words & figure:

CONTRACTOR					  EXECUTIVE ENGINEER

Public Health Engg: Division

Kamber Shahdadkot @ Kamber