(Tender Form Fees Rs.500/-(Non Refundable) 						TENDER FORM

TENDER INQUIRY NO.   21 /2016-17                                      				 DUE ON 09-06-2017

SUPPLY, INSTALLATION, TESTING OF CCTV CAMERAS AND SYSTEM  FOR THE YEAR 2016-17

Time of  issuance of Tender form upto 08-06-2017 at 02:00 P.M

Time of Submission of Tender on 09-06-2017 at 11.00 A.M

Time of Opening of Tender on 09-06-2017 at 11.30 A.M

| S.# | Description | Qty | Quoted Item / Manufacturer |  |  | Rate in Figure /unit |  | Rate in Word/unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 1 Cameras - 3.0 megapixel (2048 x 1536) resolution OR Above - Bit rate up to 16 Mbps - True Day / Night clear Vision. - Full HD1080p real-time video - DWDR & 3D DNR & BLC - EXIR illuminators: up to 50m - Additional requirements UPS with batteries. Structured cabling. All INDOOR cameras should have the following additional features: o Small light where the indoor light is not available or satisfactory. o Structured cabling. o Proper power sockets with an indoor security box for POE’s and other equipment All OUTDOOR cameras should have the following additional features: o High power search lights to cover the desired area at night to be placed at each location. o Proper outdoor water and weather proof storage/security boxes to keep all the supporting items like POE’s, switches etc. Proper power provisioning for each location to provide electric power 2 PTZ doom cameras - 1/2.8’’ Progressive Scan CMOS, up to 1920x1080 - 25 fps at HD1080p - Up to 100m IR distance - 3D DNR, D WDR - 3D intelligent positioning - 30X Optical Zoom - IP66 rating 3 NVR - 64 channel NVR with two month recording capacity - Up to 5 Mega pixel (2560×1920) recording resolution - Simultaneous HDMI, VGA and CVBS output - HDMI and VGA output up to 1920 x 1080P resolution (53) - Dual gigabit network interfaces - eSATA interface for recording or archiving - Flip open front panel for easy HDD installation - 16-ch 4CIF synchronous playback - JPEG picture capture and playback - Tag video clip, search and playback by tag - Digital zoom on live view and playback - Internal DVD drive to back up selected data - Third-party network camera supported 4 Monitor Monitoring screen(s) for 64 channels (min 40” LCD/LED) 5. Networking Complete networking solution for the proposed mode of communication. (Quote items with unit cost) 6 Other items, accessories All industry standards for the network deployment and suitable / compatible for the offered system/, including I. UPS (Where Required) II. Surge protection III. Structured cabling IV. Equipment / items for protection of equipment from extreme weather conditions, such as thunderstorms (Quote items with unit cost) 7 Software Allied software (if any) must be provided with the turnkey solution Quantity of above items can be increased or decreased | 20 | USA/ Korea / Taiwan |  |  |  |  |  |  |
|  | Labouring, Cabling, Pointing, Fitting, PVC Pipes, Channel Ducting, Tagging etc | As Per Actual |  |  |  |  |  |  |  |
| Note: 1.Responding Organization(s) may quote quantity / Price of these items after site visit. 2.Responding Organization(s) are welcome to add any other item(s), which they feel necessary and are not covered in the above mention list. But they should provide detailed justifications of adding those items/charges in the technical proposals. |  |  |  |  |  |  |  |  |  |
| NATIONAL TAX No…………………………….. GST No: ……………………………………………. C.N.I.C No: …………………………………………. |  |  |  |  | SIGNATURE ……………………………………… NAME OF CONTRACTOR ………………….…… ADDRESS ……………….…………………. CELL NO …………………..……………………….. |  |  |  |  |

I N S T R U C T I O N S.

1.The tender should be accompanied with pay order / demand draft of 2.5% of quoted amount as security Deposit  in the name of Sindh Institute of Ophthalmology & Visual Sciences (SIOVS) Hyderabad.

2.The Income Tax / GST/SST will be deducted according to rules of Govt: of Sindh / Pakistan

3.The tenderer will be responsible to deliver the tendered items on his own cost at SIOVS HYDERABAD

4.In case of short/late supply it will be purchased  from open Market and difference if any will be recovered from the

Contractor.

5.Conditional/ Incomplete Tenders will not be accepted.

6.Sample of all dry items should be provided at the time of opening of tender otherwise tender will be rejected.

7.For alternate offer separate tender purchase receipt must be enclosed.

8.The tenderer should be fulfilling all requirements as per Government rules.

9.Minimum Two(02) Hundred Thousands turn over with Bank Certificate.

10.Minimum Three(03) Years Experience in relevant field with documentary proof.

11.Three Years(03) Income Tax Return (Photo Copy Must Enclosed).

12 Bid Validity 90 Days

Director,

Sindh Institute of Ophthalmology

& Visual Sciences, Hyderabad