SCHEDULE “B”

PACKAGE NO: 10 (CIVIL WORK)

| S.No. | Name of item | Qty. | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling brick work in lime or cement mortar (SI.No.13P.10) |  |  |  |  |
|  |  | 2612.0 | 1285/63 | %CFT | Rs:33581/- |
| 2. | Dismantling 2nd class tiles roofing (SI.No.22(b)P.11) |  |  |  |  |
|  |  | 124.0 | 378/13 | %CFT | Rs:469/- |
| 3. | Removing cement or lime plaster (SI.No.53P.13) |  |  |  |  |
|  |  | 10526.0 | 121/- | %CFT | Rs:12736/- |
| 4. | Dismantling cement concrete plain 1.3.6(SI.No.19(b) P.10) |  |  |  |  |
|  |  | 120.0 | 1306/80 | %CFT | Rs:1568/- |
| 5. | Dismantling cement concrete reinforced separating reinfor cement from concrete cleaning and straightening the same. S.I No 20 P.No 10 |  |  |  |  |
|  |  | 200.0 | 5445/- | % CFT | Rs:10890/- |
| 6. | (b) Dismantling 2nd Class tile roofing. S.I NO 22 (b) P.No 11 |  |  |  |  |
|  |  | 56.0 | 378/13 | % CFT | Rs:212/- |
| 7. | Excavation in foundation of building bridges and other structures including dag belling dressing, refilling around structure with excavated earth watering and ramming lead upto 5 ft (S. I No: 18 P. No: 4) |  |  |  |  |
|  |  | 894.0 | 3176/25 | %CFT | Rs:2840/- |
| 8. | Cement concrete brick or stone ballast 1 ½ “ to 2” gauge 1:5:10 (S. I No: 4 (c) P. No: 15). |  |  |  |  |
|  |  | 440.0 | 8694/25 | %CFT | Rs:3826/- |
| 9. | Pacca brick work in foundation and plinth in cement sand mortar 1:6 (S. I No: 4 (e) P. No: 21). |  |  |  |  |
|  |  | 3260.0 | 11948/36 | %CFT | Rs:389517/- |
| 10. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) 1:2:4 (S. I No: 6 a i P. No: 17) |  |  |  |  |
|  |  | 1059.0 | 337/- | P.SFT | Rs:356883/- |
| 11. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of mild steel (S. I No: 8 (b) P. No: 17) |  |  |  |  |
|  | Tor bar 52.702 |  | 5001/70 | P:CWT | Rs:263600/- |
|  | M.Steel 9.093 |  | 4820/20 | P:CWT | Rs:43830/- |
| 12. | Erection and removal of centering for R.C.C or plain cement concrete works of Deodar wood (2nd-Class)Vertical S.I No 19 ((b) (ii) P.No 18 |  |  |  |  |
|  |  | 781.0 | 3127/41 | % SFT | Rs:24425/- |
| 13. | Supplying and fixing broden glasses on courtyard walls including 1: 3: 6 cement concrete coping. S.I No 61 P.No 105 |  |  |  |  |
|  |  | 1588.0 | 70/44 | P.LFT | Rs:111859/- |
| 14. | Pacca brick work in ground floor 1:6 in cement sand (S. I No: 5 P. No: 21). |  |  |  |  |
|  |  | 303.0 | 12674/36 | %SFt | Rs:38403/- |
| 15. | Pacca brick work in other than building i/c cement and mortor 1.6(SI.No.07P.26) |  |  |  |  |
|  |  | 5992.0 | 12346/65 | %SFt | Rs:739811/- |
| 16. | Cement plaster 1:6 upto 20’ height ½ “ thick (S. I No: 13 (b) P. No: 52). |  |  |  |  |
|  |  | 24392.0 | 2206/60 | %SFT | Rs:538224/- |
| 17. | Cement plaster 1:4 upto 20’ height 3/8” thick (S. I No: P. No: 52). |  |  |  |  |
|  |  | 24392.0 | 2197/52 | %SFT | Rs:536019/- |
| 18. | Cement pointing struck joints on walls. Ratio 1:2 . S.I No 19 (a) P.No 53 |  |  |  |  |
|  |  | 6437.0 | 1287/44 | %SFT | Rs:82873/- |
| 19. | Applying floating coat of cement 1/32 thick |  |  |  |  |
|  |  | 4097.0 | 660/- | %SFT | Rs:27040/- |
| 20. | Cement Plaster 1:4 upto 20height ¾”thick in cement sand (SI:NO: P; ) |  |  |  |  |
|  | 4097.0 |  | 3015/76 | %SFT | Rs:123556/- |
| 21. | S/F tear at site of work |  |  |  |  |
|  |  | 1.814 | 3575/- | P:CWT | Rs:6485/- |
| 22. | 2nd: tiles roofing consisting of 4 earth and 1 mud plaster with gobri over ½” thick cement plaster 1.6 with 34 lbs of hot bitumen coating and blinded provided over one layer of 12”x6”2” tiles laid in 1.6 cement mortar i/c cement pointing under neath of tiles complete including curing etc (SI.No.01P.33) |  |  |  |  |
|  |  | 123.0 | 7607/23 | %SFT | Rs:9357/- |
| 23. | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position.S.I NO 6 P NO 91 |  |  |  |  |
|  |  | 1.814 | 186/34 | P.CWT | Rs:338/- |
| 24. | Laying floor of approved with glazed tile 1/4" thick in white cement 1:2 over 3/4" thick cement mortar 1:2 complete. S.I No 24 P.No 43 |  |  |  |  |
|  |  | 308.0 | 27678/86 | % SFT | Rs:85251/- |
| 25. | White glazed tiles 1/4" thick dado jointed in white cement and laid over 1:2 cement sand mortar 3/4" thick including finishing. S.I No 37 P.No 45 |  |  |  |  |
|  |  | 1038.0 | 28253/61 | % SFT | Rs:293272/- |
| 26. | Providing and fixing G.I frames /Choukhats of size 7" x 2" or 4 1/2" x 3" for door using 20 gauge G.I sheet I/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage , tools and plants used in making and fixing. S.I No 29 P.No 93 |  |  |  |  |
|  |  | 32.0 | 228/90 | P.RFT | Rs:7325/- |
| 27. | Supplying & fixing in position iron/steel grill of 3/4" x 1/4" size flat iron of approved design including painting 3 coats etc. complete (weight not to be less than 3.7 Lbs./Sq . Foot of finished grill). S.I No 26 P.No 93 |  |  |  |  |
|  |  | 15.0 | 180/50 | P.SFT | Rs:2708/- |
| 28. | Providing and laying 1" thick topping cement concrete (1:2:4 ) including Surface finishing and dividiing into panels: 2" thick S.I No 16 C , P.No 42 |  |  |  |  |
|  |  | 247.0 | 3275/50 | % SFT | Rs:8090/- |
| 29. | First class Deodar wood framed braced & battened colors & windows complete with iron fitting & fixed in position i/c chowkats 1 ¾ “ thick with battens & ¾ planks (S. I No: 10 P. No: 59). |  |  |  |  |
|  |  | 50.0 | 902/93 | P:SFT | Rs:45147/- |
| 30. | Cement concrete plain i/c placing compacting finishing and curing complete (i/c screening and washing at stone aggregate without ratio 1:2:4(SI.No.5(f)P.15) |  |  |  |  |
|  |  | 184.0 | 14429/25 | %SFT | Rs:26550/- |
| 31. | M/F steel grated door with 1/16:thick sheeting i/c angle iron frame 2”x2”i/c and ¾”square bars 4:centere to center with looking rearrangement(SI:NO: P; ) |  |  |  |  |
|  |  | 28.0 | 726/72 | P:SFT | Rs:20348/- |
| 32. | Colour wash two coats (SI.No. P.No. ) |  |  |  |  |
|  |  | 28493.0 | 859/90 | %SFT | Rs:245011/- |
| 33. | White washing. Three coat . S.I No 26 C. P.No 54 |  |  |  |  |
|  |  | 14853.0 | 829/95 | % SFT | Rs:123272/- |
| 34. | Preparing surface and painting guards bars gates iron bars railing (S.I No: 41 (j) P.No: 69). |  |  |  |  |
|  |  | 868.0 | 1270/83 | %SFt | Rs:11031/- |
| 35. | Preparing surface painting doors and window open work three coats |  |  |  |  |
|  |  | 749.0 | 2116/41 | % Sft | Rs:15852/- |
|  |  |  |  | Total | 4242209/- |
|  | ------% Above / Below on the rates CSR |  |  |  |  |
|  | Contractor |  | EXECUTIVE ENGINEER EDUCATION WORKS DIVISION GHOTKI |  |  |

|  | Part (B) W/S & S/F |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | Providing and fixing squating type white glazed earthen ware w.c pan with front flush inletr & complete with including the cost of flushing cistern with internal fitting and flush Pipe with bend and making requisite number of holes in walls plinth & floor for Pipe connection & making good in cement concrete 1: 2: 4: (Foreign Equivalant).With 4" dia C.I Trap. S.I NO 2 B(i) P.No 1 |  |  |  |  |
|  | W.C | 7.0.Nos: | 6014/80 | Each | Rs:42104/- |
| 2. | Providing & fixing 4" x 4" x 4" dia plain C.I branch of the required degree including extra painting to match the colour of the building plain tee . S.I No 7 P.No 9 |  |  |  |  |
|  |  | 7.0.0Nos: | 704/20 | Each | Rs:4929/- |
| 3. | Providing & fixing 4" dia. C.I plain bend of the required degree including extra painting to match the colour of the building. S.I No 10 P.No 10 |  |  |  |  |
|  |  | 7.0.0Nos: | 566/70 | Each | Rs:3967/- |
| 4. | S/F swan type piller cock of superior quality with crystal head ½”dia (Si:NoL16(b)P.19) |  |  |  |  |
|  |  | 3.0 | 877/80 | Each | Rs:2633/- |
| 5. | Providing & fixing 4" dia C.I soil & vent Pipe including cutting and fitting and extra painting to match the colour of building. S.I No 1 P.No 9 |  |  |  |  |
|  |  | 48.0 | 333/29 | RFT | Rs:15998/- |
| 6. | Providing & fixing 22" x 16" lavatory basin in white glazed earthen ware complete with & I/c the cost of W.I or C.I cantilever brackets 6 inches built into walls, painted white in 2 coats after a primary coat of red lead paint, a pair of 1/2" rubber plug & chrome brass waste of approved pattern 1-1/4" dia malloable iron or c.p. ba]rass traps, malloable iron or brass unions and making requisite number of holes in walls plinth and floor for pipe connection & making good in cement cencrete 1:2:4 (Foreign or Equiivalant). S.I No 13 P.No 4 |  |  |  |  |
|  |  | 3.0 | 6237/- | Each | Rs:18711/- |
| 7. | Providing G.I pipes and specials etc i/c fixing cutting and fitting complete with & i/c the cost of cutting trench upto 2 ½”feet deep refilling watering ramming and disposal of surpluses earth within one chap and painting 2 coats of bitumen paint to pipes and special after clasaning and brass ran cloth soaked in maxphalt composition wrapped tightly round pipes and testing to pressure head 200 feet and handling. |  |  |  |  |
|  | ½ dia | 120 | 73/21 | P:RFT | Rs:8785/- |
|  | ¾”dia | 590.0 | 95/79 | P:RFT | Rs:56516/- |
|  | 1”dia | 250.0 | 128/55 | P;RFT | Rs:32138/- |
| 8. | Add extra for labour for providing & fixing of earthen ware pedestal white or coloured glazed (Foreign or Equivalent..) S.I No 9 P.No 3 |  |  |  |  |
|  |  | 3.0 No | 938/47 | Each | Rs:2815/- |
| 9. | Providing R.C.C pipe with collars slass "B" and digging the trenches to required depth & fixing inposition including cutting , fitting & jointing with maxphalt composition & cement mortor 1: 1 and testing with water pressure jto a head of 4 feet a bove the top of the heghest pipe & refilling with excavated staff. S.I No 2 P.No 24 |  |  |  |  |
|  | 4” dia | 60.0 | 146/57 | P.RFT | Rs:8794/- |
|  | 6” dia | 130.0 | 199/25 | P.RFT | Rs:25903/- |
| 10. | Supplying and fixing in position brass bib cocks S.I NO 1 P.No 16 |  |  |  |  |
|  | ½ dia | 40.0 | 337/92 | Each | Rs:13517/- |
| 11. | Add extra labour for concealed G.I Pipe & fittings I/c making recess in the wall for Pipe & making good in cement mortor etc. complete S.I No 2 P.No 12. |  |  |  |  |
|  | ½ “ dia | 150.0 | 7/82 | RFT | Rs:1173/- |
|  | ¾ “ dia | 120.0 | 8/45 | RFT | Rs:1014/- |
| 12. | S/Fixing cancealed stop cock of superir quality with c.p head 1/2" dia. S.I No 11 (a) P.No 18 |  |  |  |  |
|  |  | 30.0 No | 478/28 | Each | Rs:14348/- |
| 13. | Providing & fixing in position nyloon connections complete with 1/2" dia, brass stop cock with pair of brass nuts and lining joints to nyloon connection. S.I No 23 P.No 6 |  |  |  |  |
|  |  | 03 | 447/15 | Each | Rs:1341/- |
| 14. | Providing and fixing handle valves (china) S.I No 5 P.No 17 |  |  |  |  |
|  | ¾ “ dia | 18.0 | 271/92 | Each | Rs:4895/- |
| 15. | Supplying & fixing fibre glass tank of approved quality and design and wall thicness as specified I/c cost of nutes, bolts and fixing in plateform of cement concerete 1: 3: 6 and makin connections for in -let ,& ou-let & over flow pipes etc. complete. 250 gallons wall thickness 3.5 mm . S.I No 3 P.No 21 |  |  |  |  |
|  |  | 5.0 No | 21989/61 | Each | Rs:109948/- |
| 16. | P/L UPVC pressure pipe of class “B”(equiralent make)fixing in trenche i/c cutting with joint with one rubber rain i/c testing with water to head 61meter or 200fts (Publ.health)(S>EIP No.22 |  |  |  |  |
|  | 4” dia | 200.0 | 137/- | Each | Rs:27400/- |
|  | Part Non Shedule Item |  |  |  |  |
|  | - Filter | 140 | 76/05 | Each | Rs:10647/- |
|  | - G.I Pipe | 840 | 38/95 | Each | Rs:32718/- |
|  | - P.P | 7 | 14438/- | Each | Rs:101066/- |
|  | - H.Pump | 7 | 1442/- | Each | Rs:10094/- |
|  |  |  |  | Total | 154525/- |
|  | ------% Above / Below at Par |  |  |  |  |
|  | Contractor |  | EXECUTIVE ENGINEER EDUCATION WORKS DIVISION GHOTKI |  |  |