SPPRA  BIDDING  DOCUMENT

STANDARD BIDDING DOCUMENT

PROCUREMENT OF WORKS.

(For Contracts Costing upto Rs.2.500 million)

Standard Bidding Document is intended as a model for admeasurements                 (Percentage Rate / Unit price for unit rates in a Bill of Quantities) types of contract,               the main text refers to admeasurements contracts.

EXECUTIVE ENGINEER BUILDINGS DIVISION SHAHEED BENAZIR ABAD

BIDDING DATA

(This section should be filled in by the Engineering / Procuring Agency before issuance of the Bidding Documents)

| (a) | Name of Procuring Agency. | Executive Engineer Buildings Division Shaheed Benazir Abad. |
| --- | --- | --- |
| (b) | Brief Description of Works. | M&R TO MCH NASRI SHARIF TALUKA SAKRAND |
| (c) | Procuring Agency’s address | Executive Engineer Buildings Division at Khoja Garden Nawabshah District Shaheed Benazir Abad. |
| (d) | Estimated Cost. | Rs.0.500Million |
| (e) | Amount of Bid Security (Fill in lump sum amount or in %age of bid amount / estimated cost, but not exceeding 5%) | 5% Rs.25000/- |
| (f) | Period of Bid Validity (days). | (Not more than sixty days). |
| (g) | Security Deposit (I/c: Bid Security). (In %age of bid amount / estimated cost equal to 10%). | 10% Rs.50000/- |
| (h) | Percentage, if any, to be deducted from bills. | 8% Rs.40000/- |
| (i) | Deadline for submission of Bids alongwith time:- | upto 12.00 Noon |
| (J) | Venue, Time and Date of Bid Opening :- | Office Executive Engineer Buildings Division S.B.A. at Khoja Garden Nawabshah 2.00 Pm. |
| (k) | Time for completion from written order of commence :- | 01 Month. |
| (l) | Liquidity Damages :- | (0.05 of Estimated cost of Bid cost per day of delay, but total not exceeding 10%). |
| (m) | Deposit Receipt No. Date Amount (In words & figures). |  |
| (n) | Tender Issued to / Name of Contractor / Agency | ________________________________________ |
|  | DR. No. __________________ Rs. 1000/- Dated : - _____________________ |  |

Call Deposit No.__________________________________ dated ___________________             of the ___________________________________________ amounting to Rs: ______________________.

Rates.

____________________________

_____________________

_____________________

| (CONTRACTOR) | ## EXECUTIVE ENGINEER BUILDINGS DIVISION SHAHEED BENAZIR ABAD. |
| --- | --- |

FORM OF BID

(LETTER OF OFFER)

| Bid Reference No. | NO.XEN(B)/W&S/TC/G-55/ / OF SBA, Dated: |
| --- | --- |

To,

The Executive Engineer,

Buildings Division,

Shaheed Benazir Abad.

Gentlemen.

| 1. | Having examined the Bidding Documents including Instruction of Bidders, Bidding Data, Conditions of Contract, Contract Data, Specifications, Drawings, if any, Schedule of Prices and Addenda Nos. _______________________________________ for the execution of the above-named works, we, the undersigned, being a company doing business under the name and address ______________________ ____________________________________________________________________________________and being duly incorporated under the laws of Pakistan hereby offer to execute and complete such works and remedy any defects therein in conformity with the said Documents including Addenda thereto for the Total Bid Price of Rs.______________ (Rupees ____________________________________ __________________________________________________________________________________) or such other sum as may be ascertained in accordance with the said Documents. |
| --- | --- |
| 2. | We understand that all the Schedules attached thereto form part of this Bid. |
| 3. | As security for due performance of the undertakings and obligations of this Bid, we submit herewith a Bid Security in the amount of 5% of Estimated Cost., Rs.____________________ drawn in your favour of made payable to you and valid for a period of twenty eight (28) days beyond the period of validity of Bid. |
| 4. | We undertake, if our Bid is accepted, to commence the Works and to deliver and complete the Works comprised in the Contract within the time(s) stated in Contract Data. |
| 5. | We agree to abide by this Bid for the period of 90 Days from the date fixed for receiving the same and it shall remain binding upon us and may be accepted at any time before the expiration of that period. |
| 6. | Unless and until a formal Agreement is prepared and executed, this Bid, together with your written acceptance thereof, shall constitute a binding contract between us. |
| 7. | We undertake, if our Bid is accepted, to execute the Performance Security referred to in Conditions of Contract for the due performance of the Contract. |
| 8. | We understand that you are not bound to accept the lowest or any bid you may receive. |
| 9. | We do hereby declare that the Bid is made without any collusion, comparison of figures or arrangement with any other person or persons making a bid for the works. |

Dated this ________ day of __________________ 2017.

In the capacity of _____________ duly authorized to sign bid for and on behalf of

___________________________

(Signature of Bidder/ Contractor).

Witness :

Signature 	___________________________

Name 		________________________________________________________________

Address 	________________________________________________________________

________________________________________________________________

FORM OF BID AND SCHEDULES TO BID

SCHEDULE TO BID INCLUDE THE FOLLOWING:

- Schedule A to Bid: Schedule of Prices

- Schedule B to Bid: Specific Works Data

- Schedule C to Bid: Works to be performed by Subcontractors.

- Schedule D to Bid: Proposed Pogramme of Works.

- Schedule E to Bid: Method of Performing Works

- Schedule F to Bid: Integrity Pact.

SCHEDULE – A  TO BID

SCHEDULE OF PRICES.

SCHEDULE – A TO BID

PREAMBLE TO SCHEDULE OF PRICES.

| 1. | General |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.1 | The Schedule of Prices shall be read in conjunction with the Conditions of contract, Contract Data together with the Specifications and Drawing, if any. |  |  |
|  | 1.2 | The Contract shall be for the whole of the works as described in these Bidding Documents. Bids must be for the complete scope of works. |  |  |
| 2. | Description. |  |  |  |
|  | 2.1 | The general directions and descriptions of works and materials are not necessarily repeated nor summarized in the Schedule of Prices. References to the relevant sections of the Bidding Documents shall be made before entering prices against each item in the Schedule of Prices. |  |  |
| 3. | Units & Abbreviations. |  |  |  |
|  | 3.1 | Units of measurement, symbols and abbreviations expressed in the Bidding Documents shall comply with the System Internationals, |  |  |
|  |  | __________________________________ __________________________________ |  |  |
|  |  | Note : (The abbreviations to be used in the Schedule of Prices to be defined by the Procuring Agency). |  |  |
| 4. | Rates and Prices. |  |  |  |
|  | 4.1 | Except as otherwise expressly provided under the Conditions of Contract, the rates and amounts entered in the Schedule of prices shall be the rates as which the Contractor shall be paid and shall be the full inclusive value of the works set forth or implied in the Contract: except for the amounts reimbursable, if any to the Contractor under the contract. |  |  |
|  | 4.2 | Unless other-wise stipulated in the Contract Data, the premium, rates and prices entered by the bidder shall not be subject to adjustment during the performance of the Contract. |  |  |
|  | 4.3 | All duties, taxes and other levies payable by the Contractor shall be included in the rates and prices. |  |  |
|  | 4.4 | The whole cost of complying with the provisions of the Contract shall be included in the items provided in the Schedule of Prices and where no items are provide, the cost shall be deemed to be distributed among the rates and prices entered for the related items of the Works and no separate payment will be made for those items. |  |  |

Continued.

- : ( 2 ) : -

|  | 4.5 | (a) | The bidder shall be deemed to have obtained all information as to and all requirements relates thereto which may affect the bid price. |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | (b) | The Contractor shall be responsible to make complete arrangements for the transportation of the Plant to the Site. |  |  |
|  |  | Note : (Procuring Agency may modify as appropriate). |  |  |  |
|  |  | (Note : The abbreviations to be used in the Schedule of Prices to be defined by the Procuring Agency). |  |  |  |
|  | 4.6 | The Contractor shall provide for all parts of the Works to be completed in every respect. Notwithstanding that any details, accessories, etc. required for the complete installation and satisfactory operation of the Works, are not specifically mentioned in the Specifications, such details shall be considered as included in the Contract Price. |  |  |  |
| 5. | Bid prices. |  |  |  |  |
|  | 5.1 | Break-up of Bid Prices. |  |  |  |
|  |  | The various elements of Bid Prices shall be quoted as detailed by the Procuring Agency in the format of Schedule of Prices. |  |  |  |
|  |  | The bidder shall recognize such elements of the costs which he expects to incur the performance of the Works and shall include all such costs in the rates and amounts entered in the Schedule of Prices. |  |  |  |
|  | 5.2 | Total Bid Price |  |  |  |
|  |  | The total of bid prices in the Schedule of Prices shall be entered in the Summary of Bit Prices. |  |  |  |
| 6. | Provisional Sums and Day work. |  |  |  |  |
|  | 6.1 | Provisional Sums included and so designated in the schedule of prices if any, shall be expended in whole or in part at the direction and discretion of the Engineer / Procuring Agency. The Contractor will only receive payment in respect of Provisional Sums, if he has been instructed by the Engineer / Procuring Agency to utilize such sums. |  |  |  |
|  | 6.2 | Day work rates in the contractor’s bid are to be used for small additional amounts of work and only when the Engineer have given written instructions in advance for additional work to be paid for in that way. |  |  |  |

| (CONTRACTOR) | ## EXECUTIVE ENGINEER BUILDINGS DIVISION SHAHEED BENAZIR ABAD. |
| --- | --- |

SCHEDULE – A TO BID

SCHEDULE OF PRICES

| Name of work :- | M&R TO MCH NASRI SHARIF TALUKA SAKRAND |
| --- | --- |

| Item No. | Description | Quantity | Unit Rate (In Rs.) | Total amount (In Rs.) |
| --- | --- | --- | --- | --- |
|  | I. (Civil Work) |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
|  | II. (Internal Sanitary & Water Supply) |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
|  | III. (Electrification) |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
|  | IV. (External Development) |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
|  | V. (Miscellaneous Items) |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| Total (to be carried to summary of bid price) Add / Deduct the percentage quoted above / below on the prices of items based on composite schedule rates; |  |  |  |  |
| (CONTRACTOR); ## EXECUTIVE ENGINEER BUILDINGS DIVISION SHAHEED BENAZIR ABAD.. |

BILL OF QUANTITIES.

(A) Description and rate of Items based on Composite Schedule of Rates.

| Item No. | Quantities | Description of item to be executed at site. | Rate. | Unit. | Amount (In Rupees) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |

| Amount Total (a) _____________________% above / below on the rates of CSR. | Amount to be added / deducted on the basis of premium quoted Total (b). |
| --- | --- |
| Total (A) = a+b in words & figures. |  |

| (CONTRACTOR) | ## EXECUTIVE ENGINEER BUILDINGS DIVISION SHAHEED BENAZIR ABAD. |
| --- | --- |

Conditions of Contract.

Clause – 1: Commencement & Completion Dates of work. The contractor shall not enter upon or commence any portion or work except with the written authority and instructions of the Engineer –                        In-Charge or of in subordinate – In-Charge of the work. Failing such authority the contractor shall have no claim to ask for measurements of or payment for work.

The contractor shall proceed with the works with due expedition and without delay and complete the works in the time allowed for carrying out the work as entered in the tender shall be strictly observed by the contractor and shall reckoned from the date on which the order to commence work is given to the contractor. And further to ensure good progress during the execution of the work, contractor shall be bound, in all in which the time allowed for completion of any work exceeds one month, to achieve progress on the prorate basis.

Clause – 2: Liquidated Damages. The contractor shall pay liquidated damages to the Agency at the rate per day stated in the bidding data for each day that the completion date is later than the Intended completion date; the amount of liquidated damage paid by the contractor to the Agency shall not exceed 10% of the contract price. Agency may be deduct liquidated damages from payments due to the contractor. Payment of liquidated damages does not affect the contractor’s liabilities.

Clause – 3: Termination of the Contract.

- Procuring Agency / Executive Engineer may terminate the contract if either of the following conditions exits

  - Contractor causes a breach of any clause of the Contract;

  - The progress of any particular portion of the work is unsatisfactory and notice of  10 days has expired;

  - In the case of abandonment of the work owing to the serious illness or death of the contractor or any other cause.

  - Contractor can also request for termination of contract if a payment certified by the Engineer is not paid to the contractor within 60 days of the date of the submission of the bill;

- The Executive Engineer / Procuring Agency has power to adopt any of the following courses as may deem fit:-

  - To forfeit the security deposit available except conditions mentioned at A(iii) and (iv) above;

  - To finalize the work by measuring the work done by the contractor.

- In the event of any of the above courses being adopted by the Executive Engineer / Procuring Agency, the contractor shall have:-

- No claim to compensation for any loss sustained by him by reason of his having purchased or procured any materials, or entered into any engagements, or made any advances on account of, or with a view to the execution of the work or the performance of the contract,

- However, the contractor can claim for the work done at site duly certified by the Executive Engineer in writing regarding the performance of such work and has not been paid.

Procuring Agency / Engineer may invite fresh bids for remaining work.

Clause – 4: Possession of the site and claims for compensation for delay. The Engineer shall give possession of all parts of the site to the contractor. If possession of site is not given by the date stated in the contract data, no compensation shall be allowed for any delay caused in starting of the work on account of any acquisition of land, water standing in borrow pits / compartments or in according sanction to estimates. In such case, either date of commencement will be changed or period of completion is to be extended accordingly.

Clause – 5: Extension of Intended Completion Date. The Procuring Agency either at its own initiatives before the date of completion or on desire of the contractor may extend the intended completion date, if an event (which hinders the execution of contract) occurs or a variation order is issued which makes it impossible to complete the work by the intended completion date for such period as he may think necessary or proper. The decision of the Executive Engineer in this matter shall be final; where time has been extended under this or any other clause of this agreement, the date for completion of the work shall be the date fixed by the order giving the extension or by the aggregate of all such orders, made under this agreement.

When time has been extended as aforesaid, it shall continue to be the essence of the contract and all clauses of the contract shall continue to be operative during the extended period.

Clause – 6: Specifications. The Contractor shall execute the whole and every part of the work in the most substantial and work-man-like manner and both as regards materials and all other matters in strict accordance with the specifications lodged in the office of the Executive Engineer and initialed by the parties, the said specification being a part of the contract. The contractor shall also confirm exactly, fully and faithfully to the designs, drawing, and instructions in writing relating to the work signed by the Engineer – In - Charge and lodge in his office and to which the contractor shall be entitled to have access at such office or on the site of work for the purpose of inspection during office hours and the contractor shall, if he so requires, be entitled at his own expense to make or cause to be made copies of the specifications, and of all such designs, drawings, and instructions as aforesaid.

Clause – 7: Payments.

- Interim / Running Bill. A bill shall be submitted by the contractor as frequently as the progress of the work may justify for all work executed and not included in any previous bill at least once in a month and the Engineer – In – Charge shall take or cause to be taken the requisite measurements for the purpose of having the same verified and the claim, as for as admissible, adjusted, if possible before the expiry of 10 days from the presentation of the bill, at any time depute a subordinate to measure up the said work in the presence of the contractor or his authorized agent, whose countersignature to the measurement list will be sufficient to warrant and the Engineer – In – Charge may prepare a bill from such list which shall be binding on the contractor in all respects.

The Engineer / Procuring Agency shall pass / certify the amount to be paid to the contractor, which he considers due and payable in respect thereof, subject to deduction of security deposit, advance payment if any made to him and taxes.

All such intermediate payment shall be regarded as payments by way of advance against the final payment only and not as payments for work actually done and completed, and shall not preclude the Engineer – In – Charge from recoveries from final bill and rectification of defects and unsatisfactory items of works pointed out to him during defect liability period.

- The Final Bill. A bill shall be submitted by the contractor within one month of the date fixed for the completion of the work otherwise Engineer – In – Charge’s certificate of the measurements and of the total amount payable for the works shall be final and binding on all parties.

Clause – 8: Reduced Rates. In cases where the items of work are not accepted as so completed, the Engineer – In – Charge may make payment on account of such items at such reduced rates as he may consider reasonable in the preparation of final or on running account bills with reasons recorded in writing.

Clause – 9: Issuance of Variation and Repeat Orders.

- Agency may issue a Variation Order for procurement of works, physical services from the original contractor to cover any increase or decrease in quantities, including the introduction of new work items that are either due to change of plans, design or alignment to suit actual field conditions, within the general scope and physical boundaries of the contract.

- Contractor shall not perform a variation until the Procuring Agency has authorized the variation in writing subject to the limit not exceeding the contract cost by of 15% on the same conditions in all respects on which he agreed to do them in the work, and at the same rates, as are specified in the tender for the main work. The contractor has no right to claim for compensation by reason of alternations or curtailment of the work.

- In case the nature of the work in the variation does not correspond with items in the Bill of Quantities, the quotation by the contractor is to be in the form of new rates for the relevant items of work, and if the Engineer – In - Charge is satisfied that the rate quoted is within the rate worked out by him on detailed rate analysis, and then only he shall allow him that rate after approval from higher authority.

- The time for the completion of the work shall be extended in the proportion that the additional work bear to the original contract work.

- In case of quantities of work executed result the Initial Contract Price to be exceeded by more than 15%, and then Engineer can adjust the rates for those quantities causing excess the cost of contract beyond 15% after approval of Superintending Engineer.

- Repeat Order: Any cumulative variation, beyond the 15% of initial contract amount, shall be subject of another contract to be tendered out if the works are separable from the original contract.

Clause – 10 Quality Control.

- Identifying Defects: If at any time before the security deposit is refunded to the contractor / during defect liability period mentioned in bid data, the Engineer – In - Charge or his subordinate-in-charge of the work may instruct the contractor to uncover and test any part of the works which he considers may have a defect due to use of unsound materials or unskillful workmanship and the contractor has to carry out a test at his own cost irrespective of work already approved of paid.

- Correction of Defects: The contractor shall be bound forthwith to rectify or remove and reconstruct the work so specified in whole or in part, as the case may require. The contractor shall correct the notified defect within the Defects Correction Period mentioned in notice.

- Uncorrected Defects:

  - In the case of any such failure, the Engineer – In – Charge shall give the contractor at least 14 days notice of his intention to use a third party to correct a defect. He may rectify or remove, and re-execute the work or remove and replace the materials or articles complained of as the case may be at the risk and expense in all respects of the contractor.

  - If the Engineer considers that rectification / correction of a defect is not essential and it may be accepted or made use of; it shall be within his discretion to accept the same at such reduced rates as he may fix therefore.

Clause – 11:

- Inspection of Operations. The Engineer and his subordinates, shall at all reasonable times have access to the site for supervision and inspection of works under or in course of execution in pursuance of the contract and the contractor shall afford every facility for and every assistance in obtaining the right to such access.

- Dates for Inspection and Testing. The Engineer shall give the contractor reasonable notice of the intention of the engineer – in – charge or his subordinate to visit the work shall have been given to the contractor, then he either himself be present to receive orders and instructions, or have a responsible agent duly accredited in writing present for that purpose, orders given to the contractor’s duly authorized agent shall be considered to have the same force an effect as if they had been given to the contractor himself.

Clause – 12: Examination of work before covering up.

- No part of the works shall be covered up or put out of view / beyond the read without giving notice of not less than 5 days to the Engineer whenever any such part of the works or foundations is or are ready or about to be ready for examination and the Engineer shall, without delay, unless he considers it un-necessary and advises the contractor accordingly, attend for the purpose of examining and measuring such part of the works or of examining such foundations;

- If any work is covered up or placed beyond the reach of measurement without such notice having been given, the same shall be uncovered at the contractor’s expense, and in default thereof no payment or allowance shall be made for such work, or for the materials with which the same was executed.

Clause – 13: Risks. The contractor shall be responsible for all risks of loss of or damage to physical property or facilities or related services at the premises and of personal injury and death which arise during and in consequence of its performance of the contract, if any damage is caused while the work is in progress or become apparent within three months of the grant of the certificate of completion, final or otherwise, the contractor shall make good the same at his own expense, or in default the Engineer may cause the same to be made good by other workmen, and deduct the expenses from retention money lying with the Engineer.

Clause – 14: Measures for prevention of fire and safety measures. The contractor shall not set fire to any standing jungle, trees, bush-wood or grass without a written permit from the Executive Engineer. When such permit is given, and also in all cases when destroying, cutting or supporting trees, bush-wood, grass, etc by fire, the contractor shall take necessary measures to prevent such fire spreading to or otherwise damaging surrounding property. The contractor is responsible for the safety of all its activities i/c protection of the environment on and off the site. Compensation of all damage done intentionally or unintentionally on or off the site by the contractor’s labour shall be paid by him.

Clause – 15: Sub-contracting. The contractor shall not subcontract the whole of the works, except where otherwise provided by the contract. The Contractor shall not subcontract any part of the works without the prior consent of the Engineer. Any such consent shall not relieve the contractor from any liability or obligation under the contract and he shall be responsible for the acts, defaults and neglects of any subcontractor, his agents, servants or workmen as if these acts, defaults or neglects were those of the contractor, his agents, servants or workmen. The provisions of this contract shall apply to such subcontractor or his employees as if he or it were employees of the contractor.

Clause – 16: Disputes. All disputes arising in connection with the present contract, and which cannot be amicably settled between the parties, the decision of the Superintending Engineer of the circle / officer / one grade higher to awarding authority shall be final, conclusive and binding on all parties to the contract upon all questions relating to the meaning of the specifications, designs drawings and instructions herein before mentioned and as to the quality of workmanship, or materials used on the work or as to any other questions, claim, right, matter, or thing whatsoever in any way arising out of, or relating to the contract design, drawings, specifications, estimates, instructions, orders or these conditions or otherwise concerning the works, or the execution, of failure to execute the same, whether arising, during the progress of the work, or after the completion or abandonment thereof.

Clause – 17: Site Clearance. On completion of the work, the contractor shall be furnished with a certificate by the Executive Engineer (hereinafter called the Engineer – In-charge) of such completion, but neither such certificate shall be given nor shall the work be considered to be complete until the contractor shall have removed all temporary structures and materials brought at site either for use or for operation facilities i/c cleaning debris and dirt at the site. If the contractor fails to comply with the requirements of this clause then Engineer – In-charge, may at the expense of the contractor remove and dispose of th same as he thinks fit and shall deduct the amount of all expenses so incurred from the contractor’s retention money. The contractor shall have no claim in respect of any surplus materials as aforesaid except for any sum actually realized by the sale thereof.

Clause – 18 Financial Assistance / Advance Payment.

- Mobilization advance is not allowed:

- Secured Advance against materials brought at site.

  - Secured Advance may be permitted only against imperishable materials / quantities anticipated to be consumed / utilized on the work within a period of three months from the date of issue of secured advance and definitely not for full quantities of materials for the entire work / contract. The sum payable for such materials on site shall not exceed 75% of the market price of materials;

  - Recovery of Secured Advance paid to the contractor under the above provisions shall be affected from the monthly payments on actual consumption basis, but not later than period more than three months (even if un-utilized).

Clause – 19: Recovery as arrears of Land Revenue. Any sum due to the Government by the contractor shall be liable for recovery as arrears of Land Revenue.

Clause – 20: Refund of Security Deposit / Retention Money. On completion of the whole of the works (a work should be considered as complete for the purpose of refund of security deposit to a contractor from the last date on which its final measurements are checked by a competent authority, if such check is necessary other-wise from the last date of recording the final measurements), the defects notice period has also passed and the Engineer has certified that all defects notified to the contractor before the end of this period have been corrected, the security deposit lodged by a contractor (in cash or recovered in installments from his bills) shall be refunded to him after the expiry of three months from the date of which the work is completed.

| (CONTRACTOR) | ## EXECUTIVE ENGINEER BUILDINGS DIVISION SHAHEED BENAZIR ABAD |
| --- | --- |

FORM OF CONTRACT AGREEMENT

This contract agreement made on the _________ day of _____________________ 2017, between Executive Engineer, Buildings Division Shaheed Benazir Abad of the one part and _________________________________________________ of the other part.

Whereas, the Procuring Agency is desirous that certain Works, viz “M&R TO MCH Nasri Sharif Tal: Sakrand.”,  Should be executed by the Contractor and has accepted a Bid by the Contractor for the execution and completion of such works and the remedying of any defects therein.

Now this Agreement witnesseth as follows:

- In this Agreement words and expressions shall have the same meanings as re respectively assigned to them in the Conditions of Contract hereinafter referred to.

- The following documents after incorporating addenda, if any except those parts relating to Instructions to Bidders, shall be deemed to form and be read and constructed as part of this Agreement, viz:

- The letter of Acceptance;

- The completed Form of Bid alongwith Schedules to Bid;

- Conditions of Contract & Contract Data;

- The Priced Schedule of Prices / Bill of Quantities (BoQ);

- The Specifications; and

- The Drawings

- In consideration of the Payments to be made by the Procuring Agency to the Contractor as hereinafter mentioned, the Contractor hereby covenants with the Procuring Agency to execute and complete the Works & remedy defects therein in conformity and in all respects within the Provisions of the Contractor.

- The Procuring Agency hereby covenants’ to pay the Contractor, in consideration of the execution and completion of the Works as per Provisions of the Contract, the Contract Price or such other sum as may become payable under the provisions of the Contract at the times and in the manner prescribed by the Contract.

IN WITNESS WHEREOF the parties hereto have caused this Contract Agreement to be executed on the day, month and year first before written in accordance with their respective laws.

|  | ## DIVISIONAL ACCOUNTS OFFICER BUILDINGS DIVISION SHAHEED BENAZIR ABAD |
| --- | --- |

| (CONTRACTOR) | ## EXECUTIVE ENGINEER BUILDINGS DIVISION SHAHEED BENAZIR ABAD. |
| --- | --- |

| Name of work :- | M&R TO MCH NASRI SHARIF TAL: SAKRAND. |
| --- | --- |

SCHEDULE – B.

| Qty. | Items. | Rate. |  | Unit. | Amount. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1/- Excavation in foundation of buildings and other structure i/c dagbelling dressing refilling around structure with excavated earth watering and ramming lead upto 5,ftS.I.NO:18,P:No:4) |  |  |  |  |  |
| 600,cft: | @ | 3176/25 |  | %Ocft: | Rs:1906/- |  |
|  | 2/-Cement concrete brick or stone ballast 1-1/2 to 2” guage 1:5:10.(S.I.No:4,P;17) |  |  |  |  |  |
| 150,cft: | @ | Rs:8694/95 |  | %.Cft. | Rs:13042/- |  |
|  | 3/- Pacca brick work in foundation and plinth in cement sand mortar 1:6 (S.I No.4, P/25) |  |  |  |  |  |
| 352,cft: | @ | Rs:11948/40 |  | %cft: | Rs:42058/- |  |
|  | 4/-Damp proof course with cement (cement sand and shingle concrete 1:2:4 i/c 2 coat of asphaltic mixtue. |  |  |  |  |  |
| 113 | 3”thick @ | 4982/18 |  | %Sft: | Rs:5630/- |  |
|  | 5/-Pacca brick work ground floor in cement sand ratio 1:6 |  |  |  |  |  |
| 225,cft: | @ | Rs:12674/40 |  | %cft: | Rs:28517/- |  |
|  | 6.RCC work in roof slab beam columns rafts lintels and other structural member laid in situ or precast laid in positin complete in all respects ratio 1:2:4(90,lbs:cement 2,cft: sand 4cft¨shingle 1/8”to ¼” guage |  |  |  |  |  |
| 36,Cft: | @ | Rs:337/- |  | P.Cft: | Rs:12132/- |  |
|  | 7/-Fabrication of mild steel reinf:for cement concrete i/c cutting bending laying in position making joints and fastening i/c the cost of binding wire and also i/c removal of rust from bars. |  |  |  |  |  |
| 1.446,Cwt: | @ | Rs:5001/70 |  | P.Cwt: | Rs:7232/- |  |
|  | 8/-Cement plaster ½”thick upto 20”height rati 1:6. |  |  |  |  |  |
| 150,sft: | @ | Rs:2206/60 |  | %Sft: | Rs:3310/- |  |
|  | 9/-Cement plaster 3/8”thick upto 20”height rati 1:4. |  |  |  |  |  |
| 150 Sft | @ | Rs:2197/52 |  | %. Sft | Rs:3296/- |  |
|  | 10/-Cement concrete brick or stone ballast 1-1/2”to 2” guage 1:6:12 ratio. (S.I No. 4(D)- P/14). |  |  |  |  |  |
| 198 Cft | @ | Rs:8122/95 |  | % Cft | Rs:16083/- |  |
|  | 11/- Lying floor of approved coloured glazed tiles 1/4" thick laid in white & pigment cement 1:2 over 3/4" thick cement mortar 1:2 complete. (S.I No. 25 - P/42) |  |  |  |  |  |
| 270 Sft. | @ | Rs.27747/06 |  | %.Sft. | Rs:74917/- |  |
|  | 12/- P/L in 1" thick cement concrete topping 1:2:4 i/c surface finishing & dividing into panels. (S.I No. 16 - P/41) |  |  |  |  |  |
| 265 Sft | 3” thick @ | Rs.4411/82 |  | %.Sft | Rs:29780/- |  |
|  | 13/-First class deodar wood wrought joinery in doors and windows etc fixed in position i/c chowkats hold fasts iron tower bolts handle cleats cords with hooks etc deodar paneled or paneled and galzed or fully glazed.(b)1 ¾”Thick (only Shutter) |  |  |  |  |  |
| 92,sft: | @ | Rs:902/93 |  | P.Sft: | Rs:83070/- |  |
|  | 14/- Distempering 3 coats.(S.I.No:24,bP:No:53) |  |  |  |  |  |
| 1680,sft: | @ | Rs:1079/65 |  | %.Sft. | Rs:18138/- |  |
|  | 15/-Colour washing two coats. |  |  |  |  |  |
| 1500 Sft | @ | Rs:859/90 |  | %.Sft. | Rs:12899/- |  |
|  |  |  |  | Total | Rs:352011/- |  |
|  |  |  | Total :- |  |  | Rs. 605164/- |

Part-B W/S & S.F

|  | 1/- P/F orisa type white glazed earthen ware w.c pan with cost of low level plastic flush tank of 3-gallon capacity of approved quality i/c making requisite No:of holes in wall,plinth and floor and making good in cc 1:2:4 w.c pan orisa type 23”with plastic tank or low down 3-gallon and C.I thumble (S.I No. 3(a)(ii), P-2) |  |  |  |
| --- | --- | --- | --- | --- |
| 02 Nos. | @ | Rs.5836/60 | Each | Rs:11673/- |
|  | 2/- P/F 18”x12”(47.72x30.49cm)lavatory basin in white glazed earthen ware complete with & i/c cost of W.I or C.I cantilever 6”built into walls painted white in two coat after primary coat of red lead paint a pair of ½”dia chrom platted pillar rubber plug and chrome plated pillar traps 1-1/2”dia rubber plug and chrome brass waste of approved pattern 1-1/4”dia malleable iron cp brass unions making and making traps malleable iron or brass required No:of holes in walls plinth and floor for pipe connection and making good in CC 1:2:4 (standard pattern) (S.I No.15, P-5) |  |  |  |
| 1,No: | @ | Rs:5467/- | Each | Rs:5467/- |
|  | 3/-Providing and fixing of earthen ware pedestal white or colour glazed standard pattern. |  |  |  |
| 1,No: | @ | Rs:938/47 | Each: | Rs:938/- |
|  | 3/-P/F 6"x2" or 6"x3" C.I floor trap of approved self cleaning design with C.I screwed down gratting with or without a vent arm complete i/c making requisite number of holes in walls, plinth & floor for pipe connections & making good in c.c 1:2:4. (S.I No. 20, P-06) |  |  |  |
| 03 No. | @ | Rs.2042/43 | Each | Rs.6127/- |
|  | 4/- P/F in position nyloon connections complete with 1/2" dia, brass stop cock with pair of brass nuts & lining joints to nyloon connections. (S.I No. 23, P-6) |  |  |  |
| 02 No. | @ | Rs:447/15 | Each | Rs:894/- |
|  | 5/- S/F long bib-cock of superior quality with crystal head 1/2" dia. (S.I No. 13(b), P-19) |  |  |  |
| 04 No. | @ | Rs.1384/24 | Each | Rs:5537/- |
|  | 6/- P/F handle volves (China). (S.I No. 5(i), P-17) |  |  |  |
| 01 No. | 3/4” dia @ | Rs.271/92 | Each | Rs.272/- |
|  |  |  | Total: | Rs:30909/- |

Part –C Public Healt .

|  | 1/-Borring for tube well in all water bearing soil from ground level upto 100,cft: or 30.5 meter deoth i/c sinking and with drawing of casing pipe. |  |  |  |
| --- | --- | --- | --- | --- |
| 60,Rft: | @ | Rs:160/- | P.Rft: | Rs:9600/- |
|  |  |  | Total | Rs:9600/- |
|  | Part D Non-Schedule items |  |  |  |
|  | 1/-Providing & fixing UPVC fitting (PAK-ARAB) SCH-40 on surface by using clips /saddler /socket /reducer /bush /tee /elbow etc or recessed in masonary, C.C or R.C.C upto 20ft height & making good with C.C , including curing, finishing etc complete as per instructions of the Engineer Incharge. Specifications of the material should meet the requirements of class -B in accordance with the ASTM-D-1784 type 1 grade 1. Rate includes all costs of labour, material, cartage, scaffolding/ladder etc complete. (R/A). |  |  |  |
| 20Rft | 6” dia @ | Rs:385/- | P.Rft | Rs:7700/- |
| 50Rft | 3/4” dia @ | Rs.92/- | Each | Rs:4600/- |
| 60,Rft: | 1” dia @ | Rs.115/- | Each | Rs.6900/- |
| 20 Rft | 4” dia @ | Rs.290/- | Each | Rs. 5800/- |
|  |  |  |  | Rs.25000/- |

| (CONTRACTOR) |  | EXECUTIVE ENGINEER BUILDINGS DIVISION SHAHEED BENAZIR ABAD |
| --- | --- | --- |