Scheduled -B

| Name of Work :- | Reconditioning of road from Jamali Disposal to Village Phaat via Jamali Goth Including Masonry Structure mile 0/0-1/7 (3.0 K.m). |
| --- | --- |

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

PART-I-(FORMATION).

01. Earth work for embankment by bulldozer including ploughing mixing, clod breaking dressing and compacting with optimum moisture content, lead Upto 100ft: and lift upto 5ft: in all types of soil except rock.

(A) COMPACTING UPTO 85%MODIFIED AASHO DENSITY.

| 334298 | Cft | @ | Rs: | 3656.23 | P.%0 Cft | Rs: | 1222270.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

PART-(II)-DIAGONAL GROVES).

01. Making diagonal groves of ½”x1-1/2” at 2 FT: centre to centre in road surface.

| 122364 | Sft | @ | Rs: | 146.61 | P.% Sft | Rs: | 179398.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

PART-III-(BRICK EDGING/BASE COURSE).

01. Laying brick on end edging including supplying 9”x4-1/2”x3” first class bricks, excavation for having edging with small size parallel to the road (Rate includes lead upto site of work).

| 13596 | Rft | @ | Rs: | 2482.94 | P. % Rft | Rs: | 337581.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage upto site of work).

| 67122 | Cft | @ | Rs: | 5252.41 | P. % Cft | Rs: | 3525523.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Total Part-iii. |  | Rs: | 3863104.00 |

PART-IV-(SURFACE TREATMENT).

01. Providing surface dressing Ist coat on new or existing surface with 30 lbs of bitumen and 4.0 Cft of Crush bajri of required size including cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage upto site of work.

| 122364 | Sft | @ | Rs: | 1520.45 | P. % Sft | Rs: | 1860483.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Providing 1½”thick (Consolidated) premixed carpet in proper camber and grade including supplying 15 Cft bajri 5 Cft hill sand (of approved quality and grade) 93 lbs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion, including heating materials and cleaning the road surface (Hill sand 3 Cft for mixing and 2.0 Cft for dusting (Rate includes all cost of materials T&P and carriage upto site of work (With Paver).

| 183546 | Sft | @ | Rs: | 6488.32 | P. % Sft | Rs: | 11909052.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Total Part-iv |  | Rs: | 13769535.00 |

PART-VI-MASONRY STRUCTURE).

1. Retaining Wall to the Right Side of

| 300 | Rft | @ | Rs: | 6710.74 | P.Rft | Rs: | 2013222.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

2. 3’Span Culverts.

| 02 | Nos | @ | Rs: | 200000/- | Each. | Rs: | 400000.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3. Drain Cross. |  |  |  |  |  |  |  |
| 04 | Nos. | @ | Rs: | 31000/- | Each | Rs: | 124000.00 |
|  |  |  |  |  | G. Total | Rs: | 2537222.00 |

PART-VII-MISCELLENEOUS.

| 01. | Scheme Board. | 01 | No. | Rs: | 16956.00 | Each | Rs: | 16956.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02. | Sign board. | 05 | Nos. | Rs: | 14600.00 | Each | Rs: | 73000.00 |
|  |  |  |  |  |  | G.Total | Rs: | 89956.00 |