| Name of Work :- | Construction/Widening and Reconditioning of road from Khairpur Luqman road via Maitla Mohalla towards Sim Nala at Abra Village mile 0/0-0/1+330’. |
| --- | --- |

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

PART-I-(FORMATION).

01. Earth work for embankment by bulldozer including ploughing mixing, clod breaking dressing and compacting with optimum moisture content, lead Upto 100ft: and lift upto 5ft: in all types of soil except rock.

(A) COMPACTING UPTO 85%MODIFIED AASHO DENSITY.

| 79883 | Cft | @ | Rs: | 3656.23 | P.%0 Cft | Rs: | 292071.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

(B) COMPACTING UPTO 95-100%MODIFIED AASHO DENSITY.

| 20394 | Cft | @ | Rs: | 6190.17 | P.%0 Cft | Rs: | 126242.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

03. Excavation in existing berm for widening the road, including preparation of Sub grade after watering, rolling with power Roller, dressing the excavated stuff etc complete.

| 6118 | Cft | @ | Rs: | 526.27 | P. % Cft | Rs: | 32197.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

04. Providing Sand Cushion including supplying and spreading Pit/Canal Sand of approved quality from approved Source of supply to site of work including watering and rolling etc, complete Rate includes all cost of materials T&P labour and carriage of 3-0 Miles (Average)

| 3059 | Cft | @ | Rs: | 1479.41 | P. % Cft | Rs: | 45255.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Total | Rs: | 495765.00 |

95765.00E I/C DIAGONAL GROVES).3 and ends at over Head bridge road at Village Abra.00000000000000000000000000000000000000000000

PART-(II)-(SUB BASE COURSE I/C DIAGONAL GROVES).

01. Making Diagonal Groves of ½”x1 ½” at 2’Centre to centre road surface.

| 12236 | Sft | @ | Rs: | 146/61 | P.% Sft | Rs: | 17939.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Preparing Sub-base by supplying and spreading stone metal 1-1/2”-2”guage of approved quality from approved quarry in required thickness to proper camber and grade including hand packing filling voids with 10 Cft screening & non plastic quarry fines of approved source, watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate includes all cost of materials T&P and carriage upto site of work.

| 13766 | Cft | @ | Rs: | 4359.76 | P.% Cft | Rs: | 600165.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Total | Rs: | 618104.00 |

PART-III-(BRICK EDGING/BASE COURSE).

01. Laying brick on end edging including supplying 9”x4-1/2”x3” first class bricks, excavation for having edging with small size parallel to the road (Rate includes lead upto site of work).

| 2039 | Rft | @ | Rs: | 2482.94 | P. % Rft | Rs: | 50627.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage upto site of work).

| 18354 | Cft | @ | Rs: | 5252.41 | P. % Cft | Rs: | 964027.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Total Part-iii. |  | Rs: | 1014654.00 |

PART-IV-(SURFACE TREATMENT).

01. Providing surface dressing Two coat on new or existing surface with 48 lbs of bitumen of 80/100 Penetration and 5 ½” Cft of Crush bajri with i/cs cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage upto site of work.

| 36709 | Sft | @ | Rs: | 2378.77 | P. % Sft | Rs: | 873223.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Providing 1½”thick (Consolidated) premixed carpet in proper camber and grade including supplying 15 Cft bajri 5 Cft hill sand (of approved quality and grade) 93 lbs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion, including heating materials and cleaning the road surface (Hill sand 3 Cft for mixing and 2.0 Cft for dusting (Rate includes all cost of materials T&P and carriage upto site of work (With Paver)

| 36709 | Sft | @ | Rs: | 6488.32 | P. % Sft | Rs: | 2381797.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Total Part-iv |  | Rs: | 3255020.00 |

PART-V-(PAVEMENT MARKING).

1.Pavement Marking in reflective Tharma Plastic Paint for lines of 6”Width               (S.I.No.13 P-No.15).

| 7920 | Rft | @ | Rs: | 41.24 | P.Rft | Rs: | 326621.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

PART-VI-MASONRY STRUCTURE).

| 01. | Retaining Wall. | 1400 | Rft | Rs: | 6710.74 | P.Rft | Rs: | 9395036.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02. | 3’Span Culvert. | 02 | No | Rs: | 20000.00 | Each | Rs: | 400000.00 |
|  |  |  |  |  |  | Total | Rs: | 9795036.00 |

PART-VII-MISCELLENEOUS.

| 01. | Scheme Board. | 01 | No. | Rs: | 16956.00 | Each | Rs: | 16956.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02. | Sign board. | 06 | Nos. | Rs: | 14600.00 | Each | Rs: | 87600.00 |
|  |  |  |  |  |  | Total | Rs: | 104556.00 |