Scheduled- B

| Name of Work :- | Construction/Reconditioning of road from Khairpur Pasco Food Godown to Wapda Grid Station via Railway Station Luqman mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

MEASUREMENT CUM ABSTRACT.

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

PART-I-(FORMATION).

01. Excavation in foundation of building, bridges and other structure i/c degbelling, dressing, refilling around structure with excavated earth watering and ramming lead upto one chain and lift upto 5’(S.I.No.18b, P-5).

| 85655 | Cft | Rs:1361.25 | P.%0 Cft | Rs:116598.00 |
| --- | --- | --- | --- | --- |

02. Preparing, Sub Grade including earth excavation or filling to an average depth

Of 9”dressing to camber and consolidation with power Roller.

| 38069 | Cft | Rs:526.28 | P.% Cft | Rs:20035.00 |
| --- | --- | --- | --- | --- |

03. Providing Sand Cushion including supplying and spreading Pit/Canal Sand of approved quality from approved Source of supply to site of work including watering and rolling etc, complete Rate includes all cost of materials T&P labour and carriage of 3-0 Miles (Average)

| 47586 | Cft | Rs:1479.41 | P.% Cft | Rs:703992.00 |
| --- | --- | --- | --- | --- |
|  |  |  | Total | Rs:840625.00 |

PART-II-(SUB BASE COURSE/DIAGONAL GROVES).

01. Preparing Sub-base course by supplying and spreading Stone metal 1- ½”-2”guage of approved quality from approve quarry in required thickness to proper camber and grade including hand packing mixed with shingle Pit run gravel fines having P.I of not more than 6% in the 2:3 including.

| 38069 | Cft | Rs:3380.43 | P.% Ct | Rs:1286896.00 |
| --- | --- | --- | --- | --- |

02. Making diagonal groves of ½”x1-1/2” at 2 FT: centre to centre in road surface.

| 74778 | Sft | Rs:146.61 | P.% Sft | Rs:109632.00 |
| --- | --- | --- | --- | --- |

03. Preparing Sub-base by supplying and spreading stone metal 1-1/2”-2”guage of approved quality from approved quarry in required thickness to proper camber and grade including hand packing filling voids with 10 Cft screening & non plastic quarry fines of approved source, watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate includes all cost of materials T&P and carriage upto site of work.

| 28552 | Cft | Rs:4359.76 | P.%Cft | Rs:1244799.00 |
| --- | --- | --- | --- | --- |

Total Part-II-	 Rs:2641327.00

PART-III-(BRICK EDGING/BASE COURSE).

01.Laying brick on end edging including supplying 9”x4-1/2”x3” first class bricks, excavation for having edging with small size parallel to the road (Rate includes lead upto site of work).

| 6798 | Rft | Rs:2482.94 | P. % Rft | Rs:168790.00 |
| --- | --- | --- | --- | --- |

02. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage upto site of work). (S.I.No.10, P-4).

| 56423 | Cft | Rs:5252.41 | P. % Cft | Rs:2963567.00 |
| --- | --- | --- | --- | --- |

| Total Part-III. | Rs:3132357.00 |
| --- | --- |

PART-IV-(SURFACE TREATMENT).

01. Providing surface dressing Two coat on new or existing surface with 48 lbs of bitumen of 80/100 Penetration and 5.5 Cft of Crush bajri with i/cs cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage upto site of work.

| 112847 | Sft | Rs:2731.99 | P. % Sft | Rs:3082969.00 |
| --- | --- | --- | --- | --- |

02. Laying to proper line and grade plant mixed asphalt Concrete paver finished (Hydralic/Electronic control) prepared to specified formula according to job mix formula Asphalt by Engineer incharge i/c rolling and finishing to proper line grade level and camber etc complete. Rate i/cs all cost of materials of bitumen 80/100 penetration and all cost of materials rolling tent and carriage of materials (2”thick).

| 112847 | Sft | Rs:8855.63 | P. % Sft | Rs:9993313.00 |
| --- | --- | --- | --- | --- |

Total Part-IV- Rs:13076282.00

PART-V-(PAVEMENT MARKING).

1. Pavement Marking in reflective Tharma Plastic Paint for lines of 6”Width(S.I.No.13 P-No.15).

| 19800 | Rft | Rs:41.24 | P.Rft | Rs:816552.00 |
| --- | --- | --- | --- | --- |

PART-VI-MASONRY STRUCTURE).

| P/fixing C.C.Paver blocks on and at berms (8030 Nos) | 8030 | Nos | Rs:692.00 | P. Sft | Rs:5556760.00 |
| --- | --- | --- | --- | --- | --- |
| Const:of New Drain Nala | (1320 Rft) | Rft | Rs:2947.19 | P.Rft | Rs:3890300.00 |
| Installation of Energy Solar light Poles | 25 Nos) | Each | Rs:187000.00 | Each | Rs:4675000.00 |
| Pipe Culverts(15 Nos) | 15 | Nos | Rs:55000.00 | Each | Rs:825000.00 |
| Repairing of Foot Path Slab (1450 Rft) | 1450 | Rft | Rs:1870.65 | P.Rft | Rs:2712500.00 |
|  |  |  |  | Total | Rs:17659560.00 |

PART-VII-MISCELLENEOUS.

1.Shifting of Electric Poles 70 Nos		Rs:200000.00  Each   Rs:14000000.00

2. Scheme Board      01 No.			Rs:16956.00	Each 	Rs:16956.00

3. Sign board.	         10 Nos.			Rs:14600.00	Each 	Rs:146000.00__

Total 	Rs:14162956.00