Secton:-AB/TC/

SCHEDULE “B”

Name of Work: -	(8) Construction of Surface Drains I/C C.C. Topping and Brick Pavement for Rural Drainage Scheme Halani (Zone-C) Halepota Mohalla Taluka Mehrabpur District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I.		SURFACE DRAINS.

1.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (CSI No.18 P.No.4).

Qty:21772.00 Cft Rs.3176/25 	P.%0 Cft				Rs.69153/=.

2.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (CSI No.5 P.No.17) 1.

Ratio  1:4:8.

Qty: 3508.00  cft at Rs.11288/75 P.% cft 			         		Rs.396009/=

3.	Pacca Brick Work in foundation upto Plinth in cement 							sand mortar.( Ratio 1:6) (CSI No:3 e P.No:25).

Qty: 14546.25 cft  at  Rs.11948/36 P.% cft 	       				Rs.1738038/=

4.	Construction of Standard Open Drain cunette block of Cement 				concrete (1:2:4) cost in situ to design profile i/c cost of mold as 					per drawing i/c applying floating coat of cement 1/32” thick be					the exposed face finished along with curing etc complete as per				detailed drawing (P.H.S.I No.  .D  P.No.44).

Type I:     Qty: 2600.00 Rft @  Rs.94/00  	P.Rft.					Rs.244400/=

Type II:    Qty: 2580.00 Rft @  Rs.174/00  	P.Rft.					Rs.448920/=

6.	Cement plaster 1:4 upto 20’ height ½” thick (CSI No.11 P.50).

Qty: 13607.00 sft @  Rs.2283.93  P.% sft.   	       			 	Rs.310788/=

7.	RCC work i/c all labour  and material  except the  cost of  steel

Reinforcement and its labour for bending binding which  will be

Paid separetely. This rate also including all kind of forms moulds

Lifting shuttering curing rendering and finishing to be exposed

Surface i/c Screening and washing of Stone.RCC work in Roof

slab beams columns rafts lintels and other structure member

laid in situe or pre-cost laid in position Complete in all respects

(I) Ration 1:2:4 lbs cement 2 cft sand 4 cft shingle 1/8" to 1/4"

Guage (CSI  No.6  Page No.49).

Qty: 2536.00 Cft at Rs.337/= 		P.cft 		          			Rs.854885/=

8.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.7 P.No.18).

Qty: 90.598 Cwt. at Rs.5001/70    P/Cwt.	       				Rs.453145/=

Total 	Rs.4515339/=

PART-II.	CC Topping .

1.	Barrow Pit excavation un dressed lead upto 100 ft ordinary Soil

(CSI No.3  Page No.1).

Qty: 1228.00  cft at RS.2117/50 P.%0 cft.					Rs.2600/=

2.	Earth from out side lead 1 mile.

Qty: 1228.00  cft at RS.407/= P.% cft.					Rs.4998/=

3.	Earth work compaction (soft ordinary and hard soil) (a) Laying earth					in 6” layers leveling and dressing etc.

Qty: 23000.00  cft at RS.263/= P.%0 cft.					Rs.6049/=

4.	Cement concrete brick or sotne ballast 1/2" to 2" guage Ratio 1:4:8						(CSI No. (4) Page No.17) Ratio 1;4:8.

Qty:3036.00  cft @ RS.9416/28    	P% Cft.				Rs.285878/=

5.	Cement concrete plain including placing complete and curing complete					i/c screening and watering of stone aggregate without shuttering and 					washing of stone 1:2:4 (GSI No. 5 page No.18).

Qty: 2300.00 cft @ RS.14429/25    	P% Cft.				Rs.331873/=

Total	Rs.631398/=

PART-III.	BRICK PAVEMENT.

1.	Barrow Pit excavation un dressed lead upto 100 ft.

(CSI No.3  Page No.1).

Qty:24000.00  cft at RS.2117/50 P.%0 cft.					Rs.50820/=

2.	Earth Work compaction soft ordinary or hard soil(d) ramming earth

work all type of soil i.e laying earth 6"layers levelling and dressing etc

complete (CSI No.13 (a) P-03).

Qty: 24000.00  cft @ RS.263/=    	P%0 Cft.				Rs.6312/=

3.	Earth from outside (Lead 01 miles).

Qty:24000.00 cft @ RS.407/=     	P% cft.					Rs.97680/=

4.	Dry brick on edge paving sand grouted i/c preparation of bed by watering					ramming and bringing the same to proper camber by 1/2" thick mud plaster				(CSI No.5 Page No.46).

Qty:16000.00 Sft @ RS.3823/57     	P% sft.					Rs.611771/=

Total	Rs.766583/=

ABSTRACT OF SCHEDULE B”.

| S.NO. | NAME OF WORK | SCHEDULE ITEMS | NON SCHEDULE ITEMS | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | Surface Drains | RS.4515339/= | - | RS.4515339/= |
| 2. | CC Topping. | RS.631398/= | - | RS.631398/= |
| 3. | Brick Pavement. | Rs.766583 | - | Rs.766583 |
|  | Total | Rs.5913320/= | - | Rs.5913320/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE