Secton:-AB/TC/

SCHEDULE “B”

Name of Work: -	(5) Providing Laying Jointing and Teting P.E Pipe Delivery Main 12” Dia for Urban Water Supply Scheme Bhiria city Taluka Bhiria District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

1.	Dismantling and removing road metalling (CSI No. 51 P-13)

Qty:75.00 cft @ Rs.605/00			P%cft.				RS.454/=

2.	Excavation for pipe line in trenches and pits in soft soil i/c clay or mud trimming

and dressing sides to true alignment shape leveling of beds of trenches to correct

level & grade cutting joints holes i/c laying earth in 6” layers for construction of

banks & banks dressing disposal of surplus earth within one chain as directed

by Engineer Incharge providing fence guards lights flags temporary crossing

for non vehicular traffic where ever required lift upto 5’ft: and lead upto

one chain (PHSI No: 1 P-60)

Qty:31200 cft @ Rs.3600/00			P%0cft.				RS.112320/=

3.	Excavation for pipe line in trenches and pits in wet soil i/c clay or mud trimming and dressing

sides to true alignment shape leveling of beds of trenches to correct level & grade cutting joints

holes i/c laying earth in 6” layers for construction of banks & banks dressing disposal of surplus

earth within one chain as directed by Engineer Incharge providing fence guards lights flags

temporary crossing for non vehicular traffic where ever required lift upto 5’ft: and lead upto

one chain (PHSI No: 14 P-66)

Qty:31200 cft @ Rs.5400/00			P%0cft.				RS.168480/=

4.	Providing, Laying & fixing in trench i/c fitting, jointing & testing etc complete

in all respect the high Density Polythylene PE Pipes (HDPE-100) for W/S

confirming ISO 4427/DIN8074/8075 B.S 3580 & PSI 3051 (PHSI No. F (i) P-25).

Qty:5200.00  Rft @ Rs.1635/00			P.Rft.				RS.8502000/=

5.	Supplying of High Density Polyethlene Fitting (PE-100) (Per Each)

Equivalent make (SMI No. ix P-104).

Elbow 45*

12" dia (315 mm)

Qty:1.00 Nos @ Rs.10753/00			P.Each.				RS.10753/=

Elbow 90°

12" dia (315 mm)

Qty:3.00 Nos @ Rs.13441/00			P.Each.				RS.40323/=

Butt Fusion Joint

12"Dia

Qty:4.00 Nos @ Rs.2200/00			P.Each.				RS.8800/=

6.	Refilling the excavated stuff in 6” thick layers watering ramming to

full compaction etc complete (PHSI No: 24 P-77)

Qty:62400 cft @ Rs.2760/00			P.%0 cft.			RS.172224/=

Total	RS.9015354/=

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE