..Secton:-AB/TC/

SCHEDULE “B”

Name of Work: -	(9) Supplying Installing Pumping Machinery i/c Inter connection, PE Pipe Rising Main 6” Dia and Construction of Oxidation Pond i/c sump well and solar energy system  for Village Dewan Rural Drainage Scheme Dewan and Sawan Sahito Taluka Mehrabpur District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I	PUMPING MACHINERY I/C INTER CONNECTION

1.	Supplying and installing in position i/c transportation to site @ Disposal Work for Rural Drainage scheme Dewan Electric Motor 7.50 BHP (Siemens made) 4 Pole 1500 RPM 3-phase 400 VAC 50 cycles coupled with KSB Non clogging Sludge pump  type KWPZ-65-315 size 3”x2 ½” i/c KSB Steel frame and coupling with sleeve etc complete capable of giving discharge 250 gallons per minute against a head of 45’ft i/c installation of pumping set in cement concrete foundation ratio 1:2:4 with MS Nuts & Bolts to be embedded in CC Foundation i/c internal wiring etc complete providing internal electric wiring main or sub main PVC insulated with size 3-7 /0.44 (6 mm2& testing the pumping set at depth required head of 72 Hours etc complete.

Qty:  01 Set			@ Rs. 654000/-	P.Set			Rs.654000/-

2.	Supplying and installing Diesel oil Engine 16 BHP (Golden Made) model G-D 1110L 2200 RPM couple with Golden Non clogging Sullage pump Type GSWG-104 size 5”x4” capable discharging 250 GPM against a head of 45’ft i/c frame, pully for Engine & Pump with belt and installing of Pump Set in CC Foundation with nuts and bolts and testing of the Pumping Set against the required head at 48 Hours.

Qty: 01 Set			@ Rs. 207000/-	P-Set			Rs.207000/-

Total		Rs.861000/-

B-	INTER CONNECTION:-

1.	P.E Pipe HDPE…PN-10

For suction

i) 3” Dia 90mm

Qty: 12.0 Rft			@ Rs.178/-			P.Rft		Rs.2136/-

ii) 6”dia 160mm De1ivery

Qty: 30.0 Rft			@ Rs.530/-			P.Rft		Rs.15900/-

2.	Supplying C.I Bend of Flanged ends with holes i/c turning and

facing of flanges for all size (S.M.I No.8 Page.11)

3”Dia 90* & 6”Dia 90*

Qty: 1.18 cwt			@ Rs. 6096/-			P-Cwt		 Rs. 7193/-

3.	Supplying C.I. Tee with flanged ends with holes i/c turning and

facing of flanges for all size (S.M.I No.11 Page.11)

6”Dia

Qty: 0.46	Cwt		@ Rs. 6096/-			P-Cwt		 Rs. 2804/-

4.	Supplying C.I Tapered flate  bottomed or central teperd  Flanged ends with

holes i/c turning and facing of flanged for all size.

(S.M.I No.9 Page.11)

Qty: 0.80 Cwt			@ Rs. 6096/-			P-Cwt		 Rs. 4877/-

5.	Maniufacturing and installing M.S pipes 3”Dia made out of sheet 3/16”

thick.

M.S Pipes 3”Dia

Qty: 8.00 Rft.			@ Rs. 385/89			P-Rft.		Rs. 3087/-

6.	Maniufacturing and installing M.S pipes 6”Dia made out of sheet 3/8”

thick.

Qty: 8.00 Rft.			@ Rs. 749/34			P-Rft.		Rs. 5995/-

7.	Manufacturing & installing 3”&6”dia MS flange made out of M.S sheet 3/8” thick cutting drilling holes etc complete.

3”dia

Qty:4 Nos			@ Rs.500/89			Each		Rs.2004/-

6”Dia

Qty:12 Nos			@ Rs. 804/77			Each		Rs.9657/-

8.	Supplying C.I sluice value heavy pattern test pressure.

(PHSI of material 1 No.2 P-97) 6”Dia

Qty: 02 Nos			@Rs. 9360/-			Each		Rs. 18720/-

9.	Supplying C.I Reflex value heavy pattern test pressure.

(PHSI of material 1 No.2 P-97) 6”Dia

Qty: 2 Nos			@ Rs.4062/50			Each		Rs. 8125/-

10.	Supplying of foot valve heavy pattern test pressure. 3”Dia

Qty: 1 No			@ Rs. 731/28			Each		Rs. 731/-

6”Dia

Qty: 1 No			@ Rs. 1381/25		Each		Rs. 1381/=

11.	Butt fusion joint.

3”Dia

Qty  4Nos			@ Rs.600/-			Each		Rs.2400/-

6”Dia

Qty 12 Nos			@ Rs.1000/-			Each		Rs.12000/-

12.	Supplying flange adapter (Non sch: Item)..

3”Dia

Qty 4 Nos			@ Rs.675/-			Each		Rs.2700/-

6”Dia

Qty 12 Nos			@ Rs.2025/-			Each		Rs.24300/-

13.	Jointing C.I / M/S flanged pipe or special flanged ends inside

a trenches i/c supplying rubber packing of the required thickness

nuts bolts with washers etc and other tools required for Jointing

and Testing to the specified pressure etc complete (PHSI No.1 P.35)

3” Dia

Qty: 4.0 Joints			@ Rs.499/-			P/Joint		Rs. 1996/-

6” Dia

Qty: 12.0 Joints		@ Rs.938/-			P/Joint		Rs.11256/-

4” Dia

Qty: 2.0 Joints			@ Rs.513/-			P/Joint		Rs.1026/-

= = = = = =

TOTAL	RS.138288/-

PART-II     Providing laying jointing & Testing P.E Pipe Rising Main

1.	Excavation for pipe line in trenches and pits in soft soil i/c clay or mud trimming and dressing sides to true alignment shape leveling of beds of trenches to correct level & grade cutting joints holes i/c laying earth in 6” layers for construction of banks & banks dressing disposal of surplus earth within one chain as directed by Engineer Incharge providing fence guards lights flags temporary crossing for non vehicular traffic where ever required lift upto 5’ft: and lead upto one chain (PHSI No: 1 P-60).

0-5’ depth

6”dia.

Qty: 14000.00 Cft			@ Rs.3600/-			P%0 cft		 Rs.50400 /-

2.	Providing, Laying & fixing in trench i/c fitting, jointing & testing etc complete

in all respect the high Density Polythylene PE Pipes (HDPE-100) for

W/S confirming ISO 4427/DIN8074/8075 B.S 3580 & PSI 3051 (PHSI No. F (i) P-25).

6”dia (160 mm).

Qty: 2000.00 Rft			@ Rs.440/-			P.Rft		 Rs.880000/-

3.	Supplying of High Density Polyethlene Fitting (PE-100) (Per Each) Equivalent make                                  (SMI No. ix P-104)

Elbow 90”

6” dia (160 mm).

Qty 02 Nos			@ Rs.3480/-				Each		Rs.6960/-

Elbow 45”

6” dia (160 mm).

Qty 02 Nos			@ Rs.2784/-				Each		Rs.5568/-

But fusion Joints”

6” dia (160 mm).

Qty 04 Nos			@ Rs.1000/-				Each		Rs.4000/-

4.	Refilling the excavated stuff in 6” thick layers watering ramming to full

compaction etc complete (PHSI No: 24 P-77)

Qty: 12600.00 cft		@ Rs.2760/-				P.Rft		 Rs.34776/-

Total	Rs.991704/=

PART-III.	OXIDATION POND (SIZE 150 X 40) 2 NOS /SUMP WELL.

1.	Excavation for tanks & reservoir in soft soil i/c trimming and dressing to true

alignment & shape leveling of beds of trenches to correct level & grade

cutting joints holes disposal surplus earth within one chain as directed by

Engineer Incharge providing fence guards lights flags temporary crossing for non

vehicular traffic where ever required lift upto 5’ft: and lead upto one chain

(PHSI No: 1 P-67)

Qty: 51240.00 cft		@ Rs.3000/-				P%0ft		 Rs.153720/-

2.	Excavation for tanks & reservoir in Wet soil clay or mud i/c trimming and

dressing to true alignment & shape leveling of beds of trenches to correct

level & grade cutting joints holes disposal surplus earth within one chain as

directed by Engineer Incharge providing fence guards lights flags temporary

crossing for non vehicular traffic where ever required lift upto 5’ft: and lead

upto one chain (PHSI No: 14 P-52)

Qty: 6800.00 cft		@ Rs.4800/-				P%0ft		 Rs.32640/-

3.	Earth work compaction (Soft, ordinary and hard soil) (b) Laying earth in 6" layers

levelling and dressing watering for compaction etc completed (CSI No. 13 (b) P-3)

Qty: 50000.00 cft		@ Rs.354/-				P%0ft		 Rs.17700/-

4.	Extra for every 50’ft additional lead (2600RFT) (lead 10 mile)or part thereoff

(CSI No. 13 (b) P-3)

Qty: 8040.00 cft		@ Rs.1008/80				P%0cft		 Rs.8103/-

5.	Excavation for pipe line in trenches and pits in soft soil i/c trimming and dressing

sides to true alignment shape leveling of beds of trenches to correct level & grade

cutting joints holes i/c laying earth in 6” layers for construction of banks & banks

dressing disposal of surplus earth within one chain as directed by Engineer Incharge

providing fence guards lights flags temporary crossing for non vehicular traffic where

ever required lift upto 5’ft: and lead upto one chain (PHSI No: 1 P-60)

Qty: 960.00 cft		@ Rs.3600/=				P%0ft		 Rs.3456/-

6.	Providing RCC ASTM Pipe C-76-62 T/C-70 Class II wall "B" class and fixing in

trenches i/c cutting fitting and jointing with rubber rings i/c testing with water to

specified pressure (PHSI No. b (i) P-17).

Qty: 64.00 Rft		@ Rs.412/=				P.Rft		 Rs.26368/-

7.	Cement concrete brick or stone ballast 1 1/2" to 2" gauge Ratio 1:4:8

(GSI No. 5 P-15)

Qty: 16.58 cft		@ Rs.9416.28				P%cft		 Rs.1562/-

8.	Excavation in foundation of building bridges and other structures I/c dag belling

dressing refilling around the structures with excavated Earth watering ramming

lead up to one chain and lift up to 5’ (GSI No. 18 (b) P-04)

Qty: 99.49 cft		@ Rs.3176.25				P%0cft		 Rs.316/-

9.	R.C.C work  in roof slab beams columns rafts lintels and other structural

members laid in situ or prrecast laid in position complete in all respect.

(I) Ratio 2:2:4 Lbs cement 2 Cft, sand 4 cft shingle1/8” to ¼” gauge (GSI No. 6 P-49)

Qty: 68.39cft		@ Rs.337/=				P.cft		 Rs.23048/-

10.	Fabrication of mild steel reinforcement  for C.C i/c cutting,  bending, laying

in position making joints and fastening i/c cost of binding wire(also i/c

removal of rust from bars) (GSI No. 8 (b) P-17)

Qty: 2.75 cwt		@ Rs.5001/70				P.cwt		 Rs.13755/-

11.	Refilling the excavated stuff in 6” thick layers watering ramming to full

compaction etc complete (PHSI No. 24 P-77)

Qty: 864.00 cft		@ Rs.2760/=				P%0cft		 Rs.2385/-

Total	RS.283053/=

PART-IV.	SOLAR POWER SYSTEM FOR 7.5 HP SUBMERSIBLE PUMP.

1.	Solar Modules (Kaneka Japan) Thin Film 140 walt x 60 Panels (25 Years

Warranty of solar Modules against manufactures defect). ABB (German)

VFD MPPT Solar Inverter x 1 (1 Year of ABB Inverter against manufactures

defect). G.I Sheet (14 Guage) Panel Frames x 30 A.C / D.C wires, circuit

breakers, distribution boards, Insulation and other installation Materials

(for 140 feet distance from Panel to Inverter). Transportation and Installation

to side included. Civil works is not included). (Note:- This module is design

for 4 -5 peak hours, & Solar Modules is design for 80% efficiency). (R.A Attached)

Qty: 01 Set		@Rs.1300000/-	P-Set				Rs.1300000/-

2.		Construction of C.C Foundation 1:3:6 wall (R.A Attached)

Qty: 184.68 Rft		@Rs.815/-		P-Rft			Rs.150514/-

----------------

Total	Rs.1450514/-

ABSTRACT OF SCHEDULE B”.

| S.NO. | NAME OF WORK | SCHEDULE ITEMS | NON SCHEDULE ITEMS | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | Pumping Machinery i/c I/Connection. | RS.135588/= | Rs.863700/= | Rs.999288/= |
| 2. | P.E Pipe Rising Main | Rs.991704/= | - | Rs.991704/= |
| 3. | Oxidation Pond i/c Sump Well. | Rs. 283053/= | - | Rs.283053/= |
| 4 | Solar System. | Rs.NIL | Rs.1450514/= | Rs.1450514/= |
|  | Total | Rs.1410345/= | Rs.2314214/= | Rs.3724559/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE