Secton:-AB/TC/

SCHEDULE “B”

Name of Work: -	(6) Construction of Storage Tank, i/c Sump Well and Pump House for Urban Water Supply Scheme Bhiria city  Taluka Bhiria District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I.	STORAGE TANK I/C SUMP WELL.

1.	Jungle Clearance & removing within 100'ft.(CSI No.4 P-95).

Qty: 60000.00 sft @ Rs.75/63		P%0 sft.			Rs.4538/=

STORAGE TANK.

2.	Excavation for tanks and reservoirs in Soft soil / clay or mud i/c   trimming

and dressing sides to true alignment/design sections  profiles and shape

leveling of beds of tanks to correct level and grade  i/c laying of earth in

6”layer for construction of banks and dressing  and disposal of surplus.

Excavated earth within one chain as   directed by Engineer In charge i/c

providing fence guards. Flags where ever required lift upto 5 feet (1.52 M)

and lead upto one chain (30.5 M).  ( PHS.I No. 1 Page 49).

Qty: 119988.00 Cft @ Rs.3600/00		P%0 cft.			Rs.431957/=

3.	Excavation for tanks and reservoirs in wet soil / clay or mud i/c   trimming

and dressing sides to true alignment/design sections  profiles and shape

leveling of beds of tanks to correct level and grade  i/c laying of earth in

6”layer for construction of banks and dressing  and disposal of surplus.

Excavated earth within one chain as   directed by Engineer In charge i/c

providing fence guards. Flags where ever required lift upto 5 feet (1.52 M)

and lead upto one chain (30.5 M).  ( PHS.I No. 1 Page 49).

Qty: 146310.00 Cft @ Rs.4800/00		P%0 cft.			Rs.702291/=

4.	Earth work compaction (Soft, ordinary and hard soil) (b) Laying earth in 6"

Layers levelling  and dressing watering for compaction etc completed

(CSI No. 13 (b) P-3)

Qty: 80370.00 Cft @ Rs.354/00		P%0 cft.			Rs.28451/=

5.	Excavation in foundation of building bridges and other structure i/c Dag

belling dressing refilling around the structure with excavated Earth watering

and ramming etc complete lead upto one chain lift Upto 5’ft. (GSI No: 18 P/4).

Qty:752.25.00 Cft @ Rs.3176/25			P%0 cft.			Rs.2389/=

6.	Cement concrete brick or stone ballast 11/2”to 2”guage

.(CSI No.(b) P.No.16)Ratio 1:4:8

Qty:1113.33 Cft @ Rs.9416/28			P% cft.				Rs.104834/=

7.	Pacca brick work in foundation upto plinth in cement sand Motor.1: 6. (CSI NO: 05 P/44).

Qty: 2040.09 Cft @ Rs.11948/36			P% cft.				Rs.243757/=

8.	C.C Line 1:3:6

1. Slope.

Qty: 3670..30 Cft @ Rs.11920.43			P% cft.				Rs.437516/=

2. Beds apron.

Qty: 18274.09  Cft @ Rs.11481/80			P% cft.				Rs.2098194/=

9.	Brick lining (9”x41/2” x 3”) in cement sand mortor (CSI No.8 P-94)1:3

Qty: 13656.50  Cft @ Rs.13765/81			P% cft.				Rs.1879928/=

10.	Brick lining (9”x41/2” x 3”) in cement sand mortor (CSI No.8 P-94)1:4

Qty: 5434.73  Cft @ Rs.14204/44			P% cft.				Rs.771973/=

11.	Cement plaster 1:4 upto 20’height ½ ‘thick. (GSI No: 11 P/50).

Qty: 2052.35 sft @ Rs.2283/93			P% sft.				Rs.46872/=

12.	Dry brick on edge paving sand grouted i/c prepartion of bed by watering

ramming and bringing the same to proper camber by 1/2" thick mud plaster (CSI No. 5 P-46)

Qty: 6378.00 sft @ Rs.3823/57			P% sft.				Rs.243867/=

13.	Cement pointing struck joints on walls (CSI No.19 P.580).

Qty: 14688.50 sft @ Rs.1213/58			P% sft.				Rs.178257/=

SUMP WELL 10’DIA.

14.	Excavation in foundation of building bridges and other structures I/c

dag belling dressing refilling around the structures with excavated

Earth watering ramming lead up to one chain and lift up to 5’ (GSI No. 18 (b) P-04)

Qty: 981.30 sft @ Rs.3176/25			P%0 sft.			Rs.3117/=

15.	Extra for wet earth work (GSI No.15 P-3)

Qty: 245.30 sft @ Rs.1058/75			P%0 sft.			Rs.260/=

16.	Providing RCC Pipes of ASTM C-76.62 t/c 76-70 wall B and  fixing

trenches i/c fitting and jointing with rubber ring i/c testing with water

to specified pressure (PHSI No.1 Page No.18).

Qty: 120.0 Cft @ Rs.412/00				P%0 sft.			Rs.49440/=

17.	C.C plain i/c placing compacting finishing and curing complete i/c

Screening and Washing of Stone Aggregate without shuttering

(CSI No.5 Page NO.17).Ratio    1:4:8.

Sump well 7” dia.

Qty: 61.32.0 Cft @ Rs.11288/75			P% sft.				Rs.6922/=

18.	R.C.C Work i/c all labour and material except the cost of steel  reinforcement and

its labour for bending and binding of forms mould lifting shuttering curing rendering

and finishing the exposed  surface i/c Screening and Washing of shingle

RCC Work in roof slabs beams rafts lintels columns and other structural members

laid in situ for  pre cast laid in position in all respects etc complete (Ratio 1:2:4).

Qty: 305.06 Cft @ Rs.337/00			P. cft.					Rs.102805/=

19.	Fabrication of Mild Steel reinforcement concrete for CC i/c Cutting,

Bending Laying in position making joints and fastening i/c Cost of binding

wire also including removal of rust from bars (CSI No.8 P.No.16).

Qty: 12.25 Cwt @ Rs.5001/70		P. cwt.					Rs.61271/=

INTER CONNECTION.

20.	Excavation for pipe line in trenches in  soft soil i/c trimming and dressing to

true alignment & shape leveling of beds of trenches to correct level & grade

cutting joints holes disposal surplus earth within one chain as directed by

Engineer Incharge providing fence guards lights flags temporary crossing for

non vehicular traffic where ever required lift upto 5’ft: and lead upto one chain

(PHSI No: 1 P-49)

Qty: 1440.00 Cft @ Rs.3600/00		P%0 cf t.				Rs.5184/=

21.	Providing RCC ASTM Pipe C-76-62 T/C-70 Class II wall "B" class and

fixing in trenches i/c cutting fitting and jointing with rubber rings i/c testing

with water to specified pressure (PHSI No. 1 (b) P-17).

Qty: 120.00 Rft @ Rs.412/00		P.Rft						Rs.49440/=

22.	Refilling excavated stuff in trenches as 6”thick layers i/c watering ramming

to full compacting etc complete (PHSI No.24 P.No.77).

Qty: 1296.00 cft @ Rs.2760/00	P.%0 cft					Rs.3577/=

Total	Rs.7407400/=

PART-II.	PUMP HOUSE.

1.	Excavation in foundation of building bridges and other structures I/c dag belling

dressing refilling around the structures with excavated Earth watering ramming

lead up to one chain and lift up to 5’ (GSI No. 18 (b) P-04)

Qty: 214.00 cft @ Rs.3176.25/00	P.%0 cft					Rs.680/=

2.	Cement concrete brick or stone ballast 1 ½” to 2” gauge Ratio 1:4:8

(GSI No. 4 (b) P-15)

Qty: 100.91 cft @ Rs.9416.28 	P.% cft						Rs.9502/=

3.	Pacca brick work in foundation of upto plinth cement, sand, mortar Ratio 1:6

(GSI No. 4 P-23)

Qty: 338.08  cft @ Rs.11948/36  	P.% cft						Rs.40395/=

4.	Damp proof course with cement sand & shingle concrete 1:2:4 i/c two coats

asphalt mixture 2"thick (GSI No. 7 P-19)

Qty: 57.60cft @ Rs.3912/85   	P.% cft						Rs.2254/=

5.	Pacca brick work (1:6) in ground floor cement, sand, mortar (GSI No. 1 P-20)

Qty: 342.75 cft @ Rs.12674/36   	P.% cft						Rs.43441/=

6.	R.C.C work  in roof slab beams columns rafts lintels and other structural members

laid in situ or prrecast laid in position complete in all respect. (I) Ratio 2:2:4 Lbs

cement2 Cft, sand 4 cft shingle1/8” to ¼” gauge

(GSI No. 6 (b) P-17)

Qty: 115.66 cft @ Rs.337/=   	P.cft							Rs.38977/=

7.	Fabrication of mild steel reinforcement  for C.C i/c cutting,  bending, laying in

position making joints and fastening i/c cost of binding wire(also i/c removal of

rust from bars) (GSI No. 8 (b) P-17)

Qty: 4.64 cwt @ Rs.5001/70   	P.cwt						Rs.23208/=

8.	Filling water and ramming earth under floor with surplus earth excavated

from out side lead upto one chain and lift upto 5 ft: (CSI No.22 P-4)

Qty: 459.39 cft @ Rs.1512/50   	P%0cft						Rs.695/=

9.	Providing and laying 1” thick to toping Cement Concrete (1:2:4) I/c surface,

finishing and dividing in to panel (GSI No. 16 (c) P-42)

Qty: 140.00 sft @ Rs.4411/82   	P% sft						Rs.6177/=

10.	Fabrication of heavy steel work with angle tee flats iron round iron and sheet iron

for making tr uses griders tanks etc i/c cutting bending drilling reviting handing

assesmling and fixing etc complete (CSI No. 2 P-91)

Qty: 3.96 cwt @ Rs.4928/49   	P.cwt						Rs.19517/=

11.	Cement, Plaster (1:6) 1/2" thick  up to 12’ height  (GSI No.11 P-47)

Qty: 1223.67 sft @ Rs.2206/60   	P% sft						Rs.27002/=

12.	Preparing surface and painting guards bars gates of iron bars getting railing

i/c standard brasses etc and similar open works (CSI No 5 P-68) Two coats

Qty: 93.50 sft @ Rs.2116/41   	P% sft						Rs.1979/=

13.	Primary coat of chalk distempering.

Qty: 1223.67 sft @ Rs.442/75   	P% sft						Rs.5418/=

14.	Distembring two coats

Qty:1223.67 sft @ Rs.1043/90   	P% sft						Rs.12774/=

15.	Supplying and fixing in position in iron steel grill of 3/4" x 1/4" size flat iron of

approved design i/c painting Three coats etc complete (wieght not less then 3.7 Lbs

p.Sq ft of fininshing) (CSI No.31 P99)

Qty:24.00  sft @ Rs.180/50   	P% sft							Rs.4332/=

16.	Cement, Plaster (1:4) 3/8" thick  up to 12’ height  (GSI No.11 P-47)

Qty:1223.67  sft @ Rs.2197/52   	P% sft						Rs.26890/=

Total	Rs.263241/=

ABSTRACT OF SCHEDULE B”.

| S.NO. | NAME OF WORK | SCHEDULE ITEMS | NON SCHEDULE ITEMS | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | Storage Tank i/c Sump Well and i/connection | Rs.7407400/= |  | Rs.7407400/= |
| 2 | Pump House. | Rs.263241/= |  | Rs.263241/= |
|  |  | Rs. 7670641/= |  | Rs. 7670641/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE