Secton:-AB/TC/

SCHEDULE “B”

Name of Work: -	(4) Construction of CC Nala (2.5’ Width ) i/c Catch Pits from Mithiani chowk to Mula Hakeem Chowk to main drain for Urban Drainage Scheme Tharushah Taluka Bhiria District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I.	C.C. COVERED NALA (2.50 WIDTH).

1.	Dismantling cement concrete Plain 1:2:4 (CSI No.18 (c)P-10).

Qty:79.20 Cft			@Rs.3327/50		P%Cft			Rs.2635/-

2.	Dismantling cement concrete reinforcement separating reinforcement

From concrete cleaning and straightening the same (CSI NO.20 P-10)

Qty:49.50 Cft			@Rs.5445/-		P%Cft			Rs.2695/-

3.	Dismantling brick work in lime or cement mortor (CSI No.13 P-10)

Qty:150.0 Cft			@Rs.1285/63		P%Cft			Rs.1928/-

4.	Dismantling and removing road metalling. (GSI No.51 Page #13)

Qty: 7238.0 Cft		@ Rs.605/=		P%Cft			Rs. 43790/-

5.	Excavation for pipe lines in trenches and pits in soft soil i/c trimming

dressing sides to true alignment and shape leveling of beds of trenches

to correct level and grade cutting joint holes and  disposal  of  Surplus

Earth within  one chain as directed  by Engineer  Incharge  Providing

fence guards, lights, flags and temporary crossing  for  non  Vehicular

traffic wherever required lift up to 5’ft and lead up to one chain.

(P.H.S.I No.1 P.No.60).

### Qty:32571.0 Cft.		@ Rs.3600/-		P%0Cft.		Rs.117256/-

6.	Excavation for pipe lines in trenches and pits in Wet soil i/c trimming

dressing sides to true alignment and shape leveling of beds of trenches

to correct level and grade cutting joint holes and  disposal  of  Surplus

Earth within  one chain as directed  by Engineer  Incharge  Providing

fence guards, lights, flags and temporary crossing  for  non  Vehicular

traffic wherever required lift up to 5’ft and lead up to one chain.

(P.H.S.I No.14 P.No.66).

### Qty:7238.0 Cft.		@ Rs.5400/-		P%0Cft.		Rs.39085/-

7.	Add every 50’additional lead or part thereoff. (CSI No.8 P-13)(Lead 1 mile)

Qty: 43197.0 cft		@ Rs.407/00		P% cft			Rs.175812/-

8.	 Cement concrete plain including placing complete and curring

Complete i/c screening and watering of stone aggregate without

Shuttering and washing of stone 1:4:8. (GSI No: 5 P/17).

Qty: 3619.0 cft	 	  @Rs: 11288/75        	 P%CFT		RS:408540/-

Ratio 1:2:4

Qtty: 13954.25 cft	 	  @Rs: 14429/25        	 P%CFT		RS: 2014215/-

9.	Erection and removal of centering for R C C or Plain

CC work for partial Wood  (GCI No.19/B Page No.18).

Qty;  39095.0 Sft 		@ Rs.3127/41 	P% sft.			Rs.1222661/-

10.	RCC work in roof, slab, beams, columns, rafts, lintels, and other

Structural members laid in situ or pre cost laid in position complete.

In all respects (GSI NO.6 P.16)

Qty: 2061.22 Cft. 		@ Rs.337/00		P/Cft.			Rs. 694631/-

11.	Fabrication of mild steel rein forcemeat for cement concrete

i/c cutting, bending laying in position. Making joints and fastening

i/c cost of binding wire also includes removal of rust from bars

(Tore Bar) (CSI No. 7 P.20)

Steel provide @ Rs.4.50 Lbs/Cft of RCC

Qty:73.61 Cwt. 		@ Rs.5001/70		P/Cwt.			Rs.368175/-

12.	Small iron work such as gusset plate knees bends, stirrups, rings etc

i/c cutting drilling evetting handling, assembling and fixing but

excluding errection in position  (CSI NO.1 P.N.91).

Qty: 8.0 Cwt			@6420/61		P-Cwt			Rs.51365/-

13.	Supplying and mixing chemical admixture palatinates B.V 40

Equavalent mixing with CC 1:2:4 ML Platinates BV 40 with

Qty: 90.69 Litres		@Rs.80/-	     	P.Litre 			Rs.7255/-

14.	Refilling the excavated stuff in 6” thick layers watering ramming

to full compaction etc complete (PHSI No: 24 P-77)

Qty:3850.0 Cft		@Rs.2760/-		P%0Cft		Rs.10626/-

15.	Providing PVC Pipe 2”Dia (Non pressure) (SMI No.1 P-5)

2”Dia

Qty:280.0 Rft			@Rs.30/49		P-Rft			Rs.8537/-

Total	Rs.5169206/-

PART-II.	CATCH PITS (6 Nos).

1.	Excavation in foundation of building bridges and other structures I/c dag

belling dressing refilling around the structures with excavated Earth watering

ramming lead up to one chain and lift up to 5’ (GSI No. 18 (b) P-04)

Qty: 96.0 cft			@Rs.3176.25		P%0cft			Rs.305/-

2.	Cement concrete plain i/c placing compacting finishing and curing

complete i/c screening and washing of stone aggregate without shuttering

(GSI No. 5 P-16)

A-RATIO 1:4:8

Qty: 7.92 cft			@Rs.11288/80	P%cft			Rs.894/-

B-RATIO 1:2:4

Qty: 2.00 cft			@Rs.14429/30	P%cft			Rs.289/-

3.	Pacca brick work in foundation and plinth in cement, sand, mortar (1:6)

(GSI No 4 (E) P-25)

Qty: 46.11 cft			@Rs.11948/36	P%cft			Rs.5509/-

4.	Cement, Plaster (1:4) 1/2" thick  up to 20’ height  (GSI No.13 P-52)

Qty: 54.00 Sft			@Rs.2283/93		P%Sft			Rs.1233/-

5.	Providing RCC Pipe with collars and digging the trenches to required

depth and fixing to in position cutting and jointing with maxphalt

composition and cement mortor 1:1 and testing with rater pressure

etc a head executed stuff (PHSI No.1 P-15)

Qty: 8.00 Rft			@Rs.401/97		P-Rft			Rs.3216/-

6.	R.C.C work  in roof slab beams columns rafts lintels and other structural

members laid in situ or prrecast laid in position complete in all respect.

(I) Ratio 2:2:4 Lbs cement 2 Cft, sand 4 cft shingle1/8” to ¼” gauge

(GSI No. 6 P-49)

Qty: 6.35 cft			@Rs.337/00		P-cft			Rs.2141/-

7.	Fabrication of mild steel reinforcement  for C.C i/c cutting,  bending,

laying in position making joints and fastening i/c cost of binding wire

(also i/c removal of rust from bars) (GSI No. 8 (b) P-17)

Qty: 0.227 Cwt			@Rs.5001/70	P-cwt			Rs.1135/-

8.	Small iron work such as guessed in best plate knees bends strops sings

etc i/c cutting drilling riveting handling assembling and fixing excluded

errection and in position (Cover of catch pits) (CSI No.1 P-8)

Qty: 0.223 Cwt			@Rs.6420/61	P-cwt			Rs.1433/-

Total	RS.16155/=

Rs. 16155 x 6 =	Rs.96930/=

ABSTRACT OF SCHEDULE B”.

| S.NO. | NAME OF WORK | SCHEDULE ITEMS | NON SCHEDULE ITEMS | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | C.C Covered Nala | Rs.5169206/- |  | Rs.5169206/- |
| 2 | Catch Pits. (6 Nos). | Rs.96930/= |  | Rs.96930/= |
|  | Total | Rs.5266136/= | - | Rs.5266136/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE