SCHEDULE OF QUANTITIES

WORK No: 01

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	      8,32,595/-  		Earnest Money Rs:	         16,652/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | CLEANING BY WINCHING OF SEWER FROM LYARI EXPRESSWAY TO STREET No:17 BLOCK “B” SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Cleaning inside sewerage lines completely restoring original silt free space/diameter of pipe lines by labour and equipment. (Mechanical/electrically driven) including accessories like pulley’s steel rope, bucket and daggers sizing from 6” to18” dia. Their to and from pulling action (no of passes shall be as many as required) would be taken in two phase (First phase entire length from downstream to upstream) bringing down peack hours sewerage to flow inside pipe line ( as free flow) and in second phase from upstream to downstream of entire length ensuring no silt is observed in buckets (6” dia to 18” dia) except only sewage water and finally passing a steel ball of dia 12” lesser that the diameter of pipe but not exceeding 48” dia ball even for larger dia pipe to ensure perfect cleaning. The job including all the hire/cost of equipment and accessories of above winching drum set having, steel rope pully mounted over it steel bucket from 6” to 18” dia hooks and other protection likes safety barriers, traffic sings, traffic cones ensuring no damage to pipe along with ensuring safety to labour and other public property/lives and removal of silt/solid during desilting and clearance of site etc. complete as per full satisfaction of site engineer. ………………………………….….36” Dia……………..………………. | 2,873 Rft | 289.80 | P/Rft | 8,32,595.00 |
| Total Estimated Cost |  |  |  |  | 8,32,595.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 02

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 7,06,147 (+) O/R  		Earnest Money Rs:	 14,123 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | PROVIDING/LAYING R.C.C. PIPE DRAIN 12” DIA IN MAREE CHOWK UC-42, MACHER COLONY KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 6,000 Cft | 4650.00 | 1000 Cft | 27,900.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………………...…12” Dia………………..……...….……….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 44 Nos | 9307.86 | Each | 4,09,546.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 44 Rft | 1609.15 | P/Rft | ( - ) 70,803.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5,400 Cft | 2760.00 | 1000 Cft | 14,904.00 |
| 6. | Supply of Labour on Hiring Basis for lifting of Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharage. | 320 Nos. |  | Each |  |
| Total Estimated Cost |  |  |  |  | 7,06,147 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 03

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 8,15,308 (+) O/R  		Earnest Money Rs:	 16,306 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REMOVAL OF SLOGAN AND HIRING OF MACHINERY FOR LEVELING AND DRESSING OF FOOTPATH FROM GULBAI CHOWK TO MARINE ACADEMY KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Distempering Plaster surface 03 coats. | 38,894 Sft | 1079.65 | %Sft | 4,19,919.00 |
| 2. | Removal old debris and rubbish of all kinds and size within 03 miles. | 34,050 Cft | 1065.04 | %Cft | 3,62,646.00 |
| 3. | Painting surface and painting Iron work etc 02 coats. | 2,198 Sft | 1489.68 | %Sft | 32,743.00 |
| 4. | Hiring of 02 Nos. Tractor Blade for scraping / leveling, dressing and cleaning of road i/c the wage of Driver, Assistant and Fuel etc complete. | 48 Hours |  | P/Hour |  |
| 5. | Hiring of Labours for different work any types as per instruction of Engineer Incharge. | 100 Nos. |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,15,308 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 04

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 7,96,513 (+) O/R  		Earnest Money Rs:	 15,930 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA ZAROOBI COLONY IN UC-44, BHUTTA VILLAGE KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 6,000 Cft | 4650.00 | 1000 Cft | 27,900.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. …………………………………………12” Dia…………….………...….……….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 40 Nos | 9307.86 | Each | 3,72,314.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 40 Rft | 1609.15 | P/Rft | ( - ) 64,366.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5,400 Cft | 2760.00 | 1000 Cft | 14,904.00 |
| 6. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 20 Nos. | 1906.60 | Each | 38,132.00 |
| 7. | Shifting and fixing R.C.C. precast ring slab in C.C. 1:2:4 ratio. | 20 Nos. | 563.00 | Each | 11,260.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. …………………………………………12” Dia…………….………...….……….. | 73 Nos. | 913.63 | Each | 66,695.00 |
| 9. | Supply of Labour on Hiring Basis for lifting of Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharage. | 225 Nos. |  | Each |  |
| Total Estimated Cost |  |  |  |  | 7,96,513 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 05

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 8,20,589 (+) O/R  		Earnest Money Rs:	 16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN AT AMNA MASJID BLOCK-05, SIKENDERABAD UC-46, KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5,625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ……………………………….…12” Dia…………………...….……….. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5,063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,665.00 |
| 9. | Shifting and fixing R.C.C. precast ring slab in C.C. 1:2:4 ratio. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page 02

Page 02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………………………………..…21” Dia………………….…………….. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting of Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharage. | 195 Nos. |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,589 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 06

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 8,48,675 (+) O/R  		Earnest Money Rs:	 16,974 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN IN STREET No:16 TO 25 AT AKBER ROAD IN SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5,625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. …………………………………………12” Dia…………….………...….……….. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 45 Nos | 9307.86 | Each | 4,18,854.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 45 Rft | 1609.15 | P/Rft | ( - ) 72,412.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5,063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 6. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos. | 1906.60 | Each | 47,665.00 |
| 7. | Shifting and fixing R.C.C. precast ring slab in C.C. 1:2:4 ratio. | 25 Nos. | 563.00 | Each | 14,075.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. …………………………………………12” Dia…………….………...….……….. | 100 Nos. | 913.63 | Each | 91,363.00 |
| 9. | Supply of Labour on Hiring Basis for lifting of Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharage. | 165 Nos. |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,48,675 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 07

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 6,11,295 (+) O/R		Earnest Money Rs:	 12,226 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REPAIR OF NALLAH WITH CLEANING AT TOHEED MASJID UC-46, SIKENDERABAD KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 600 Cft | 5445.00 | %Cft | 32,670.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 17,880 Cft | 1065.04 | %Cft | 1,90,429.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 600 Cft | 337.00 | P/Cft | 2,02,200.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 18.75 Cwt | 5001.70 | P/Cwt | 93,782.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 430 Nos |  | Each |  |
| 6. | Cement concrete plain i/c placing compaction finishing and curing complete i/c screening and washing at stone aggregate without ratio 1:2:4. | 405 Cft | 14429.25 | %Cft | 58,438.00 |
| 7. | Erection and removal of centering for RCC or plain cement concrete works of Deodar Wood (2nd class). | 1,080 Sft | 3127.41 | %Sft | 33,776.00 |
| Total Estimated Cost |  |  |  |  | 6,11,295 (+) OPEN RATE |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 08

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 4,98,784 (+) O/R		Earnest Money Rs:	 9,976 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | REPAIR OF NALLAH AND CLEANING AT IBRAHIM MASJID IN UC-46, SIKENDERABAD KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 600 Cft | 5445.00 | %Cft | 32,670.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 9,240 Cft | 1065.04 | %Cft | 98,410.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 600 Cft | 337.00 | P/Cft | 2,02,200.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 18.75 Cwt | 5001.70 | P/Cwt | 93,782.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 555 Nos |  | Each |  |
| 6. | Cement concrete plain i/c placing compaction finishing and curing complete i/c screening and washing at stone aggregate without ratio 1:2:4. | 315 Cft | 14429.25 | %Cft | 45,452.00 |
| 7. | Erection and removal of centering for RCC or plain cement concrete works of Deodar Wood (2nd class). | 840 Sft | 3127.41 | %Sft | 26,270.00 |
| Total Estimated Cost |  |  |  |  | 4,98,784 (+) OPEN RATE |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 09

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 7,96,513 (+) O/R  		Earnest Money Rs:	 15,930 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA OKHAI COLONY IN UC-44, BHUTTA VILLAGE KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 6,000 Cft | 4650.00 | 1000 Cft | 27,900.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. …………………………………………12” Dia…………….………...….……….. | 800 Rft | 412.00 | P/Rft | 3,29,600.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 40 Nos | 9307.86 | Each | 3,72,314.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 40 Rft | 1609.15 | P/Rft | ( - ) 64,366.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5,400 Cft | 2760.00 | 1000 Cft | 14,904.00 |
| 6. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 20 Nos. | 1906.60 | Each | 38,132.00 |
| 7. | Shifting and fixing R.C.C. precast ring slab in C.C. 1:2:4 ratio. | 20 Nos. | 563.00 | Each | 11,260.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. …………………………………………12” Dia…………….………...….……….. | 73 Nos. | 913.63 | Each | 66,695.00 |
| 9. | Supply of Labour on Hiring Basis for lifting of Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharage. | 225 Nos. |  | Each |  |
| Total Estimated Cost |  |  |  |  | 7,96,513 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 10

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 8,46,205 (+) O/R  		Earnest Money Rs:	 16,924 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN IN STREET No:29, BLOCK “B” MUHAMMADI ROAD, SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5,625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 2. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. …………………………………………12” Dia…………….………...….……….. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 3. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching and channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 45 Nos | 9307.86 | Each | 4,18,854.00 |
| 4. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 45 Rft | 1609.15 | P/Rft | ( - ) 72,412.00 |
| 5. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5,063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 6. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 24 Nos. | 1906.60 | Each | 45,758.00 |
| 7. | Shifting and fixing R.C.C. precast ring slab in C.C. 1:2:4 ratio. | 24 Nos. | 563.00 | Each | 13,512.00 |
| 8. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. …………………………………………12” Dia…………….………...….……….. | 100 Nos. | 913.63 | Each | 91,363.00 |
| 9. | Supply of Labour on Hiring Basis for lifting of Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharage. | 165 Nos. |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,46,205 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 11

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	 4,34,436 (+) O/R		Earnest Money Rs:	 8,689 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | IMPROVEMENT OF NALLAH AL-FALAH MASJID IN UC-44, BHUTTA VILLAGE KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete R.C.C. (1:2:4) disposal lead upto 10 miles etc. | 500 Cft | 5445.00 | %Cft | 27,225.00 |
| 2. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10”x5”x3” or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 16,125 Cft | 1065.04 | %Cft | 1,71,738.00 |
| 3. | R.C.C. working roof slab beams, column, rafts, lintels and other structural members laid in situ or pre-cast laid in position complete in all respect 1:2:4. | 500 Cft | 337.00 | P/Cft | 1,68,500.00 |
| 4. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars) using tor bars. | 13.39 Cwt | 5001.70 | P/Cwt | 66,973.00 |
| 5. | Supply of Labour on Hiring Basis for lifting the Garbage and cleaning of Nallah/Sewerage System (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 627 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 4,34,436 (+) OPEN RATE |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 12

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	     9,98,184/-		Earnest Money Rs:	         19,964/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | HIRING OF MACHINERY AND LABOUR FOR CLEANING OF NALLAH IN MACHER COLONY UC-42, KEAMAIR ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Hiring of Machinery with POL. |  |  |  |  |
| i). | Front Loader / Excavator………………………….......... | 88 Hours | 3088.13 | P/Hour | 2,71,755.00 |
| ii). | Dumper (Hino)…………………………......................... | 352 Hours | 2006.90 | P/Hour | 7,06,429.00 |
| 2. | Hiring of Skilled Labour for Cleaning of Nallah for 8 Hours. | 20 Nos. | 750.00 | Each | 15,000.00 |
| 3. | Overtime of Private Labour after 8 Hours. | 40 Hours | 125.00 | P/Hour | 5,000.00 |
| Total Estimated Cost |  |  |  |  | 9,98,184.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 13

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	     9,50,420/-  		Earnest Money Rs:	         19,008/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L C.C. FLOORING JAMIA MASJID MADNI UC-40, MAURIPUR KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Earth work excavation undressed road cutting etc. | 6,875 Cft | 2208.37 | 1000 Cft | 15,183.00 |
| 2. | Cement concrete brick or stone ballast 1½” to 2” gauge (1:4:8). | 3,630 Cft | 9416.28 | %Cft | 3,41,811.00 |
| 3. | Providing and laying 2” thick topping and concrete (1:2:4) i/c surface finishing and dividing into panels. | 11,000 Sft | 3275.50 | %Sft | 3,60,305.00 |
| 4. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ……………………………………12” Dia…………….………...….….. | 120 Nos. | 913.63 | Each | 1,09,636.00 |
| 5. | Manufacturing and supplying of R.C.C. ring slab of 21” inside 36” outside 7.5” width and 6” thick i/c 3/8” for steel bars two concentric ring with 3/8” 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 50 Nos. | 1906.60 | Each | 95,330.00 |
| 6. | Shifting and fixing R.C.C. precast ring slab in C.C. 1:2:4 ratio. | 50 Nos. | 563.00 | Each | 28,155.00 |
| Total Estimated Cost |  |  |  |  | 9,50,420.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 14

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:	     9,98,008/-		Earnest Money Rs:	         19,960/-

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | HIRING OF MACHINERY AND LABOUR FOR CLEANING OF NALLAH IN SULTANABAD UC-45, KEAMAIR ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Hiring of Machinery with POL. |  |  |  |  |
| i). | Front Loader / Excavator………………………….......... | 80 Hours | 3088.13 | P/Hour | 2,47,050.00 |
| ii). | Dumper (Hino)…………………………......................... | 320 Hours | 2006.90 | P/Hour | 6,42,208.00 |
| 2. | Hiring of Skilled Labour for Cleaning of Nallah for 8 Hours. | 135 Nos. | 750.00 | Each | 1,01,250.00 |
| 3. | Overtime of Private Labour after 8 Hours. | 60 Hours | 125.00 | P/Hour | 7,500.00 |
| Total Estimated Cost |  |  |  |  | 9,98,008.00 |

Nomenclature, Quantities and Rates etc checked

and found correct.

EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE

I/We hereby quoted Rates @                           %  above/below the S/R.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address: