| ## NAME OF SCHEME:- | Reconditioning / Construction of Internal roads of Kotdiji City. |
| --- | --- |

(SCHEDULE ‘B’/ BOQ)

| QTY: | ITEM OF WORK | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |
|  | 1/-Dismantling & removing road metalling . |  |  |  |
| 1200 | Cft: | @Rs.605/- | P. %Cft: | Rs.7260/- |
|  | 2/-Cement concrete brick or stone ballast of 1 ½” – 2”guage of Ratio (1:4:8) |  |  |  |
| 600 cft |  | @Rs.9416/28 | P. % Cft: | Rs:56497/ - |
|  | 3/-Errection & removal of shuttering for RCC or CC Plain (2nd: Class) wood |  |  |  |
| 150 Cft: |  | @Rs:3127/41 | P.% Cft: | Rs:3909/- |
|  | 4/-Cement concrete plain i/cs: placing compacting finishing & curing complete i/cs: screening & washing of stone aggregate without shuttering ratio (1:2:4). |  |  |  |
| 94 Cft. |  | @Rs.14429/28 | P.%Cft. | Rs.13563/- |
|  | 5/-Crushed fire bricks grated as sand (S.I No:51 P/No:04) |  |  |  |
| 204 Cft |  | @ Rs.2500/- | P.% Cft: | Rs.5100/ |
|  | 6/-Supplying & filling sand under floor and plugging in walls (S.I No:29 P/No:31) |  |  |  |
| 204 Cft: |  | @Rs:151/25 | P.% Cft | Rs:308/- |
|  | 7/-Providing & fixing cement paving blocks flooring having size of 197x97x60 (mm) of city / quddra / cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/s: filling the joints with hill sand and laying in specified nammer / pattern and design etc. complete (CSI No:71 P/No.49) |  |  |  |
| 1200 Sft |  | @ Rs.199/77 | P. Sft: | Rs.239724/- |
|  | 8/-Extra lead for carriage of material of 100 Cft / 5 tons of all material like stone, aggregate spinal coat and etc. by truck or other vehicle (S.I No.8, P/No.02) (as per PHED schedule) |  |  |  |
| 1200 Cft. |  | @Rs.2059/94 | P.%0Cft. | Rs.2472/- |
|  |  |  | Total Rs:328833/- |  |
|  | 328833/100/12 = 274.02 P.Sft There for will be 111352x274.02 = Rs.30512675/= |  |  |  |

NOTE:-    Subject to any Changes/

Variation in Rates/

Quantity as per T.S.

___________ % Above/Below (Amount to be added/deducted)												Rs.__________________

Deduct (-)Difference Cost of Bitumen		Rs.`			      .

Grand Total:				Rs.__________________

Total Amount in Word: _____________________________________________

__________________________________________________________________

| Contractor | Executive Engineer |
| --- | --- |
|  | Highway Division |
|  | Khairpur. |