SCHEDULE-B:- 	ESTIMATE FOR UP-GRADATION OF SINDH GOVERNENT PRINTING PRESS @ KHAIRPUR

COMPOUND WALL

S.NO.	NAME OF WORK		.	QTY.	RATE.	UNIT.	AMOUNT.

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty:865.0 cft			@Rs:3176/25			% cft		RS:2747/-

2/- Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.17)

Qty:112.0 cft			@Rs:9416/28			% cft		RS:10546/-

Qty:61.0 cft			@Rs:8694/95			% cft		RS:5304/-

3/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty:14.775 cwt		@Rs:5001/70			P.cwt		RS:73900/-

4/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge.

Qty:313.0 cft		@Rs:337/-			P.cft		RS:105481/-

05/-	Random, rubble masonry (un-Coursed ) (iv)Ratio.   1:6S.I N: 1 (iv)P-27).

Qty: 300 Cft			@RS: 17723/-			% Cft		RS; 53169/-

06/-	Coursed Rubble masonry including hammer dressing in cement /sand mortar Ratio (1:6).

qty:360.0 cft			@Rs:25321/-			% cft		RS:91156/-

6/-	Pacca Brick work in other than building including sticking of joints upto 20’ height in (e) cement sand mortar ratio 1:6. (S.I.No.7(i-e) P.No.26)

Qty:431.0 cft		@Rs:12346/65		% cft		RS:53214/-

7/-	Cement Plaster 1:6 up to 20” ht: ½” thick.

Qty:1400.0 sft		@Rs:2206/60			% sft		RS:30892/-

8/-	Cement Plaster 1:4 up to 20” ht: 3/8hick.

Qty:1400.0 sft		@RS.2197/52			%sft		RS.30765/-

9/-	Colour Washing 02 coat.

Qty:1400.0 sft		@RS.1276/53			%sft		RS.17871/-

10/-	Cement pointing on stone work (Raised) in cement sand mortar Ratio (1:3)

Qty:400.0 sft		@Rs:1758/08			% sft		RS:7032/-____

Total		RS.432077/-

Cost of 100 Rft 508295/100 = 5082.95 P.Rft

Cost of  200 rftx5080.95		=	1016590/-			RS” 101660/-

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “B” STEEL GATE I/C GATE PILLAR.

| 1/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | .0 | 200 | CFT | 3176/25 | %0CFT | 635/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 38 | CFT | 8694/95 | %CFT | 3304/- |

| 3/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 200 | CFT | 337/- | P.CFT | 67400/- |

| 4/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NET QTY: | 395 / 112 | 3.527 | Cwt | 5001/70 | P.CWT | 17641/- |

| 5/- | Making & fixing steel grated doors complete with locking arrangement angle iron frame 2" x 2" 3/8"and 3/4" sq: bars 4" centre to center. (S.I.No.28 P-91). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 140 | SFT | 594/57 | P.SFT | 83240/- |

| 6/- | P.N.S, Painting guard bars gates, iron bars railing etc. complete. (S.I.No. P- . |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 140 | SFT | 1270/83 | %SFT | 1779/- |

|  |  | RS: | 173999/- |
| --- | --- | --- | --- |

CONTRACTOR					   EXECUTIVE ENGINEER

PROVINCIAL BUILDINGS DIVISION

SHAHEED BENAZIR ABAD