SCHEDULE-B

PROVIDING, LAYING, JOINTING & TESTING PVC PIPE 6’’,4’’ & 3’’ DIA FOR IMPROVEMENT AND EXTENSION OF WATER SUPPLY SCHEME KANTIO TALUKA CHACHRO DISTRICT THARPARKAR

| 1. | Dismantling and removing road mantling (G.S.I. No: 51 P-13). |
| --- | --- |

101.0 Cft		@ Rs. 605/=			P% Cft		Rs. 611/=

| 2. | Providing PVC pipe of class “B” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” Joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. (P.H.S.I. No. 1 P-22). |
| --- | --- |

6" Dia

1500.0 Rft		@ Rs. 262/=			P.Rft			Rs. 3,93,000/=

4" Dia

1000.0 Rft		@ Rs. 137/=			P.Rft			Rs. 1,37,000/=

3" Dia

1500.0 Rft		@ Rs. 90/=			P.Rft			Rs. 1,35,000/=

| 3. | C.I sluice valve heavy pattern test pressure 21 kg/sq:cm or 300 lbs/sq: inch. (P.H.S. of Mat: Item No. 2 P-9). |
| --- | --- |

6" Dia

1 No:			@ Rs. 9360/=		Each			Rs. 9,360/=

4" Dia

1 No:			@ Rs. 5460/=		Each			Rs. 5,460/=

3" Dia

2 Nos:			@ Rs. 4290/=		Each			Rs. 8,580/=

| 4. | PVC “Z” Joint fitting (P.H.S.O.M. Item No. 16 P-28). |
| --- | --- |

(i)	PVC Tee  6" dia

1 No.			@ Rs. 1800/=    		Each			Rs. 1,800/=

(ii)	PVC Tee  4" dia

1 No:			@ Rs. 1181/=    		Each			Rs. 1,181/=

(ii)	PVC Tee  3" dia

2 Nos:			@ Rs. 900/=    		Each			Rs. 1,800/=

(iii)	PVC Reducer 4" dia

3 Nos:			@ Rs. 225/=    		Each			Rs. 675/=

(vi)	End Plug 3" dia

4 Nos:			@ Rs. 175/=    		Each			Rs. 700/=

(v)	Valve socket 3" dia

4 Nos:			@ Rs. 350/=    		Each			Rs. 1,400/=

(vi)	Valve socket 4" dia

4 Nos:			@ Rs. 500/=    		Each			Rs. 2,000/=

(vii)	Valve socket 6" dia

4 Nos:			@ Rs. 1650/=    		Each			Rs. 6,600/=

(viii)	C.I Gibault Joint 3" dia

6 Nos:			@ Rs. 568/75    		Each			Rs. 3,413/=

(ix)	C.I Gibault Joint 4" dia

6 Nos:			@ Rs. 731/25    		Each			Rs. 4,388/=

(x)	C.I Gibault Joint 6" dia

6 Nos:			@ Rs. 1062/-    		Each			Rs. 6,372/=

(xi)	PVC Bend 6" dia

1 No:			@ Rs. 1913/=    		Each			Rs. 1,913/=

(xii)	PVC Bend 4" dia

1 No:			@ Rs. 956/=    		Each			Rs. 956/=

(xiii)	PVC Bend 3" dia

1 No:			@ Rs. 731/=    		Each			Rs. 731/=

| 5. | Cast iron special for A.C pressure pipe (Standard Weight A.C.I.L “B” Class) (P.H.S.O.M.I No:10 P-12). |
| --- | --- |

(i)	Short Piece/Tail Piece

6" dia

2 Nos:			@ Rs. 618/75    		Each			Rs. 1,238/=

4" dia

2 Nos:			@ Rs. 487/50		Each			Rs. 975/=

3" dia

4 Nos:			@ Rs. 375/=   	 	Each			Rs. 1,500/=

| 6. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including i/c applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

6" dia

16 Joints		@ Rs. 938/=			P.Joint		Rs. 15,008/=

4" dia

16 Joints		@ Rs. 513/=			P.Joint		Rs. 8,208/=

3" dia

16 Joints		@ Rs. 499/=  		P.Joint		Rs. 7,984/=

| 7. | Making joints to PVC specials fitting including laying of specials and cost of solvent cement of required diameter and testing the joints with along with PVC pipe “B, “C” & “D” class to the specied pressure and making good o all leaky joints etc complete.. (P.H.S.I.No. 1 P-41). |
| --- | --- |

6" dia

11 Joints		@ Rs. 96/=  			P.Joint		Rs. 1056/=

4" dia

10 Joints		@ Rs. 70/=  			P.Joint		Rs. 700/=

3" dia

5 Joint		@ Rs. 59/=  			P.Joint		Rs. 295/=

| 8. | Excavation for pipe line trenches and pits in sandy soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags, temporary crossings for non vehicular traffic where ever required lift upto 5 ft (1.52 m) and lead upto one chain (30.5 m). (P.H.S.I. No. 2 P-60). |
| --- | --- |

6" dia,	4" dia	& 3" dia

26813.0 Cft		@ Rs. 3750/=		6P‰ Cft		Rs. 1,00,549/=

| 9. | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete. (P.H.S.I.No.24 P-77). |
| --- | --- |

24132.0 Cft		@ Rs. 2760/=		P‰ Cft		Rs. 66,604/=

| 10. | Providing chamber 3' x 2' (915 x 615 mm) inside dimension 4 ½' (1372 mm) deep as per approved design for sluice valve 3" to 12" dia with 18" (457 mm) dia inside cost iron cover and frame (wt = 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick (with 5 Lbs steel per cft) 9" (299 mm) thick brick masonry wall set in 1:6 cement mortar 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25 mm) thick cement concrete 1:2:4 flooring ½" (12.5 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S foot rest at every one foot beyond 2 ½ ft depth curing, excavation, back filling and disposal of surplus earth etc complete. (P.H.S.I.No. 1 P-49). |
| --- | --- |

3 Nos:			@ Rs. 18820/=		Each			Rs. 56,460/=

Total	Rs. 9,83,517/-

CONDITIONS

- The rates should be inclusive of all the Taxes i/c. Sales Tax, Octroi. Royalties etc, if any.

- No separate carriage will be allowed to the contractor.

- The Testing of the works/Pumping Machinery/Pipe will be arranged by the contractor at his own cost, No separate payment will be made.

- No premium will be allowed on Non-Schedule items.

- Any error or omission in the specifications/items will be governed by the relevant specifications and schedule of rates.

- The decision of the Executive Engineer will be final and binding on all the parties in any dispute.

- The contractor will have to submit the certificate of manufacturer at the time of supply of Machinery.

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |