SCHEDULE-B

REPAIR OF R.C.C RESERVOIR 18’ DIA I/C ROCK WALL CIVIL STRUCTURE AND ELECTRIC FITTING FOR IMPROVEMENT AND EXTENSION OF WATER SUPPLY SCHEME KANTIO TALUKA CHACHRO DISTRICT THARPARKAR

ABSTRACT OF COST

PART-A	Repair of RCC Reservoir 18’ dia		Rs. 3,49,260/-

PART-B	Rock Wall Civil Structure			Rs. 2,75,000/-

PART- C 	Electric Fitting					Rs. 91,767/-

Total	Rs. 7,16,027/-

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |

PART-A	REPAIR OF EXISTING RCC RESERVOIR 18’ DIA

| 1. | Dismantling cement concrete reinforced separating rein for cement from concrete cleaning and straightening the same. (G.S.I No: 20 P-10) |
| --- | --- |

300.00 Cft 		@ Rs. 5445/= 		P% Cft		Rs. 16,335/=

| 2. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) RCC work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. (G.S.I.No.6 P-16). |
| --- | --- |

457.58 Cft		@ Rs. 337/=			P.Cft			Rs. 1,54,204/=

| 3. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (G.S.I.No.7 (b) P-16) (USING TAR BARS). |
| --- | --- |

20.427 Cwt		@ Rs. 5001/70		P.Cwt			Rs. 1,02,170/=

| 4. | Cement Plaster (1:3) upto 20’ height ¾” thick (G.S.I.No: 10 (c) P-51) (For Water Proofing). |
| --- | --- |

662.00 Sft		@ Rs. 2795/30 		P% Sft			Rs. 18,505/=

| 5. | Providing and fixing C.I .Man Hole cover with frame i/c cost of material (P.H.S.I. No:1 P-35). |
| --- | --- |

3.0 Cwt		@ Rs. 6985/=		P.Cwt			Rs. 20,955/=

| 6. | Providing/ Fixing C.I ventilator (Weight not less than 80 – Lbs/ Each) in tank roof of approved design with water light joint i/c painting etc complete. (P.H.S.I.No 4 P-55). |
| --- | --- |

2 Nos:			@ Rs. 4870/=		Each			Rs. 9,740/=

| 7. | Supplying C.I Tail Piece with one end flanged and other end spigot (for all size) (P.H.S.of Mat: I.No. 5 P-11). |
| --- | --- |

0.50 Cwt		@ Rs. 6096/=		P.Cwt			Rs. 3,048/=

| 8. N.S.I | Supplying and embedding C.I Footsteps of approved design weight not less than 15 Lbs (R.A. Approve). |
| --- | --- |

10 Nos:		@ Rs. 965/-			Each			Rs. 9,650/=

| 9. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. (G.S.I. No: 16 (c) P-42). |
| --- | --- |

575.0 Sft		@ Rs. 2548/29		P% Sft			Rs. 14,653/=

Total		Rs. 3,49,260/=

PART-B	ROCK WALL OUT SIDE COMPOUND WALL AND PUMP HOUSE.

| 1. | Preparing the surface and applying rock wall/ shield (Natural wall texture) coating to provide durable crust to wall, thickness b/w 2mm to 32mm (1/8’’) with acrylic co-polymer emulsion, selected marble chips, adhesive and bactericides, water resistance and fire and termite resistance (upto 20’-0 height). (G.S.I No. 43 P-55) |
| --- | --- |

6105.0 Sft		@ Rs. 4504/50		P% Sft			Rs. 2,75,000/=

Total	Rs. 2,75,000/=

PART-C	ELECTRIC FITTING

| 1. | Wiring for light or fan point with 3/.029 PVC insulated wire in 20mm (3/4’’) PVC conduit on surface as required. (Item No. 123 P-15) |
| --- | --- |

PUMP HOUSE 3 NOS

21.0 Point		@ Rs. 797/-			P.Point		Rs. 16,737/=

| 2. | Wiring for Plug point with 3/.029 PVC insulated wire in 20mm (3/4’’) PVC conduit on surface as required. (Item No. 125 P-15) |
| --- | --- |

5.0 Point		@ Rs. 669/-			P.Point		Rs. 3,345/=

| 3. | Providing & laying (Main or Sub Main) PVC insulated with size 2-7/.029 copper conductor in ¾’’ dia PVC conduit recessed in the wall or column as required. (Item No. 10 P-02) |
| --- | --- |

250.0 Mtr		@ Rs. 222/-			P.Mtr			Rs. 55,500/=

| 4. | Providing & Fixing brass battern holder swiveling type. (Item No. 233 P-34) |
| --- | --- |

18.0 Nos.		@ Rs. 69/-			P.No			Rs. 1,242/=

| 5. | Providing & Fixing Bakelite ceiling rose with two terminals. (Item No. 228 (B) P-33) |
| --- | --- |

3.0 Nos.		@ Rs. 72/-			P.No			Rs. 216/=

| 6. | Providing & Fixing circuit breaker 6,10,15,20,30,40,50 & 63 amp SP (TB5S) on prepared board as required. (Item No. 203 P-31) |
| --- | --- |

4.0 Nos.		@ Rs. 916/-			P.No			Rs. 3,664/=

| 7. | Providing & Fixing Brass ceiling fan 56’’ (Good quality). (Item No. 235 P-34) |
| --- | --- |

3.0 Nos.		@ Rs. 3185/-			P.No			Rs. 9,555/=

| 8. | Providing & Fixing one way SP 10/15 amp switch surface type. (Item No. 218 P-33) |
| --- | --- |

26.0 Nos.		@ Rs. 58/-			P.No			Rs. 1,508/=

Total	Rs. 91,767/-

CONDITIONS

- The rates should be inclusive of all Taxes i/c. Sales Tax, Octroi. Royalties etc, if any.

- No separate carriage will be allowed to the contractor.

- The Testing of the works/Pumping Machinery/ pipe will be arranged by the contractor at his own cost, No separate payment will be made.

- No premium will be allowed on Non-Schedule items.

- Any error or omission in the specifications/ items will be governed by the relevant specifications and schedule of rates.

- The decision of the Executive Engineer will be final and binding on all the parties in any dispute.

- The contractor will have to submit the certificate of manufacturer at the time of supply of Machinery.

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |