- SCHEDULE (B)

- Name of work:-Construction of shel terless two roomed Primary school in

- District Ghotki.2012-13 programme @ GPS Khair Mohammad Mahar Taluka Daharki.

| S:NO | DESCRIPTION | QTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |

01 Excavation in foundation of Building bridges and other                                                                                    structure dog belling dressing refilling around structure with                                                                                          excavated earth lead up to one chain and lift up

to 5’(S:I:NO:18bPNO:4 )

- =2448.0 cft:	3176/25 P%cft				 Rs:7775/=

- 02. Cement concrete brick or stone ballast 11/2”x2” ratio 1:5:10 (S:I:NO:               )

- =853.0cftx 8694/95 p%cft				Rs:74168/=

- Ratio:1:6:12

- =289.0 cft x8122/95 P%cft				Rs:23475/=

03. Pacca brick work in foundation with cement sand                    			     	                                                             mortar 1:6(S:I:NO:4e P:NO21)

- =   1785.0 cft x11948/36 P%cft    			  Rs:213278/=

- 04. Reinforced cement concrete work i/c all labor and material                                                                         Except the cost of steel for its bending and binding which will                                                                         be paid separate . This rate also i/c all kinds of forms moulds                                                                                     lifting shuttering curing and rendering the exposed surface                                                                                              .RC work in roof slab beams columns rafts lintels laid in                                                                            position complete in all respects.(S:I:NO:6A P:NO:16)

=258.0cftx337/=Pcft					Rs:86946/=

05. Fabrication of mild steel reinforcement concrete i/c cutting                                                                               bending and laying in position making joints and fastening i/c                                                                                              cost os steel wire also i/c the removal rust from bars.                                                                                                         (S:I:NO:8a P:NO:17)

Using tor baar.	10.366cwtx5001.70pcwt			Rs:51848/=

Mild 		1.151 cwtx4820/20  --				Rs:5548/=

- Damp proof course with (cement sand and shingle concrete 1:2:4)

i/c 2 coats of asphaltic mixture.3” thick(S:I:NO:28P:NO:19)

=300.0 sftx4982/18p%sft					Rs:14946/=

08.Filling watering and ramming Earth in floor with                                                                                                               surptwo warth foundation lead up to one Chain                                                                                                       and lift up to5`(SI: NO:21 P:NO:4)

=1632.0cft x1512/50 P%0cft					Rs:2468/=

08.Filling watering and ramming earth under floor                                                                                               With new earth Excavated from out side lead up to                                                                                             one chain and lift up to 5’(S:I:NO:32P:NO:4)	 				       								=456.0 cft               3630/  p%0cft:		Rs 1655/=

09.Add Extra Lead for (1) mile.	=456.0cft x407/- P%cft			Rs:1856/=

10. Pacca Brick work in Ground Floor with cement sand mortor(1:6)

(S:I:NO:5(e)P:NO:21

=1609.0cft x12674/36 P%cft			Rs:203930/=

11. Providding and fixing G:I frame chowckets of size 7”x2”                                                                                  or 4 ½”x3” for door using 20 guage. GI sheet i/c welded hinges                                                                            and fixing at site with necessary hold fast .filling with cement                                                                                     and sand slurry of ratio 1:6 and repairing the job .The cost also                                                                                    i/c all carriages ,tools and plants used in making and fixing                                                                                             .(SI:NO:29 P:NO:93)

=67.0.0rft	atRs:228/90prft		Rs:15336//=

12. Prov/Fixing.G:I frame an chowckets of size 7”x2” or 4 ½”x3”               	                                                      fr window using 20 guage. GI sheet i/c welded hinges and fixing                                                                      at site with necessary hold fast .filling with cement and sand slurry                                                                     of ratio 1:6 and repairing the job .The cost also i/c all carriages ,tools                                                                        and plants used in making and fixing.(SI:NO:28 P:NO:93).												=102.0rftx240/50 Prft    			   Rs:24531/=

13. Supplying and Fixing Iron Girders at site of work.

=22.50 cwt x3850/= Pcwt				Rs:86625/=

14. Supplying and Fixing T:Iron at site of work.

=17.835 cwtx 3575/=Pcwt				Rs:63760/=

15. Errection of Rolled steel beam.

=40.335 cwt x186/34 pcwt				Rs:7516/=

16. Second class tiles roofing consisting of 4”earth and 1”

Mud plaster with gobree leeping over ½”thick cement plaster

1:6 with 34 lbs of hot bitumen coating sand blinded provided

Over one layer of 2”x6”x1’ tiles laid in 1:6 cement mortor i/c

1:2 cement pointing under neighth of tiles comolete i/c curring

Etc.(S:I:NO:2P:NO:33)

=1092.0sft x7607/25 p%sft				Rs:83071/=

17. Pacca Brick work in other than building i/c striking of

Joints up to 20’ height.(S:I:NO:7e P:NO:22)

=511.0cft x12346/65 P%cft				Rs:63091/=

18.  Cement Plaster (1:6) up to 12’ height1/2”thick.(S:I:NO:13b P:NO:52)

=5179.0sft x2206/60 P%sft				Rs:114280/=

19. Cement Plaster (1:4) up to 12’ height3/8” (S:I:NO:11c P:NO:52)

=5179.0sft x2197/52 p%sft				Rs:113810/=

20. Cement Pointing struct joints on wall  1:3 (S:NO:19b P:NO:53)

=2430.0sftx1213/58p%sft				Rs:29490/=

21.(a) Providing and laying 1” thick topping cement concrete (1:2:4)

Including surface finishing and deviding in to panels (d)2”thick

(SI:NO:16 P:NO:42)

=925.0x3275/50 p%sft				Rs:30298/=

22.  Making and fixing steel granted door complete with locking                                                                          arrangement angle iron frame 2”x2”x3/8” and ¾” squire bars 4”                                                                                    center to center .(S:I:NO: 23 p:no:92)

=48.0sftat Rs:726/72psft.				Rs:34883/=

23.  Supplying and fixing Iron steel GRILL ¾”position flate                                                                                      iron of approved design i/c painting3coats etc (weight not less                                                                                       than 3.7lbs of finished griLL(S:I:NO:26P:NO9)

55.0sft x:180/50 psft					Rs:9928/=

24.(a) First class deodar wood wrought ,joinery in doors                                                                                         and windows etc fixed in position i/c chowckets hold fasts                                                                                                ,hinges iron tower, bolts ,cleats handles and cards with hooks                                                                                      etc.(only shutters)1x3/4” thick.(S:I:NO:36bP:NO:53)

=139.0 sftx902/93 psft				Rs:125507/=

25. Reinfor cement concrete concrete spouts including fixing

In position 2’x1/2”x6” (S:I:NO:14 P:NO:18)

=5.0 NOx261/25 					Rs:1306/=

26. Making Notice Board made with cement.(S:I;NO 01 P:NO: 95)

=64.0sftx58/11 psft					Rs:3719/=

27. Colour washing 2 coats.(S:I:NO:25b P57)

3815.0sftx859.90 p%sft				Rs:32805/=

30. Preparing new surface Painting doors and windows any type

i/c edge.New surface(S:I:NO:4cP:NO:68)

=326.0sft x2116/41p%sft				Rs:6899/=

31. Painting new surface painting Guard bars ,gates of Iron                         			                                                                   brs gratting ,railing including standerd branches etc on

Similar open work.(S:I;NO:5d P:NO:70)

=735.0sft x1270/83 p%sft				Rs:9341/=

32.White washing two coats.(S:I:NO:25bP:NO:54)

=2196.0stx829/95 p%sft				Rs:18225/=

_________________

Total= 1562314/=

________% above/below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR

PART  B  WATER SUPPLY & SENATORY)

01 Providing Fixing type white glazed earthen ware wc pan with cost of flushing cictern with internal                                     fitting and flushing and internal pipe with bend flush pipe an making requisite number of holes in walls

Pipe an floor for pipe connection in cc 1:2:4 (b)wc of not less than 19” clear opening between flushing

Rimps an 3 Gallons flushing tank with 4” dia CI trap and CI thimble.

02 NOS	x6014/80		P:NOs		  Rs:12030/=

02. S:F 4”x4”x4” dia CI branches of the required degree with access doors rubber washer 1/8”thick and

Bolts and nuts and extra painting to mach the colour of the building.

02 NOS 	x704/20	PNOs	              Rs:1408/=

03.P:F 4” dia CI Plain bend of the required degree i/c extra painting to match the colour of the building.

(S:I:NO:10P-9)

02 NOs		x566/70	P:NOs                 Rs:1133/=

04.P:F 4” dia CI soil and vent pipe i/c cutting and fitting and extra painting to match the colour of the

Building.(S:I:NO1P:NO8)

2x6.0 =12rft	x333/29	PNOs      	   Rs:3999/=

____________

Total 	Rs: 18570.0

Part (B) ________% above/below/@ Par on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR