PART’A’ROAD WORK

| ## NAME OF SCHEME:- | Reconditioning of road from old N.H.Way – Wassanabad Murad Gopang road to village Ghulam Qasim Bugti mile 0/0 – 0/7 (1.40 Km) |
| --- | --- |

(SCHEDULE “B” / BOQ)

| QTY: | ITEM OF WORK | RATE |  | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1-Laying Brick on end edging i/cs: supply of 9”x4½”x3”Ist: Class Burnt bricks excavation for having edging with small size parallel to the road (Rate i/cs: lead upto side of work) |  |  |  |  |  |
| 9517 | Rft | @ Rs.2729/95 |  |  | P.% Rft: | Rs.259809/- |
|  | 2/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in r-=tequired thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) |  |  |  |  |  |
| 28552 | Cft | @ Rs.6539/47 | P.% Cft: |  |  | Rs.1867149/- |
|  | 3/-Providing Surface Dressing Ist: Coat on new or existing surface with 30lbs: bitumen & 4.00 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs |  |  |  |  |  |
| 57103 | Sft. | @ Rs.1588/39 |  | P.% Sft: |  | Rs.907018/- |
|  | 4/-Providing 1 ”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |
| 57103 | Sft. | @ Rs.4464/80 |  | P.% Sft: |  | Rs.2549535/- |
|  | 5/-Earth work for embankment from barrow pit i/cs: laying in 6”layers clod breaking remaining dressing complete lead upto 100’ & lift upto 5.0’ (In ordinary Soil (S.I No: 2 P/No: 1). |  |  |  |  |  |
| 130862 | Withextra lead | @Rs.6278/37 |  | P. %0Cft |  | Rs.821600/- |
|  | Cft. | Total Rs:6405111/- |  |  |  |  |
|  |  | Decreased Cost of Bitumen 24.72 ton @Rs:22606/- P.Ton |  |  |  | Rs(-)558820/- |
|  |  | G.Total |  |  |  | Rs: 5846291/- |

___________ % Above/Below (Amount to be added/deducted)														Rs.__________________

Deduct (-)Difference Cost of Bitumen			Rs.`			      .

Grand Total:					Rs.__________________

Total Amount in Word: _____________________________________________

__________________________________________________________________

| Contractor | Executive Engineer |
| --- | --- |
|  | Highway Division |
|  | Khairpur. |

PART’A’I M/STRUCTURE

| ## NAME OF SCHEME:- | Repair of Culvert along road from old N.H.Way – Wassanabad Murad Gopang road to village Ghulam Qasim Bugti mile 0/0 – 0/7 (1.40 Km) |
| --- | --- |

(Schedule “B” / BOQ)

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 4/-Cement concrete plain i/cs: placing compacting finishing & curing complete i/cs: screening & washing of stone aggregate without shuttering ratio (1:2:4) |  |  |  |  |
| 21 Cft: |  | @Rs:14429/25 |  | P. % Cft: | Rs.3030/ |
|  | 2/-Fabrication of mild steel reinforcement for cement concrete i/cs: cutting bending lying in position making joints & fastening i/cs: cost of mild steel & binding wire also rust from bars. |  |  |  |  |
| 3.86 Cwt |  | @Rs.4820/20 |  | P. Cwt: | Rs.18605/- |
|  | 6/-Reinforced cement concrete work i/cs: all labor & materials except the cost of steel reinforcement & its labor for bending & binding which will be paid separately. This rate also i/cs: all kinds of forms moulds lifting shuttering curing rending & finishing surface (I/cs: screening & washing of shingle) RCC work in roof slabs beams columns rafts & other structure member laid in situ or precuts in position complete in all respect ratio (1:2:4) 90 lbs: cement 2 cft: sand 4 cft: shingle 4/8”-1/4”gauge. |  |  |  |  |
| 157 Cft: |  | @ Rs.337/= |  | P. Cft: | Rs.52909/= |
|  | 7/-Cement plaster (1:3) upto 20’height ½”thick. |  |  |  |  |
| 198 Sft: |  | @ Rs.2344/59 |  | P. % Sft: | Rs.4642/= |
|  |  | Total Therefore will be 02 Nos: |  |  | Rs.79186/- Rs.158372/- |

___________ % Above/Below (Amount to be added/deducted)														Rs.__________________

Grand Total:					Rs.__________________

Total Amount in Word: _____________________________________________

__________________________________________________________________

| Contractor | Executive Engineer |
| --- | --- |
|  | Highway Division |
|  | Khairpur. |

| ## NAME OF SCHEME:- | Construction of Toe wall along road from old N.H.Way – Wassanabad Murad Gopang road to village Ghulam Qasim Bugti mile 0/0 – 0/7 (1.40 Km) |
| --- | --- |

(Schedule “B” / BOQ)

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Excavation in foundation of building bridges & other structure i/cs: degbelling dressing refilling around the structure lead upto 100’& lift upto 5.0’in ordinary soil. |  |  |  |  |
| 700 | Cft: | @Rs:3176/25 |  | P.%0 Cft | Rs:2223/- |
|  | 2/-Cement concrete brick or stone ballast of 1 ½”-2”guage ratio (1:4:8) |  |  |  |  |
| 300 | Cft: | @Rs:9416/28 |  | P. % Cft: | Rs.28249/- |
|  | 3/-Errection & removal of shuttering for R.C.C or plain cement concrete works of petrel wood of 2nd: Class (Vertical) |  |  |  |  |
| 1015 | Sft: | @Rs:3127/41 |  | P. % Cft: | Rs.3174/- |
|  | 4/-Cement concrete plain i/cs: placing compacting finishing & curing complete i/cs: screening & washing of stone aggregate without shuttering ratio (1:2:4) |  |  |  |  |
| 750 | Cft: | @Rs:14429/25 |  | P. % Cft: | Rs.108219/ |

|  |  |  | Total | Rs.141865/- |
| --- | --- | --- | --- | --- |

___________ % Above/Below (Amount to be added/deducted)														Rs.__________________

Deduct (-)Difference Cost of Bitumen			Rs.`			      .

Grand Total:					Rs.__________________

Total Amount in Word: _____________________________________________

__________________________________________________________________

| Contractor | Executive Engineer |
| --- | --- |
|  | Highway Division |
|  | Khairpur. |

PART’C’ SCHEME BOARD

| ## NAME OF SCHEME:- | Reconditioning of road from old N.H.Way – Wassanabad Murad Gopang road to village Ghulam Qasim Bugti mile 0/0 – 0/7 (1.40 Km) |
| --- | --- |

(Schedule “B” / BOQ)

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Providing of Scheme board of M.S Sheet of 3 ½”x3’x3’ of 1/8”guage and welding with 2”dia pipe around the sheet & for stand 2 Nos: i/cs: cutting painting writing & fixing at site in situ (1:3:6) plate from (2’x2’x2) etc: complete. |  |  |  |  |
| 01 No: |  | @Rs:16956/- |  | P.Each | Rs:16956/- |
|  |  | Total |  |  | Rs.16956/- |

___________ % Above/Below (Amount to be added/deducted)														Rs.__________________

Grand Total:					Rs.__________________

Total Amount in Word: _____________________________________________

__________________________________________________________________

| Contractor | Executive Engineer |
| --- | --- |
|  | Highway Division |
|  | Khairpur. |