SCHEDULE B

PART”A” ROAD WORK

NAME OF SCHEME: - CONST: OF ROAD AND PAVING BLOCK

OF SOBHODERO MAIN ROAD AT(SOBHODERO RANIPUR ROAD)

U/C SOBHODERO TALUKA SOBHODERO DISTT: KHAIRPUR

| QTY. | ITEM OF WORK | QTY | RATE. | UNIT | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

1/-Prepariong Sub –Base course by supplying & spreading stone

metal 1-1/2’’ gauge of approved quality from approved quarry in

required thickness to proper camber & grade i/cs: hand packing

mixed with shingle pit run grabel fines having P.I of not more than

then 6% in the ratio 2:3 including watering and compacting to achieve

98-100% density as per modified AASHO Specification (Rate i/cs:

all cost of materials T&P& carriage up to 3 chains  )

(S.I No:8 P/No:3)

43507    Cft:                   @Rs: 5515/14            P.%Cft:                  Rs: 2399482/=

3/-Preparing Base course i/cs: supplying & spreading stone

metal of approved quality properly grade to maximum size of

1-1/2 in required thickness to proper camber & grade i/cs:

supplying & spreading 15 cft: screening & Non plastic quarry

fines filling depression with stone metal after initial rolling

i/cs: watering & compacting the same so as to achieve 100%

density as per modified AASHO Specification.(Rate i/cs: all cost

of materials T&P& carriage up to site of work.

10876     Cft:                  @Rs:7702/02           P.% cft:                 Rs.837733/=

.4/Providing Surface dressing  Two Coat on new or existing surface

With30+18 = 48  lbs: of bitumen of 80/100 penetration and 4.0+1.5 = 5.50  cft:

Crusher bajri with i/cs: cleaning the road surface rolling etc complete. Rate i/c

all cost of materials T&P carriage up to site of work.

21753     Sft:                    @Rs:2886/61/=       P.% Sft:                 Rs.627924/=

5/-Providing I ½ “  thick (consolidate e) Premixed Carpet in proper/

camber & grade i/cs: supplying of 15 Cft: Crush bajri 5 cft:

Hill Sand (of approved quality and grade) with 93  lbs: bitumen

of 80/100 penetration i/cs: fixing in mechanical mixer in required

i/c mixing the material T&P& carriage up to site of work.

21753              Sft:            @Rs:6395/58                P.% Sft:      Rs. 1391230/=

______________

Total Rs:5256369/=

Decreased Cost of Bitumen

13.69 Tons   @Rs:29860/=   P.ton (-)    Rs:408865/=

_______________

G.Total  Rs:4847504/=

Note: - Subject to any changes/

Variation quantities/rates in

The technical Sanction

(CONTRACTOR)                                                              EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR

SCHEDULE B

PART”B”PAVING BLOCK

NAME OF SCHEME: - CONST: OF ROAD AND PAVING BLOCK

OF SOBHODERO MAIN ROAD AT(SOBHODERO RANIPUR ROAD)

U/C SOBHODERO TALUKA SOBHODERO DISTT: KHAIRPUR

| QTY. | ITEM OF WORK | QTY | RATE. | UNIT | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

1/-Prepariong Sub –Base course by supplying & spreading stone

metal 1-1/2’’ gauge of approved quality from approved quarry in

required thickness to proper camber & grade i/cs: hand packing

mixed with shingle pit run grabel fines having P.I of not more than

then 6% in the ratio 2:3 including watering and compacting to achieve

98-100% density as per modified AASHO Specification (Rate i/cs:

all cost of materials T&P& carriage up to 3 chains  )

(S.I No:8 P/No:3)

100x32x2+1.50+2.50           =  6400    Cft:

3

6400      Cft:             @Rs: 3380/=           P.% Cft:                     Rs.216320/=

02. Cement concrete brick in stone ballast 1-1/2 to 2” gauge Ratio  1:4:8.

(S.I.No.4, P-17).

100x32.0x0.37               =  1184    Cft:

_________

978   Cft:

1184       Cft:              @Rs: 9416/28          P.%cft:                       Rs.111488/=

03.Crushed fire bricks grated as sand (S.I.No. 51 P.No.04)

100x32.0x0.17                 =  544  Cft:

544        Cft:                @Rs: 2500/=            P.%cft:                        Rs.13600/=

04. Supplying and filling Sand under floor and plugging in wells.(S.I.No29,P-31)

544          Cft:                  @Rs: 151/25              P. %cft:                  Rs.822/=

P/2

| QTY | ITEM OF WORK | RATE. | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |

07. Providing & fixing cement paving blocks flooring having size of 197x97x

80 (mm) of city/ quddra /cobble shape with natural colours, having strength

B/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified

Manner/pattern and design etc complete.

3200               Sft:   @Rs: 223/91                 P.sft:                          Rs.716704/=

____________

Total Rs: 1058934/=

1058934  - 100    =  10589/34

There fore 862  x 10589/34  =  9128011/=

Note:- Subject to any changes/

Variation quantities/rates in

the technical Sanction

(CONTRACTOR)                                                      EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR

SCHEDULE B

PART’’C’’   DRAIN NALA

NAME OF SCHEME: - CONST: OF ROAD AND PAVING BLOCK

AND SURFACE DRAINS OF SOBHODERO CITY PORTION TALUKA

SOBHODERO ROAD  MILE 0/0-2/0 ( in portion)

| QTY | ITEM OF WORK | RATE. | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |

1. Excavation in foundation of building bridges and other structure including

Dig belling dressing refilling around structure with excavated earth watering and

Ramming lead up to one chain lift up to 5.o’ in ordinary soil.

1200     Cft:                       @Rs: 3176/25            P.%0 Cft:                                      Rs. 3811/=

2. Cement concrete brick in stone ballast 1-1/2 to 1:4:8. (Hill / sand to be used)

225      Cft:                         @Rs: 9416/28               P.%cft:                                      Rs. 21187/=

3. Pacca brick work in foundation in plinth i1:4.

2x100x0.75x1.50            =         225      Cft:

533     Cft:                           @Rs: 12501/41          P.%cft:                                        Rs. 66632/=

4.Errection & removal or centering for RCC or plain CC work of partal wood

(S.NO:  18 b P (No:21)

100      Cft;         Vertical     @Rs:  3127/41          P.%Sft:                                       Rs: 3127/=

184     Cft:          Horizantal @Rs: 3588/48            P.%Sft:                                        Rs. 6603/=

5.Cement Concrete plain i/c plashing compacting finishing and curing complete (i/c

screening and washing of stone aggregate without shuttering Ratio (1:2:4).

183      Cft:                           @Rs: 14429/25           P.%cft:                                       Rs.26406/=

6. Cement plaster (1:3) up to 20’hieght ½ “ thick.

348     Sft:                            @Rs: 2344/59            P.% Sft:                                     Rs: 8159/=

7. Fabrication of mild steel reinforcement for cement concrete i/cs:

Cutting bending laying in position making joints & fastening i/cs:

Cost of mild steel & binding wire also from bars

5.42   Cwt:                          @Rs: 4820/20           P.Cwt                                         Rs: 26125/=

8. R.C.C  Work including all labor and material   except the Cost of steel reinforcement

and its labor for bending and binding which will paid Separately. This rate also includes

all kinds of forms mould lifting shuttering curing rendering and finishing the exposed

surface (including screening and washing of shingle)(a)RCC works in roof slab, beams

columns rafts lintels and other structural member laid  in situ recast laid in position

complete in all respect ratio (1:2:4)  90 lbs: cement 2cft:sand 4cft:shingle 4/8’’guage.

135     Cft:                         @Rs: 337/=                P.% cft:                                 Rs: 45495/=

9. Small Iron works such as gusted plate knees bends stirrups rings etc: including

Cutting ceiling reinforcing handing asserting and fixing bat excluding refection

in position

3.87  Cwt:                          @Rs: 6420/61           P.Cwt                               Rs: 24848/=

10. Errection and fitting in position or iron trusses slinging of water tents etc:

3.87    Cwt:                          @Rs: 271/04              P.Cwt                               Rs: 1049/=

11.Provising & fixing pre cost edge blocks 3750 Psi industrial made size 6” thick

X12” inches long  x12” high including the cost of cartage excavation farm work

For huncting 1450 Psi Item concrete 2250 Psi concrete for hunting 1:4 cement sand

mortor.

103   Rft:                             @Rs: 297/01             P.Sft:                                 Rs: 30592/=

12. Cement Pointing sticking joints on walls (1:2) (S.I NO: 19 P No: 59 )

137    Rft”                            @RsL: 1287/44        P.%Sft:                                Rs: 1763/=

13. Pointiing New Surface preparing economically paint on walls (coats)

137    Sft:                             @Rs: 2116/41           P.%Sft                               Rs: 2899/=

14. Preparing Hard Murmm proudly from Base complete of watering leveling and

Compacting etc complete.

42  Cft:                                @Rs: 1800/=               P.%Cft:                             Rs: 756/=

15. Providing and fixing Cement paving blocks flooring size of 1917x97060 city /

Quadra / Cables shape with Natural Colour  having Strength b/w 5000 Psi to

8500 Psi  filling the joints with hil sand and having in specified manner / Paver

And design etc: complete.

254    Sft;                            @Rs: 199/77                P.%Sft:                          Rs: 50742/=

______________

Total Rs: 320194/=

320194    -      100        =  3201/94

There fore      660  x    3201/94       =     2113280/=

Note:- Subject to any changes/

Variation quantities/rates in

the technical Sanction

(CONTRACTOR)                                                              EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR