PART’A’ROAD WORK

| ## NAME OF SCHEME:- | Construction of Internal road for village Haji Jan Muhammad Tumrani Taluka Kotdiji mile 0/0 – 0/6. |
| --- | --- |

(SCHEDULE “B” / BOQ)

| QTY: |  | ITEM OF WORK | RATE |  | UNIT |  | AMOUNT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1/-Earth work for road embankment By bulldozers i/cs: plugging mixing clod breaking dressing & compacting with optimum moisture content lead upto 100’ & lift upto 5ft: in all types of soil. (S.I No: 3 P/No: 1). A) COMPACTING UPTO 85%MODIFIED AASHO DENSITY. |  |  |  |  |  |  |  |
| 192456 |  | Cft: withextra lead | @Rs.7723/95 |  | P. %0Cft: |  | Rs.1486520/- |  |  |
|  |  | #### B) COMPACTING UPTO 95-100%MODIFIED DENSITY |  |  |  |  |  |  |  |
| 57103 |  | Cft: withextra lead | @Rs.10253/72 |  | P. %0 Cft: |  | Rs:585518/- |  |  |
|  |  | 2/-Preparing Sub-Base course by supplying & spreading stone metal 1 ½”-2”gauge of approved quality from approved quarry in required thickness to proper camber & grade i/cs: hand packing filling voids with 10 cft: screening & Non plastic quarry fines of approved source watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 8© P/No: 3). |  |  |  |  |  |  |  |
| 24473 |  | Cft. | @ Rs.6074/77 |  | P.% Cft: |  | Rs.1486678/- |  |  |
|  |  | 3/-Laying Brick on end edging i/cs: supply of 9”x4½”x3”Ist: Class Burnt bricks excavation for having edging with small size parallel to the road (Rate i/cs: lead upto side of work) |  |  |  |  |  |  |  |
| 8158 |  | Rft | @ Rs.2750/79 |  |  | P.% Rft: | Rs.224409/- |  |  |
|  |  | 4/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in r-=tequired thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) |  |  |  |  |  |  |  |
| 12236 |  | Cft | @ Rs.7027/38 | P.% Cft: |  |  | Rs.859870/- |  |  |
|  |  | 5/-Providing Surface Dressing Ist: Coat on new or existing surface with 30lbs: bitumen & 4.00 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs |  |  |  |  |  |  |  |
| 48946 |  | Sft. | @ Rs.1596/99 |  | P.% Sft: |  | Rs.781663/- |  |  |
|  |  | 6/-Providing 1 ”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |  |  |
| 48946 |  | Sft. | @ Rs.4557/24 |  | P.% Sft: |  | Rs.2230587/- |  |  |
|  |  | 7/-Earth work for embankment from barrow pit i/cs: laying in 6”layers clod breaking remaining dressing complete lead upto 100’ & lift upto 5.0’ (In ordinary Soil (S.I No: 2 P/No: 1). |  |  |  |  |  |  |  |
| 27799 |  | Cft. | @Rs.6278/37 |  | P. %0Cft |  | Rs.174531/- |  |  |
|  |  |  | Total Rs:7829776/- |  |  |  |  |  |  |
|  |  |  | Decreased Cost of Bitumen 21.19ton @Rs:22606/- P.Ton |  |  |  | Rs(-)479021/- |  |  |
|  |  |  | G.Total |  |  |  | Rs.7350755/- |  |  |
|  | ___________ % Above/Below (Amount to be added/deducted) Rs.____________________________ Deduct (-)Difference Cost of Bitumen Rs.` ____ . Grand Total: Rs._____________________________ Total Amount in Word: _____________________________________________________________________________________________ ______________________________________________________________________________________ Contractor; Executive Engineer. Highway Division. Khairpur.. |  |
|  | ## ART’A’I M/STRUCTURE ## NAME OF SCHEME:-; Construction of 3.0’ Span Culvert along road from village Haji Jan Muhammad Tumrani Taluka Kotdiji mile 0/0 – 0/6. (Schedule “B” / BOQ) QTY:; ITEM OF WORK; RATE; UNIT; AMOUNT. 1/-Excavation in foundation of building bridges & other structure i/cs: degbelling dressing refilling around the structure lead upto 100’& lift upto 5.0’in ordinary soil.. 1250Cft:; @Rs:3716/25; P.% 0 Cft:; Rs:3970/=. 2/-Cement concrete brick or stone ballast of 1 ½”-2”guage ratio (1:4:8). 245 Cft:; @Rs:9416/28; P. % Cft:; Rs.23070/-. 3/-Pacca Brick work in foundation & plinth in cement send mortar ratio (1:4). 357 Cft:; @Rs:12501/41; P. % Cft:; Rs.44630/=. 4/-Cement concrete plain i/cs: placing compacting finishing & curing complete i/cs: screening & washing of stone aggregate without shuttering ratio (1:2:4). 21 Cft:; @Rs:14429/25; P. % Cft:; Rs.3030/. 5/-Fabrication of mild steel reinforcement for cement concrete i/cs: cutting bending lying in position making joints & fastening i/cs: cost of mild steel & binding wire also rust from bars.. 6.82 Cwt; @Rs.4820/20; P. Cwt:; Rs.32922/=. 6/-Rienforced cement concrete work i/cs: all labor & materials except the cost of steel reinforcement & its labor for bending & binding which will be paid separately. This rate also i/cs: all kinds of forms moulds lifting shuttering curing rending & finishing surface (I/cs: screening & washing of shingle) RCC work in roof slabs beams columns rafts & other structure member laid in situ or precuts in position complete in all respect ratio (1:2:4) 90 lbs: cement 2 cft: sand 4 cft: shingle 4/8”-1/4”gauge.. 157 Cft:; @ Rs.337/=; P. Cft:; Rs.52909/=. 7/-Cement plaster (1:3) upto 20’height ½”thick.. 198 Sft:; @ Rs.2344/59; P. % Sft:; Rs.4642/=. Total:-; Rs.165173/-. |  |
|  | ___________ % Above/Below (Amount to be added/deducted) Rs.____________________________ Grand Total: Rs._____________________________ Total Amount in Word: _____________________________________________________________________________________________ ______________________________________________________________________________________ Contractor; Executive Engineer. Highway Division. Khairpur.. |  |
|  | ## NAME OF SCHEME:-; Construction of Drain Cross Culvert along village Haji Jan Muhammad Tumrani Taluka Kotdiji mile 0/0 – 0/6. (Schedule “B” / BOQ) QTY:; ITEM OF WORK; RATE; UNIT; AMOUNT. 1/-Excavation in foundation of building bridges & other structure i/cs: degbelling dressing refilling around the structure lead upto 100’& lift upto 5.0’in ordinary soil.. 162 Cft:; @Rs:3716/25; P.% 0 Cft:; Rs:514/-. 2/-Cement concrete brick or stone ballast of 1 ½”-2”guage ratio (1:4:8). 24 Cft:; @Rs:9416/28; P. % Cft:; Rs.2260/-. 3/-Pacca Brick work in foundation & plinth in cement send mortar ratio (1:4). 64 Cft:; @Rs:12501/41; P. % Cft:; Rs.8000/-. 4/-Cement concrete plain i/cs: placing compacting finishing & curing complete i/cs: screening & washing of stone aggregate without shuttering ratio (1:2:4). 10 Cft:; @Rs:14429/25; P. % Cft:; Rs.1443/-. 5/-Fabrication of mild steel reinforcement for cement concrete i/cs: cutting bending lying in position making joints & fastening i/cs: cost of mild steel & binding wire also rust from bars.. 0.85 Cwt; @Rs.4820/20; P. Cwt:; Rs.4097/-. 6/-Rienforced cement concrete work i/cs: all labor & materials except the cost of steel reinforcement & its labor for bending & binding which will be paid separately. This rate also i/cs: all kinds of forms moulds lifting shuttering curing rending & finishing surface (I/cs: screening & washing of shingle) RCC work in roof slabs beams columns rafts & other structure member laid in situ or precuts in position complete in all respect ratio (1:2:4) 90 lbs: cement 2 cft: sand 4 cft: shingle 4/8”-1/4”gauge.. 34 Cft:; @ Rs.337/=; P. Cft:; Rs.11458/-. 7/-Cement plaster (1:3) upto 20’height ½”thick.. 60 Sft:; @ Rs.2344/59; P. % Sft:; Rs.1407/-. Total Therefore will be 05 Nos.; Rs.29179/- Rs.145895/-. |  |
|  | ___________ % Above/Below (Amount to be added/deducted) Rs.____________________________ Grand Total: Rs._____________________________ Total Amount in Word: _____________________________________________________________________________________________ ______________________________________________________________________________________ Contractor; Executive Engineer. Highway Division. Khairpur.. |  |

Part B’ II Paver Block

| ## NAME OF SCHEME:- | Providing Paver Block for Internal roads for Haji Jan Muhammad Tumrani Taluka Kotdiji mile 0/0 – 0/6 |
| --- | --- |

(Schedule B / BOQ)

| QTY: | ITEM OF WORK | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |
|  | 1/-Providing Sand Cushion including supplying and spreading pit /canal sand of approved quality from approved source of supply to site of work including watering and rolling etc: complete. (Rate i/cs: all costs of materials T&P labour and carriage of 3-0 Miles (Average) (R.A.A) |  |  |  |
| 500 | Cft: | @Rs.1479/41 | P. %Cft: | Rs.7397/- |
|  | 2/-Cement concrete brick or stone ballast of 1 ½” – 2”guage of Ratio (1:4:8) |  |  |  |
| 669 cft |  | @Rs.9416/28 | P. % Cft: | Rs:62995/- |
|  | 3/-Crushed fire bricks grated as sand (S.I No:51 P/No:04) |  |  |  |
| 170 Cft |  | @ Rs.2500/- | P.% Cft: | Rs.4250/- |
|  | 4/-Supplying & filling sand under floor and plugging in walls (S.I No:29 P/No:31) |  |  |  |
| 170 Cft: |  | @Rs:151/25 | P.% Cft | Rs:257/- |
|  | 5/- Errection & removal of shuttering for R.C.C or plain cement concrete works of petrel wood of 2nd: Class (Vertical) |  |  |  |
| 500 Cft: |  | @Rs:3127/41 | P.% Cft: | Rs:15637/- |
|  | 6/- Cement concrete plain i/cs: placing compacting finishing & curing complete i/cs: screening & washing of stone aggregate without shuttering ratio (1:2:4) |  |  |  |
| 500 Cft: |  | @Rs:14429/25 | P.% Cft: | Rs:72146/- |
|  | 7/-Providing & fixing cement paving blocks flooring having size of 8”x4”x3” (mm) of city / quddra / with pigmented having strength b/w 5000 psi to 8500 psi over bed of 3”thick C.C (1:3:6) i/cs: filling the joints with hillsand and lying in specified manner / paver and design etc: complete (Eviscerate pavers) |  |  |  |
| 1030 Sft |  | @ Rs.199/77 | P. Sft: | Rs.205763/- |
|  |  |  | Total Rs:368445/- |  |
|  |  | Therefore will be 200 Rft: |  | Rs:736890/- |

___________ % Above/Below (Amount to be added/deducted)

Rs.____________________________

Grand Total:	                      			            Rs._____________________________

Total Amount in Word: _____________________________________________________________________________________________

______________________________________________________________________________________

| Contractor | Executive Engineer |
| --- | --- |
|  | Highway Division |
|  | Khairpur. |

PART’C’ SCHEME BOARD

| ## NAME OF SCHEME:- | Construction of Internal road for village Haji Jan Muhammad Tumrani Taluka Kotdiji mile 0/0 – 0/6 |
| --- | --- |

(Schedule “B” / BOQ

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Providing of Scheme board of M.S Sheet of 3 ½”x3’x3’ of 1/8”guage and welding with 2”dia pipe around the sheet & for stand 2 Nos: i/cs: cutting painting writing & fixing at site in situ (1:3:6) plate from (2’x2’x2) etc: complete. |  |  |  |  |
| 01 No: |  | @Rs:16956/- |  | P.Each | Rs:16956/- |
|  |  | Total |  |  | Rs.16956/- |

___________ % Above/Below (Amount to be added/deducted)

Rs.____________________________

Grand Total:	                      			            Rs._____________________________

Total Amount in Word: _____________________________________________________________________________________________

______________________________________________________________________________________

| Contractor | Executive Engineer |
| --- | --- |
|  | Highway Division |
|  | Khairpur. |