PART’A’ROAD WORK

| ## NAME OF SCHEME:- | Rehabilitation of road from Hindiary – Rasoolpur road to village Muhammad Akram Shar mile 0/0 – 0/5 (1.0 Km) |
| --- | --- |

(SCHEDULE “B” / BOQ)

| QTY: | ITEM OF WORK | RATE |  | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1/-Laying Brick on end edging i/cs: supply of 9”x4½”x3”Ist: Class Burnt bricks excavation for having edging with small size parallel to the road (Rate i/cs: lead upto side of work) |  |  |  |  |  |
| 6200 | Rft | @ Rs.2729/15 |  |  | P.% Rft: | Rs.169207/- |
|  | 2/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in r-=tequired thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) |  |  |  |  |  |
| 19900 | Cft | @ Rs.7846/59 | P.% Cft: |  |  | Rs.1561471/- |
|  | 3/-Providing Surface Dressing Ist: Coat on new or existing surface with 30lbs: bitumen & 4.00 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs |  |  |  |  |  |
| 37100 | Sft. | @ Rs.1617/31 |  | P.% Sft: |  | Rs.600022/- |
|  | 4/-Providing 1 ”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |
| 37100 | Sft. | @ Rs.4538/99 |  | P.% Sft: |  | Rs.1683965/- |
|  | 5/-Earth work for embankment from barrow pit i/cs: laying in 6”layers clod breaking remaining dressing complete lead upto 100’ & lift upto 5.0’ (In ordinary Soil (S.I No: 2 P/No: 1). |  |  |  |  |  |
| 325400 | Cft. | @Rs.6278/37 |  | P. %0Cft |  | Rs.2042982/- |
|  |  | Total Rs:6057647/- |  |  |  |  |
|  |  | Decreased Cost of Bitumen 16.06 ton @Rs:22606/- P.Ton |  |  |  | Rs(-)363052/- |
|  |  | G.Total |  |  |  | Rs: 5694595/- |

| NOTE: Subject to any Changes/ Variation in Rates/ Quantity as per T.Sanction. |  |
| --- | --- |
| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |

PART’A’I M/STRUCTURE

| ## NAME OF SCHEME:- | Repair of culvert along road from Hindiary – Rasoolpur road to village Muhammad Akram Shar mile 0/0 – 0/5 (1.0 Km) |
| --- | --- |

(Schedule “B” / BOQ)

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Dismantling of cement concrete reinforcement separating reinforcement from cement concrete from cleaning. |  |  |  |  |
| 106Cft: |  | @Rs:5445/- |  | P. % Cft: | Rs.5772/- |
|  | 2/-Fabrication of mild steel reinforcement for cement concrete i/cs: cutting bending lying in position making joints & fastening i/cs: cost of mild steel & binding wire also rust from bars. |  |  |  |  |
| 5.96 Cwt |  | @Rs.4820/20 |  | P. Cwt: | Rs.28728/- |
|  | 3/-Rienforced cement concrete work i/cs: all labor & materials except the cost of steel reinforcement & its labor for bending & binding which will be paid separately. This rate also i/cs: all kinds of forms moulds lifting shuttering curing rending & finishing surface (I/cs: screening & washing of shingle) RCC work in roof slabs beams columns rafts & other structure member laid in situ or precuts in position complete in all respect ratio (1:2:4) 90 lbs: cement 2 cft: sand 4 cft: shingle 4/8”-1/4”gauge. |  |  |  |  |
| 157Cft: |  | @ Rs.337/= |  | P. Cft: | Rs.52909/- |
|  |  | Total . Therefore will be 02 Nos. |  |  | Rs.87409/- Rs.174818/- |

| NOTE: Subject to any Changes/ Variation in Rates/ Quantity as per T.Sanction. |  |
| --- | --- |
| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |

| ## NAME OF SCHEME:- | Construction of Retaining Wall along Hindiary Rasoolpur to village Muhammad Akram Shar mile 0/0 – 1/2 |
| --- | --- |

(Schedule B / BOQ)

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Excavation in foundation of building bridges & other structure i/cs: degbelling dressing refilling around the structure lead upto 100’& lift upto 5.0’in ordinary soil. |  |  |  |  |
| 300 | Cft: | @Rs:3176/25 |  | P.%0 Cft | Rs:953/- |
|  | 2/-Cement concrete brick or stone ballast of 1 ½”-2”guage ratio (1:4:8) |  |  |  |  |
| 100 | Cft: | @Rs:9416/28 |  | P. % Cft: | Rs.9416/- |
|  | 3/-Pacca Brick work in foundation & Plitnh in cement send mortor ratio (1:4) |  |  |  |  |
| 346 | Sft: | @Rs:12501/41 |  | P. % Cft: | Rs.43255/- |
|  | 4/-Cement concrete plain i/cs: placing compacting finishing & curing complete i/cs: screening & washing of stone aggregate without shuttering ratio (1:2:4) |  |  |  |  |
| 28 | Cft: | @Rs:14429/25 |  | P. % Cft: | Rs.4040/- |
|  | 5/-Cement plaster (1:3) upto 20’height ½”thick. |  |  |  |  |
| 175 | Sft | @rs.2344/59 |  | P.%Sft. | Rs.4103/- |

|  |  | Total Therefore will be 630 | Rs.61767/- Rs.389132/- |
| --- | --- | --- | --- |

| NOTE: Subject to any Changes/ Variation in Rates/ Quantity as per T.Sanction. |  |
| --- | --- |
| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |

PART’C’ SCHEME BOARD

| ## NAME OF SCHEME:- | Rehabilitation of road from Hindiary – Rasoolpur road to village Muhammad Akram Shar mile 0/0 – 0/5 (1.0 Km) |
| --- | --- |

(Schedule “B” / BOQ

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Providing of Scheme board of M.S Sheet of 3 ½”x3’x3’ of 1/8”guage and welding with 2”dia pipe around the sheet & for stand 2 Nos: i/cs: cutting painting writing & fixing at site in situ (1:3:6) plate from (2’x2’x2) etc: complete. |  |  |  |  |
| 01 No: |  | @Rs:16956/- |  | P.Each | Rs:16956/- |
|  |  | Total |  |  | Rs.16956/- |

| NOTE: Subject to any Changes/ Variation in Rates/ Quantity as per T.Sanction. |  |
| --- | --- |
| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |