Scheme No: 04/07

SCHEDULE “B”

Name of Work:	Providing and Fixing Paver Block & Rehabilitation of Surface Drains from Umar Farooq Masjid to Noor-Ul-Islam Masjid

| S.No | Description of Item | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I	PAVER BLOCK

- Preparing base course including supplying and spreading stone metal of approved quality properly gauge to maximum size 1 ½” in required thickness to proper camber and gauge i/c supplying and spreading 15 Cft: screening and on plastic quarry fines, filling depressions with stone metal after initial rolling i/c watering and compacting the same so as to achiever 100% density as per modified ASSHO specification. Rate includes providing and using templates, camber pates, screens forms as directed. Rate all cost of material T & P and carriage up site of work etc.

Qty: 7280.81		@Rs: 8569/43	 	P%Cft 				Rs: 623924/-

- Erection and removal of centering for R.C.C or plain cement concert works of partal wood.

Qty: 325			@Rs: 3127/41	 	P%Sft	 			Rs: 10164/-

- C.C Plain 1:2:4

Qty: 243.75		@Rs: 14429/25 		P% Cft 				Rs: 35171/-

- Providing and fixing cement paving blocks flooring having size of 197x97x80 (mm) of city/quddra/cobble shape with natural colours, having strength between 5000 psi ti 8500 psi i/c. filling the joint with hill sand and laying in specified manner/pattern and design etc complete.

Qty: 18850		@Rs: 223.97	 	P-Sft 				Rs:    4221834/-

Total	Rs:    4891093/-

PART-II	SURFACE DRAINS

- Dismantling of C.C Plain 1:2:4 (CSI No: 19 (c) P/10)

Qty: 47.0 cft 			@Rs: 3327/50			P%CFT		Rs: 1564/-

- Dismantling of pacca brick in line or cement mortar.

(CSI No: 13 P/10)

Qty: 37.0 cft 			@Rs: 1285/63			P%CFT		Rs: 482/-

- Exta for every 50ft additional lead or part thereof in O. Soil 2000 ft.

Qty: 84.0 cft 			@Rs: 4031/20			P%0CFT	Rs: 339/-

- Excavation in foundation of building bridges and other structure i/c

Dag belling dressing refilling around the structure with excavated

Earth watering and ramming etc complete lead upto one chain lift

Upto5’ft. (GSI No: 18 P/4).

Qty: 1097.0 cft 		@Rs: 3176/25			P%0CFT	Rs: 3485/-

5.      Cement concrete plain including placing complete and curing

Complete i/c screening and watering of stone aggregate without

Shuttering and washing of stone 1:4:8 (GSI No: 5 P/17).

Qty: 273.00 cft	     	  	@Rs: 11288/75          		P%CFT           	Rs: 30819/-

- Construction of standard open drain Conettee block of cement

Concrete 1:2:4

T-I

Qty: 270.0 Rft			@Rs: 94/-			P-RFT		Rs: 25380/-

T-II

Qty: 110.0 Rft			@Rs: 174/-			P-RFT		Rs: 19140/-

6.	Pacca brick work in foundation up to plinth in cement sand

Motor.1:6.(CSI NO: 05 P/44).

Qty: 734.0 cft    		@Rs: 11948/36        		P%CFT           	Rs: 87701/-

7.	Cement plaster 1:4 up to 20’height ½ ‘thick. (GSI No: 11 P/58).

Qty: 1060.0 sft                        	@Rs: 2283/93          		P%SFT     	Rs: 24210/-

8.    	 R.CC work in roof slabs beams columns rafts lintels and other

Structural member laid in position situ or precost laid in position

Complete in all respects Ratio: 1:2:4 (CSI NO 6 (a) P/No 17)

Qty: 65.0 cft			@Rs: 337/-			P-CFT		Rs: 21905/-

9.	Fabrication of mild steel reinforcement for C.C i/c bending

Laying in position making joints and fastending i/c cost of

binding wires also include removal of rust from bars.

(CSI NO: 7 (a) P/No: 18).

Qty: 3.20 cwt                 		@Rs: 4820/20            		P-CWT             	Rs:   14461/-

Total	Rs: 229486/-

#### QUOTED BID SUMMARY

Part – I 	Paver Block	% percent above/below______________	Total Amount	________________

Part – II Surface Drain	% percent above/below______________	Total Amount	________________

Total “I+II”	________________

##### CERTIFICATE

1.	Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications shown in the estimate.

###### CONTRACTOR 		ASSISTANT EXECUTIVE ENGINEER

TOWN COMMITTEE KANDIARO