Scheme No: 02/07

SCHEDULE “B”

Name of Work:	Recondition of Road from Lakha Petrol Pump to Chandan Escape

| S.No | Description of Item | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

- Preparing base course including supplying and spreading stone metal of approved quality properly gauge to maximum size 1 ½” in required thickness to proper camber and gauge i/c supplying and spreading 15 Cft: screening and on plastic quarry fines, filling depressions with stone metal after initial rolling i/c watering and compacting the same so as to achiever 100% density as per modified ASSHO specification. Rate includes providing and using templates, camber pates, screens forms as directed. Rate all cost of material T & P and carriage up site of work etc.

Qty: 16900		@Rs: 8569/43 		P- Cft 				Rs: 1448234/-

- Laying bricks on end edging including supplying 9” x 4 – ½” x 3” 1st class bricks, excavation for having edging with small size parallel to the road. Rate including lead up to 3 chains.

Qty: 2884			@Rs: 2729/96		P-Rft	 			Rs: 78732/-

- Providing surface dressing 2 coats on new or existing surface with 30 + 25 = 55 lbs bitumen and 6.75 Cft crush bajri of required size including cleaning the road surface rolling etc complete. Rate includes all costs of materials T&P and carriage up to 3 chains.

Qty: 50449		@Rs: 2932/85 		P% Sft 				Rs: 1479593/-

- Laying to proper line & grade plant mixed Asphalt concrete pavers finisher (Hydraulic / electric control) prepared to specified formula according to job mix formula approved by engineer i/c rolling & finished proper line grade level & camber etc complete. Rate i/c all cost of material with bitumen 80/100 penetration & all cost of materials roller and carriage of materials.

Qty: 50449			@Rs: 9056/69	 	P% Sft 				Rs:  4569009/-

- Line making with thermo plastic 6” width paint (NS.I Sanctioned by C.E.H/W Hyd).

Qty: 4740			@ 41/24		P-Rft				Rs:    195478/-

Total	Rs: 7771046/-

#### QUOTED BID SUMMARY

Part – I 	Road Work 	% percent above/below______________	Total Amount	________________

##### CERTIFICATE

1.	Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications shown in the estimate.

###### CONTRACTOR 		ASSISTANT EXECUTIVE ENGINEER

TOWN COMMITTEE KANDIARO