GOVERNMENT OF SINDH

W/No:

DETAILED WORKING ESTIMATE CONSTRUCTION OF CULVERT

WARD NO.01

TOWN COMMITTEE DHORONARO

ESTIMATED CONST RS.   	720000/=

PREPARED BY:

Town Committee Dhoronaro

TOWN COMMITTEE DHORONARO

FUND HEAD:		TOWN COMMITTEE DHORONARO

MAJOR HEAD:		OWN FUND

MINOR HEAD:

DEPARTMENT HEAD: 	DETAILED WORKING ESTEMATE CONSTRUCTION OF CULVERT WARD NO.01

Estimated  framed    in   the   office    of   the  Municipal Engineer Town Committee Dhoronaro for the probable expenditure that will be incurred on Construction of Culvert Ward No.01

ESTIMATED COST RS.	720000/=

GENERAL DESCRIPION

- The Work carried on availability of funds.

- The rates as  provided in this estimate as per approval

schedule of rates.

Sub- Engineer				                   Municipal Engineer

Town Committee Dhoronaro			          Municipal Committee Umerkot

OFFICE OF THE TOWN COMMITTEE DHORONARO

DETAILED WORKING ESTIMATE CONTRUCTION CULVERT WARD NO.01

A B S T R A CT

| Part | Amount |
| --- | --- |
| CULVERTS | Rs.144000/= |
| 1X5 Nos | Rs.720000/= |

Sub- Engineer				                   Municipal Engineer

Town Committee Dhoronaro			          Municipal Committee Umerkot

Town Officer				                          Chairman

Town Committee Dhoronaro			           Town Committee Dhoronaro

DETAILED WORKING ESTIMATE CONSTRUCTION CULVERTS WARD NO.01

| 1. | Excavation in foundation of Building, Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth, watering and ramming lead upto 5.00 ft. (in ordinary soil) (G.S.I No.18(a) P.No.4 |
| --- | --- |

Trench		1X1X16.0X7.0X3.0			=	336.0

Abutment Wall	1X2X16.0X2.88X2.0		=	184.0

Wing Wall	1x4x4.0x2.62x1.75			=	73.0

593.0

593.0	Cft		@ Rs.3176/25				P%0 Cft 	Rs.1884/=

| 2. | Cement Concrete brick or stone ballast 1 1/2 to 2" gauge. Ratio 1:4:8. (G.S.I No.18(a) P.No.4) |
| --- | --- |

Abutment Wall	1X2X16.0X2.88X1.0		=	92.0

Bed		1X1X16.0X2.25X0.50		=	18.0

Wing Wall	1X4X4.0X2.62X0.75		=	31.0

141.0

141.0	Cft		@ Rs.9416/28			P% Cft 		Rs.13277/=

| 3. | Pucca brick work in foundation and plinth in cement sand mortar 1:4 (G.S.I No.4(i) P.No.22) |
| --- | --- |

Abutment Wall	1X2X16.0X1.88X1.0		=	60.0

Wing Wall	1X4X4.0X1.88X1.0			=	30.0

Abutment Wall	1X2X16.0X1.50X2.0		=	96.0

Wing Wall	1X4X4.0X1.50X2.0			=	48.0

Abutment Wall	1X2X16.0X1.12X1.0		=	36.0

Wing Wall	1X4X4.0X1.12X1.0			=	18.0

Wing Wall	1X4X4.0X0.75X1.0			=	14.0

302.0

302.0	Cft		@ Rs.12501/41			P% Cft 		Rs.37754/=

| 4. | Cement concrete plain including placing,compacting, finishing and curing, complete (including screening and washing of stone aggregate without shuttering (Ratio 1:2:4). (G.S.I No.5(f) P.No.17 |
| --- | --- |

Bed Plate	1X2X16.0X1.13X0.50		=	18.0

Side of Slab	1X2X16.0X0.62X0.67		=	13.0

Bed Floor	1X1X16.0X2.25X0.25		=	9.0

W/W(Toping)	1X4X4.0X0.75X0.25		=	3.0														43.0

43.0	Cft		@ Rs.14429/25			P% Cft 		Rs.6205/=

| 5. | Febrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars.) (G.S.I No.7(a) P.No.18) |
| --- | --- |

SlabM bar 3/4”	1X32X5.0X1.043			=	166.88

bars ½” dia	1X1X7X16X0.667			=	74.70

M bars ½”	1X32X5.0X0.667			=	106.72

Dist Bars1/2” 	1X1X9X16.0X0.667			=	96.04

P/PWall 1/8” 	1X2X6X5.0X0.375			=	22.50

P/PWall ¼”	1X2X4X5.0X0.167			=	6.68

473.52 lbs

Total in Cwt	=	4.22

4. 22	Cft		@ Rs.5001/70			P% Cwt 	Rs.21107/=

| 6. | Reinforced cemnt concrete work including all labour and materials except the cost of steel reinforcement and its labour for bending and binding which wil be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle). R.C. work in roof slab, brams, columns rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects. Ratio (1:2:4) 90 lbs cement 2 Cft. sand 4 Cft. Shingle 1/8" to 1/4" gauge. (G.S.I No.6(i) P.No.18) |
| --- | --- |

Slab		1X1X16.0X4.25X0.75		=	51.0

Parapet Wall	1X2X4.25X1.0+0.50X1.50		=	9.56

2				60.56

Say	61.0

61.0	Cft		@ Rs.337/=				P% Cft 		Rs.20557/=

| 7. | Erection and removal of centering for R.C.C. or plain cement concrete works of Partal wood (2nd class) (a) Vertical (G.S.I No.18(b) P.No.19 & 20) |
| --- | --- |

Parapet Wall	1X2X5.25X1.50			=	15.75

Parapet Side	1X2X0.50+1.13X1.50		=	03

2					18.75

Say	19.0

19.0	Cft		@ Rs.3127/41			P% Sft 		Rs.1181/=

| 8. | Cement Plaster 1:3 upto 20.0' height. 1/2" thick. (G.S.I No.10(b) P.No.53) |
| --- | --- |

Abutment Wall	1X2X16.0X3.0			=	96.0

Wing Wall	1X4X4.0X4.16			=	67.0

W.Wall (Side)	1X4X0.75X4.16			=	12.0

175.0

175.0	Cft		@ Rs.2344/59			P% Sft 		Rs.4103/=

Total==	Rs.109531/=

Add:10% Ceiling	Rs.10953/=

Add: Cartage	Rs.23430/=

Total==	Rs.143914/=

Say		Rs.144000/=

1X5X144000=720000/=

Sub-Engineer				                 Municipal Engineer

Town Committee Dhoronaro			Municipal Committee Umerkot

MATERIEL STATEMENT

| S/No | Item | Qty | S/Ballast | S/Bajri | Hill/Sand | Cement | Brick | Steel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C.C Brick or ston Ballast | 141.0 | 135.0 | - | 68.0 | 14.0 | - | - |
| 2 | Pucca brick work in foundation | 302.0 | - | - | 78.0 | 10.0 | 4077.0 | - |
| 3 | C.C Plain 1:2:4 | 43.0 | - | 38.0 | 19.0 | 7.0 | - | - |
| 4. | R.C.C 1:2:4 | 61.0 | - | 54.0 | 37.0 | 10.0 | - | 4.22 |
| 5. | Cement Plaster ½”thick | 175.0 | - | - | 6.0 | 2.0 | - | - |
|  |  | Say | 135.0 | 92.0 | 198.0 | 43.0 | 4077.0 | 4.22 |

| 7 | Carriage of 100 cft / 5 tons of all material like stone aggregate spawal coal, lime surkhi etc.B.G Rail fastening points & crossing bridges, Girders pipes, sheets Railss M.H Bars etc or 1000 Nos Bricks 10”x5”x3” or 1000 Nos tiles 12”x6”x2” or 150 cft of timber, or 100 Mounds of fuels wood by trucks or any other means wounded by the contractor (Sch: of carriage of material item No.1 Page No.1) (Lead 3 miles) |
| --- | --- |

1.	Stone Ballast from Shah Nihal to site of wor (lead  95 miles)

135.0	Cft		@ Rs.3669/80		P%Cft				Rs.4954/=

2.	Stone Bajri from Khanote to site of work (lead	123 miles)

92.0	Cft		@ Rs.4581/48		P%Cft				Rs.4215/=

3.	Hill Send from Bholari to site of work (lead 103 mile)

198.0	Cft		@ Rs.3930/28		P%Cft				Rs.7782/=

4.	Cement from Zeal Pak cement factory to site or work(lead 92 mile)

43.0	Bags		@ Rs.62/13			P.Bag				Rs.2646/=

5.	Bricks from Killin to site of work (lead 17 miles)

4077.0Nos		@ Rs.904/09			P%Nos			Rs.3686/=

6.	Steel from Hyderabad to site of work (lead 89 miles)

4.22	Cwt		@ Rs.34/74			P%Cwt			Rs.147/=

Total==	Rs.23430/=

Sub-Engineer				         Municipal Engineer

Town Committee Dhoronaro		Municipal Committee Umerkot