S.No.54

PART – IV

Scheduled to Bid

OFFICE OF THE TOWN COMMITTEE DHORONARO

NAME OF WORK: 	CONSTRUCTION OF WATER TANK KANO KOLHI & WASHANO KOLHI DHORONARO TOWN WARD NO 01

Issued to M/s		____________________________________________

BILL OF QUANTITIES

(A)    Description and rates of Items based on Composite Schedule of Rates.

| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 01 | 4406.0 | Excavation for tanks and reservoir in soft Soil i/c trimming and dressing sides to true alignment/ design correct/ profiles and shape, leveling of beds of trenches to correct level grade i/c laying of earth in 6” layers of Construction of tanks and dressing and disposal of surplus Excavated earth within one chain as directed by Engineer Incharge i/c Providing Fence guard, lights, flags where ever required lift upto 5 feet (1.52mm) and lead upto One chain (30.5mm)( PHSI. NO. 1(ii) P-67) | 3000/= | P%0 Cft | Rs.13218/= |
| 02 | 728.0 | Excavation for tanks and reservoirs in Wet soil clay or mud i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards light flags and temporary crossing for non vehicular traffic where ever required lit upto 5 ft (1.52m) and lead upto one chain (30.5m) ( PHSI No. 14 P-74) | 4800/= | P%0 Cft | Rs.3494/= |
| 03 | 360.0 | Excavation for Pipe line in trenches and pits in soft soils i/c timing and dressing sides to true alignment and shape of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth with a one chain as directed by Engineer In charge Providing fence guards lights flags and temporary crossing for non –vehicular l traffic where ever required lift upto 5 ft (1.52)m and lead upto one chain (30.5m) (PHSI No.1 P-60 | 3600/= | P%0 Cft | Rs.1296/= |
| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| 04 | 685.0 | Excavation in foundation building of Bridge and other structure i/c dage belling dressing refilling around structure with excavated earth watering and ramming lead upto 5’ ft on ordinary soil (GSI No. 18/ b- page -5). | 3176/25 | P%0 Cft | Rs.2176/= |
| 05 | 792.0 | Cement concrete plain I/c placing compacting finishing and curing complete (i/c screening & washing of stone aggregate without shuttering) (GSI No. 5/ b-page -16) | 11288/75 | P% Cft | Rs.89406/= |
|  | 459.0 | Ratop (1:2:4) | 14429/25 | P% Cft | Rs.66230/= |
| 06 | 6171.0 | Pacca brick work in foundation and plinth in cement sand mortar (1:6) (GSI No. 4/e P-21) | 11948/36 | P% Cft | Rs.80173/= |
| 07 | 142.0 | Damp proof course with cement sand ans shingle 1:2 4 i/c two coats of asphalt mixture (GSI NO. 27 /c P- 22). 2“ Thick | 3912/85 | P% Sft | Rs.5556/= |
| 08 | 318.0 | Pucca work other than building i/c striking of joints upto 20' ft height in cement mortor (1:6) (GSI NO. 7/c P-25) | 12346/65 | P% Sft | Rs.39262/= |
| 09 | Part-A 643 | Cement plaster ( 1:6) upto 12 ' ft height ½ " thick (GSI No. 11 P- 57) | 2283/93 | P% Sft | Rs.58743/= |
|  | Part-B 2572.0 |  |  |  |  |
| 10 | 2572.0 | Cement plaster ( 1:4) upto 12 ' ft height 3/8" thick (GSI No. 12 P- 57). | 2197/52 | P% Sft | Rs.56520/= |
| 11 | 12.0 | Making & fixing steel grated door with 1/16” thick sheeting i/c angle iron frame 2”x2” 3/8” and ¾” square bars 4” centre to centre with locking arrangement (GSI No.23 P-92) | 726/72 | P% Sft | Rs.8721/= |
| 12 | 36.0 | Providing PVC pipe & class –B and testing with water to head 4.5 ( {PHSI No1 P-22) | 262/= | P.Rft | Rs.9432/= |
| 13 | 324.0 | Refilling the excavated stuff in trenches I/c watering ramming to full compaction etc complete . (P.HSI No. 24 P-53) 90% of excavated earth | 2760/= | P%0 Cft | Rs.894/= |
| 14 | 14.38 | R.CC work i/c labour and material except the cost of steel reinforcement and its labour for bending includes all kinds of forms mould lifting shuttering curing rendering and screening finishing of exposed surface i/c screening and washing of shingle (a) Rcc Work in roofs slabs beams columns rafts lintels and other structural members laid in situ or pre-cost laid in position complete in all respect (I) Ratio (1:2:4) (GSI No.6 P-18) | 337/= | P% Cft | Rs.9564/= |
| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| 15 | 0.562 | Fabrication of Mild Steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also i/c removal of rust from bars (GSI No.7(ii) P-32) | 5001/70 | P.Cwt | Rs.2811/= |
| 16 | 0.590 | Providing fixing C.I main hole cover with frame i/c cost of material ( PHSI No. 1 Page- 23) | 6985/= | P.Cwt | Rs.4121/= |
| 17 | 1929.0 | White Washing 2 coat (GSI No.26/a P-54). | 425/84 | P% Sft | Rs.8214/= |
| 18 | 1929.0 | Distempering 2 coat (GSI No.26/a P-54) | 1043/90 | P% Sft | Rs.20137/= |
| 19 | 24.0 | # Preparing surface and painting of doors and window any type i/c edges. (GSI No.5/c P-76). | 2116/41 | P% Sft | Rs.508/= |
| 20 | 48.0 | Providing and laying HALA or pattern tiles glazed 6” x 6” x ¼” on floor or wall facing in on wall facing in required color and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick including washing and filing of joints with slurry of white cement and pigment in desired shape with finishing cleaning and cost of was polish etc complete including cutting tiles to proper profile. (GSI NO. 61 P-48). | 47651/56 | P% Sft | Rs.22873/= |
| - | - | - | Total==== |  | Rs.492457/= |

| Total Amount | Rs._______________ |
| --- | --- |

| I, M/s ________________________________________ do hereby quotes the rates ________% above / below the rates of CSR in Words: (____________________________) to be added/deducted on the basis of Premium quoted | Rs._______________ |
| --- | --- |
| Total | Rs._______________ |

Contractor 			       Engineer/Procuring Agency