S.No.44

PART – IV

Scheduled to Bid

OFFICE OF THE TOWN COMMITTEE DHORONARO

NAME OF WORK: 	IMPROVEMENT OF ROAD SHAFIQUE RAJPUT KABIN TO GOVERNMENT GIRLS HIGH SCHOOL

Issued to M/s		____________________________________________

BILL OF QUANTITIES

(A)    Description and rates of Items based on Composite Schedule of Rates.

| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 01 | 206.0 | Laying Brick on end edging including supplying 9"x41/2"x3" first class bricks, excavation for laying edging with small size parallel to the road. Rate includes all cost of materials, T&P, labours, and carriage upto site of work.(3 chains) | 2813/32 | P%Rft | Rs.5795/= |
| 02 | 3441.0 | Preparing Base Course by supplying and spreading stone melal of 1 1/2" in gauge of approved quality properly graded in required thickness of 6" to proper camber & grade including supplying and spreading 15 Cft. Screening and non-plastic quarry fines filling depressions with stone metal after initial rolling watering and compacting to achieve 100% density as per modified AASHO. Specification. (Also include providing & using templates, camber plates, Screen forms and carriage uptosite of work. (3 Chains) | 11572/74 | P% Cft | Rs.398218/= |
| 03 | 8343.0 | Providing 1st coat surface dressing on new or existing surface with 4.00 cftof crush bajri of 3/4" - 3/8" in guage in 30 lbs of Bitumen 80/100 penetration I/e cleaning the road surface rolling after coat etc. complete includes all cost of materials T&P carriage upto site of work. | 1700/83 | P% Cft | Rs.141900/= |
| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| 04 | 8343.0 | Providing 1" Thick (consolidated) premixed carpet in proper camber and grade including supplying 14 cft crush bajri, 2 cft hill sand (of approved quality and grade), 67 Ibs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion including heating materials and cleaning the road surface (Hill sand 2 cft for mixing and 2 cft for dusting). Rate includes all cost of materials T&P and carriage upto site of work. | 4709/77 | P% Sft | Rs.392936/= |
| - | - | - | Total==== |  | Rs.938849/= |

| Total Amount | Rs._______________ |
| --- | --- |

| I, M/s ________________________________________ do hereby quotes the rates ________% above / below the rates of CSR in Words: (____________________________) to be added/deducted on the basis of Premium quoted | Rs._______________ |
| --- | --- |
| Total | Rs._______________ |

Contractor 			       Engineer/Procuring Agency