.S.No:47

PART – IV

Scheduled to Bid

OFFICE OF THE TOWN COMMITTEE CHHORE

NAME OF WORK: 	CONSTRUCTION OF COMPOUND WALL FOR GRAVEYARD CHHORE TOWN WARD NO.01

Issued to M/s		____________________________________________

BILL OF QUANTITIES

(A)    Description and rates of Items based on Composite Schedule of Rates.

| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 01 | 2429.0 | Excavation in foundation of building bridges and other structures i/c dag belling dressing refilling earth around structure with excavated earth watering and ramming lead upto 5’ ft ( in ordinary soil) (G.S.I No. 18 P-4). | 3176/25 | P%0 Cft | Rs.7715/= |
| 02 | 484.0 | Cement concrete Brick or stone Ballast ½” to 2” guage (ratio 1:4:8) (GSI No.4/b P-15) | 9416/28 | P% Cft | Rs.45575/= |
| 03 | 910.0 | Pacca brick work in foundation and plinth in cement sand mortar Ratio (1:6) (G.S.I.No. 4/e P-20). | 11948/36 | P% Cft | Rs.72885/= |
| 04 | 404.0 | RCC work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) (G.S.I No.6 P.No.17).. | 337/= | P- Cft | Rs.136232/= |
| 05 | 16.23 | Fabrication of Mild Steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also i/c removal of rust from bars (GSI No.8(a) P-17). | 5001/70 | P. Cwt | Rs.81178/= |
| 06 | 808.45 | Pacca brick work other than building including striking of joints upto 20 feet height in 1:4(G.S.I No.7 P.No.22) | 12899/70 | P% Cft | Rs.104230/= |
| 07 | 50.0 | Making & fixing Steel gravted door with 1/16” thick sheeting i/c iron frame 2”X2”X3/8” & ¾” sq bar i/c with locking arrangement (GSI NO. 24 P-92). | 726/72 | P-Sft | Rs.36336/= |
| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| 08 | 5093.0 | Cement plaster 1:6 upto 12” height ½” thick (GSI No.13 Page No. 52). | 2206/60 | P% Sft | Rs.112381/= |
| 09 | 5093.0 | Cement plaster (1:4) upto 12’ height (a) ½“ 3/8” thick (GSI No.11 Page No.52). | 2197/52 | P% Sft | Rs.111920/= |
| 10 | 5093.0 | White washing (c) two coats (S.I No.26 Page No.53) | 425/52 | P% Sft | Rs.21672/= |
| 11 | 5093.0 | Distempering (b ) two coats (.G.S.I No.24 PageNo.54) | 1043/90 | P% Sft | Rs.53166/= |
| 12 | 100.0 | Preparing surface and painting corrugated surface patent roofing (G.S.I No.5 P.No.69) | 1489/68 | P% Sft | Rs.1490/= |
| - | - | - | Total==== |  | Rs.784780/= |

| Total Amount | Rs._______________ |
| --- | --- |

| I, M/s ________________________________________ do hereby quotes the rates ________% above / below the rates of CSR in Words: (____________________________) to be added/deducted on the basis of Premium quoted | Rs._______________ |
| --- | --- |
| Total | Rs._______________ |

Contractor 			       Engineer/Procuring Agency