SCHEDULE-B	CONSTRUCTION   OF   GOVERNMENT DISPENSARY AT VILLAGE ZAFAR ALI HISBANI TALUKA   FAIZ GUNJ   DISTRICT KHAIRPUR.

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

MAIN BUILDING

01/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty: 1910 Cft	@RS: 3176.25		% Cft	RS: 6066/-

|  |  |  | Thirty one hundreds seventy six rups twenty five ps: |  |
| --- | --- | --- | --- | --- |

02/-       	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.14)

Qty: 389 Cft	@RS: 8694.95		% Cft	RS: 33823/-

|  |  |  | Eighty six hundreds ninety four rups ninety five ps: |  |
| --- | --- | --- | --- | --- |

03/-	Un- Coursed rubble masonry dressing dry masonry (S.I No:1 P-27).

Qty: 875 Cft	@RS: 17723/-		% Cft	RS: 155076/-

|  |  |  | seventeen thousands seven hundreds twenty three rupees.: |  |
| --- | --- | --- | --- | --- |

04/-	Coursed Rubble Masnory 1:6 including hammer dressing (S.I No: 2 d P.26).

Qty: 627 Cft	@RS: 25321/-		% Cft	RS: 158763/-

|  |  |  | Twenty five thousands three hundreds twenty one rupes: |  |
| --- | --- | --- | --- | --- |

05/-	Pacca Brick work in foundation / Plint in cement

cement  & sand  mortar  ratio (1:6)(S.I No:05 (c)P.No:20)

Qty: 198 Cft	@RS: 11948.36  	% Cft	RS:23658/-

|  |  |  | Eleven thousands nine hundreds fourty eight rupes thirty six ps:: |  |
| --- | --- | --- | --- | --- |

06/-	Supplying Sand under floor plugging into wall (S.I NO: 29 P.NO: 26)

Qty: 1919 cft	@RS: 1141.25		% Cft	RS: 21901/-

07/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) .(S,I No:06 P-19).

Qty:  1246cft	@RS: 337/-		P.Cft	RS: 419902/-

|  |  |  | Three hundred thirty seven rupes |  |
| --- | --- | --- | --- | --- |

08/-	Fabrication of mild steel reinforcement for 	cement Concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a)  P.No.20)

Qty: 55.62cwt 	 @RS:5001.70		P.cwt	RS:278195/-

|  |  |  | Five thousands one rupees seventy ps: |  |
| --- | --- | --- | --- | --- |

09/-	Pacca  Brick  work in Ground Floor   in cement sand mortarRatio (1:6) (S.I No:5 P-25).

Qty: 2253 Cft	@RS: 12674.65	 % Cft	RS: 285553/-

(Tweleve thousands six hundred seventy four rupees, sixty five ps.)

10/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (b) P.No.14)

Qty: 260 cft	@RS: 8122.95		% Cft	RS: 21120/-

Eighty one hundreds twenty two rupees ninty five ps:

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

11/-	Cement Concrete plain including placing Compacting

Finishing & curing, complete 	i/c screening & washing

Of stone aggregate Ratio (1:2:4) (S.I No:5 P-18).

Qty: 134 cft	@RS: 14429.25	% Cft	RS:19335/-

( fourteen thousands four hundreds twenty nine rupes twenty five ps:)

12/-	Providing & Laying tiles 6”x6”x1/4” on floor on wall facing in required color and pattern of side specification jointed in white cement and pigment over a base 1:2 grey cement mortar ¾” i/c washing and filling of joints slurry  of white cement and pigment in desired  shape with finishing cleaning and coats polishing etc complete (S.I No:   P.NO:    )

Qty: 775 sft	@RS: 30509.77	% Sft	RS: 236450/-

( THIRTY  thousands FIVE hundred  nine rups: seventy seven ps.)

13/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. (S.I NO: 13(b)    P: NO:52   )

Qty: 4803 Sft	@RS: 2206.60		% Sft	RS: 105983/-

|  |  |  | Twenty two hundreds six rupes sixty ps: |  |
| --- | --- | --- | --- | --- |

14/-	Cement Plaster 1:4 up to 20”-0 heights 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 4803 Sft	@RS: 2197.52		% Sft	RS: 105547/-

|  |  |  | Twenty One hundreds ninety seven rupes fifty two ps: |  |
| --- | --- | --- | --- | --- |

15/-	Cement Pointing struck joints on wall raio 1:6 (P.NO: 19 P-59).

Qty: 789 sft	@RS: 1287.44		% Sft	RS: 10158/-

|  |  |  | Twelve hundreds eighty deven rupees fourty four ps: |  |
| --- | --- | --- | --- | --- |

16/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 28 Page No. 93)

Qty: 373 Rft	@RS: 228/90		P.Rft	RS: 85380/-

(Two hundred twenty eight rupees, ninty ps.)

17/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.(S.I No: 54 P.NO:  65).

Qty: 410 Sft		@RS:902.92		P.Sft	RS: 370201/-

Nine hundreds two rupees ninty two ps:

18/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for window using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 28 Page No. 93)

Qty: 130 Rft	@RS: 240.50		P.Rft	RS: 23465/-

(Two hundred fourty rupees, fifty ps.)

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

19/-	Providing & Fixing Wire gauze fixed to chowakats etc complete.

Qty: 32 Sft	@RS: 190.72		P.Sft	RS: 6103/-

One hundreds ninty rupees seventy two ps:

20/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing and dividing into panels.  (S.I.No.16 (d) P.No.42).(2” thick).

Qty: 1546 sft	@RS: 3275.50	% Sft	RS: 50639/-

Thirty two hundreds seventy five rupees fifty ps:

21/-	White washing 03 three coats (S.I No:    P.NO:    ).

Qty:1154  Sft	@RS: 829.95		% Sft	RS: 9578/-

Eight hundreds twenty nine rupees ninty five ps:

22/-	Colour washing two 02 coats etc

Qty:789 Sft	@RS: 859.90		% Sft	RS: 6785/-

Eight hundreds fifty nine ruees ninty ps:

23/-	Priming coat of Chalk under distemper (S.I NO:23 P-53).

Qty: 4803 Sft	@RS: 442.75		P.Sft	RS: 21265/-

Four hundreds fourty two rupees seventy five ps:

24/-	Distempering 02 wo coats (S.INO:24(b)P.53).

Qty: 4803 Sft	@RS: 1043.90		% Sft	RS; 50139/-

Ten hundreds fourty three rupees ninty ps:

25/-	Painting doors & Windows any type including edges of three coats (S.I NO:40 ii P 67).

Qty: 820 Sft	@RS:1662.21		% Sft	RS: 13630/-

Sixteen hundreds sixty two rupes twenty one ps:

==================================

Total						RS: 2394600/-

______% Above / Below (Amount to be added as per/deducted)RS:___________

G.Total					RS:

Total Amount in word:________________________________________________

__________________________________________________________________

Note:- 	Quantities ans rates may be changed as per/according to the Techinical Sanction vy the competent authority.

EXECUTIVE  ENGINEER

(BUILDINGS) WORKS & SERVICES

DEPARTMENT  KHAIRPUR

MATERIAL STATEMENT

| S# | NAME OF ITEM | QTY: | CEMENT | H/SAND | BAJRI | STONE | BRICKS | Steel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C.C 1:5:10 | 389 | 30 | 187 | - | 381 | - | - |
|  | Coursed 1:6 | 1502 | 18 | 438 | - | 1502 | - | - |
|  | R.C.C 1:2:4 | 1246 | 219 | 548 | 1096 | - | - | - |
|  | Fabrication61/20 | 2.785 | - | - | - | - | - | 2.785 |
|  | P.B.Work 1:6 | 2451 | 74 | 629 | - | - | 33089 | - |
|  | C.C 1:6:12 | 260 | 17 | 127 | - | 255 | - | - |
|  | C.C 1:2:4 | 134 | 24 | 59 | 118 | - | - | - |
|  | Cement Plaster 1:6 | 4803 | 25 | 192 | - | - | - | - |
|  | Cement Plaster 1:4 | 4803 | 27 | 144 | - | - | - | - |
|  | Cement Pointing 1:3 | 789 | 06 | 16 | - | - | - | - |
|  | P/L C.C Topping | 1546 | 46 | 108 | 227 | - | - | - |
|  | Tiles | 789 | 14 | 41 | - | - | - | - |
|  |  | Total | 500 | 2640 | 1446 | 2441 | 33089 | 0.142 |
|  |  | Rate | 80.13 | 5117.15 | 3148.34 | 2599.39 | 773.84 | 603.72 |
|  |  | Unit | P.beg | % cft | % 0 Nos | % CFT | % cft | P.T |
|  |  | Amount | 40065 | 135090 | 45532 | 63451 | 25606 | 85 |

Total		RS.309829/-

ASSISTANT  ENGINEER				EXECUTIVE  ENGINEER

(BUILDINGS)  SUB-DIVISION		     	       (BUILDINGS) WORKS & SERVICES

KOT DIJI					   DEPARTMENT  KHAIRPUR

PART-B DRY LATRINE 02 NOS 	SCHEDULE-B-

| Sr | Item of work | NO:.L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

01/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty: 138 Cft	@RS:3176.25		% Cft	RS:438/-

Thirty one hundred seventy six rups:twenty five ps:

02/-       	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.14)

Qty: 46 Cft	@RS: 8694.95		% Cft	RS:4000/-

(Eight six hundreds ninty four rupees,ninty five ps:)

03/-	Pacca Brick work in foundation / Plint  in cement

cement  & sand  mortar  ratio (1:6)(S.I No:05 (c)P.No:20)

Qty: 40 Cft	@RS: 11948.36  	% Cft	RS:4779/-

|  |  |  | Eleven thousands nine hundreds fourty eight rupes thirty six ps:: |  |
| --- | --- | --- | --- | --- |

04/-	Coursed Rubble Masnory 1:6 including hammer dressing (S.I No: 2 d P.26).

Qty: 62 Cft	@RS: 25321/-		% Cft	RS: 15699/-

|  |  |  | Twenty five thousands three hundreds twenty one rupes: |  |
| --- | --- | --- | --- | --- |

05/-	Damp Proof Course with cement sand and shingle cemncrete 1:2:4 i/c 2 coats of asphalted mixture (S.I NO: 27 P-230).

Qty: 47 Sft	@RS: 4982.18		% Sft	RS: 2342/-

Fourty nine hundreds eighty two rupees eighteen ps:

06/-	Supplying Sand under floor plugging into wall (S.I NO: 29 P.NO: 26)

Qty: 104 cft	@RS: 1141.25		% Cft	RS: 1187/-

Eleven hundreds fourty one rupees twenty five ps:

07/-	Pacca  Brick  work in Ground Floor   in cement sand mortarRatio (1:6) (S.I No:5 P-25).

Qty: 140 Cft	@RS: 12674.65	 % Cft	RS: 17744/-

(Tweleve thousands six hundred seventy four rupees, sixty five ps.)

08/-	Supplying  & Fixing T-Iron (2’x2’x1/4”) Material Schedule.

[						Qty: 1.0 cwt  @RS: 3775/-		P.cwt	RS: 3575/-

Thirty seven hundreds seventy five rupees

09/-	Supplying & Fixing R.S Girder  (4”x8”) Material Schedule .

Qty: 1.74 cwt  @RS: 3850/-		P.cwt	RS: 6699/-

Thirty eight hundreds fifty rupees

10/-	Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 34 Lbs of hot bitumen coating sand blinded provided over one layer of 12”x6”x2” tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc.S.I.NO. 2 Page No. 33).

Qty: 67 Sft	@RS: 7607.25		% Sft	RS: 5097/-

Seventy Six hundreds seven rupees twenty five ps:

11/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) .(S,I No:06 P-19).

Qty:  03cft	@RS: 337/-		P.Cft	RS: 1011/-

Three hundreds thirty seven rupees

12/-	Fabrication of mild steel reinforcement for 	cement Concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a)  P.No.20)

Qty: 0.16  cwt 	 @RS:5001.70		P.cwt	RS:800/-

Five thousands one rupe seventy ps:

13/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 28 Page No. 93)

Qty: 47 Rft	@RS: 228/90		P.Rft	RS: 10758/-

Two hundreds twenty eight rupees ninty ps:

14/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.(S.I No: 54 P.NO:  65).

Qty: 33 Sft	@RS:902.92		P.Sft	RS: 29797/-

Nine hundreds two rupees ninty two ps:

15/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (b) P.No.14)

Qty: 25 cft	@RS: 8122.95		% Cft	RS: 2031/-

Eighty one hundreds twenty two rupees ninty five ps:

16/-	Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing and dividing into panels.  (S.I.No.16 (d) P.No.42).(2” thick).

Qty: 83 sft	@RS: 3275.50	% Sft	RS: 2719/-

Thirty two hundreds seventy five rupees fifty ps:

17/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. (S.I NO: 13(b)    P: NO:52   )

Qty: 450 Sft	@RS: 2206.60		% Sft	RS: 9930/-

Twenty two hundreds six rupees sixty ps:

18/-	Cement Plaster 1:4 up to 20”-0 heights 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 450 Sft	@RS: 2197.52		% Sft	RS: 9889/-

Twenty one hundreds ninty seven rupees fifty two ps:

19/-	White washing 03 three coats (S.I No:    P.NO:    ).

Qty:450  Sft	@RS: 829.95		% Sft	RS: 3735/-

Eight hundreds twenty nine rupees ninty five p:

20/-	Cement Pointing struck joints on wall raio 1:6 (P.NO: 19 P-59).

Qty: 291 sft	@RS: 1287.44		% Sft	RS: 3746/-

Tweleve hundreds eight seven rupees fourty four ps:

21/-	Painting doors & Windows any type including edges of three coats (S.I NO:40 ii P 67).

Qty: 82 Sft	@RS:1662.21		% Sft	RS: 1363/-

Sixteen hundreds sixty two rupees twenty one ps:

==================================

Total						RS: 137339/-

______% Above / Below (Amount to be added as per/deducted)RS:___________

G.Total					RS:

Total Amount in word:________________________________________________

__________________________________________________________________

Note:- 	Quantities ans rates may be changed as per/according to the Techinical Sanction vy the competent authority.

EXECUTIVE  ENGINEER

(BUILDINGS) WORKS & SERVICES

DEPARTMENT  KHAIRPUR

MATERIAL STATEMENT

| S# | NAME OF ITEM | QTY: | CEMENT | H/SAND | BAJRI | STONE | BRICKS | Steel | Earth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C.C 1:5:10 | 46 | 04 | 23 | - | 45 | - | - | - |
|  | P.B Work 1:6 | 180 | 06 | 45 | - | - | 2430 | - | - |
|  | Coursed 1:6 | 62 | 03 | 21 | - | 62 | - | - | - |
|  | D.P.C 1:2:4 | 49 | 02 | 05 | 11 | - | - | - | - |
|  | R.C.C 1:2:4 | 03 | 01 | 01 | 03 | - | - | - | - |
|  | C.C 1:6: ½ | 25 | 02 | 12 | - | 25 | - | - | - |
|  | P/L C.C Topping 2” | 83 | 02 | 06 | 12 | - | - | - | - |
|  | Cement Plaster 1:6 | 450 | 03 | 18 | - | - | - | -- | - |
|  | Cement Plaster 1:4 | 450 | 03 | 14 | - | - | - | - | - |
|  | Cement Pointing 1:3 | 297 | 01 | 06 | - | - | - | - | - |
|  |  | Total | 29 | 152 | 26 | 132 | 2430 | - | - |
|  |  | Rate | 80.13 | 5117.15 | 3148.34 | 2599.39 | 773.84 | 603.72 | - |
|  |  | Unit | P.beg | % cft | % 0 Nos | % CFT | % cft | P.T | % sft |
|  |  | Amount | 2323 | 7778 | 819 | 3431 | 1880 | 16231 |  |

Total		RS.16231/-

ASSISTANT  ENGINEER			EXECUTIVE  ENGINEER

(BUILDINGS)  SUB-DIVISION		     	       (BUILDINGS) WORKS & SERVICES

KOT DIJI					   DEPARTMENT  KHAIRPUR

PART-C COMPOUND WALL

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

01/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty: 1295 Cft	@RS:3176.25		% Cft	RS:4112/-

Thirty one hundred seventy six rups:twenty five ps:

02/-       	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.14)

Qty: 264 Cft	@RS: 8694.95		% Cft	RS:22955/-

Eighty six hundreds ninty four rupees ninty five ps:

03/-	Pacca Brick work in foundation / Plint  in cement

cement  & sand  mortar  ratio (1:6)(S.I No:05 (c)P.No:20)

Qty: 450 Cft	@RS: 11948.36  	% Cft	RS:53768/-

Eleven thousands nine hundreds fourty eight rupees thirty six ps:

04/-	Coursed Rubble Masnory 1:6 including hammer dressing (S.I No: 2 d P.26).

Qty: 375 Cft	@RS: 25321/-		% Cft	RS: 94954/-

Twenty five thousands three hundreds tweny one rups:

05/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) .(S,I No:06 P-19).

Qty:  184cft	@RS: 337/-		P.Cft	RS: 62008/-

Three hundred thirty three rupees

06/-	Fabrication of mild steel reinforcement for 	cement Concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a)  P.No.20)

Qty: 8.214  cwt  @RS:5001.70	P.cwt	RS: 41084/-

Five thousands one rupees seventy ps:

07/-	Pacca Brick work in Other than building 1:6 (S.I No:7 eP-21).

Qty: 641 cft	@RS: 12346.65	% Cft	RS: 79142/-

Tweleve thousands three hundreds fourty siz rupees sixty five ps:

08/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. (S.I NO: 13(b)    P: NO:52   )

Qty: 1034 Sft	@RS: 2206.60		% Sft	RS: 22816/-

Twenty two hundreds six rupees sixty ps:

09/-	Cement Plaster 1:4 up to 20”-0 heights 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 1034 Sft	@RS: 2197.52		% Sft	RS: 22722/-

Twenty one hundreds ninety seven rupees fifty two ps:

10/-	Cement Pointing struck joints on wall raio 1:6 (P.NO: 19 P-59).

Qty: 800 sft	@RS: 1287.44		% Sft	RS: 10300/-

Twelve hundreds eighty seven rupees fourty four ps:

11/-	Colour washing two 02 coats etc

Qty: 1034 Sft	@RS: 859.90		% Sft	RS: 8891/-

Eight hundreds fifty nine rupees ninty ps:

12/-	Making & Fixing  Steel grated door with 1/16” thick sheeting  including angle iron frame 2”x2”x3/” and square bars 4” center to center with locking arrangement .(S.I NO:24)P.NO:92

Qty: 60 sft	@RS: 726.72		P.Sft	RS: 43603/-

Seven hundreds twenty six rupees seventy two ps:

13/-	Painting guard bars gets iron bars grating , railing including standard braces (etc) and similar open work. (S.I NO:4(d)P.NO:69).

Qty: 120 Sft	@RS: 674.61		% Sft	RS: 810/-

Six hundreds seventy four rupees sixty one ps:

14/-	Providing & Laying HALA or pattern tiles glazed 6”x6”x1/4” on floor or wall facing in required color & pattern od STILE specification jointed in white cement & pigment over a base of 1:2 grey cement mortar ¾” thick including washing & filling or joints with slurry of  white cement and pigment in desired shape with finishing cleaning and cost of wax polish etc. complete including cutting tiles to proper profile (S.I No:61 P.NO:480)

Qty: 32 Sft	@RS: 47651.56	% Sft	RS: 15248/-

Fourty seven thousands six hundreds fifty one rupees fifty six ps:

==================================

Total						RS:482413/-

______% Above / Below (Amount to be added as per/deducted)RS:___________

G.Total					RS:

Total Amount in word:________________________________________________

__________________________________________________________________

Note:- 	Quantities ans rates may be changed as per/according to the Techinical Sanction vy the competent authority.

EXECUTIVE  ENGINEER

(BUILDINGS) WORKS & SERVICES

DEPARTMENT  KHAIRPUR

MATERIAL STATEMENT

| S# | NAME OF ITEM | QTY: | CEMENT | H/SAND | BAJRI | STONE | BRICKS | Steel | Earth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C.C 1:5:10 | 264 | 20 | 12 | - | 258 | - | - | - |
|  | P.B Work 1:6 | 1091 | 37 | 280 | - | - | 14728 | - | - |
|  | Coursed 1:6 | 375 | 17 | 128 | - | 375 | - | - | - |
|  | R.C.C 1:2:4 | 184 | 32 | 81 | 162 | - | - | - | - |
|  | Fabrication | 8.214/20 | - | - | - | - | - | 0.410 | - |
|  | Cement Plaster 1:6 | 1034 | 5 | 41 | - | - | - | - | - |
|  | Cement Plaster 1:4 | 1034 | 06 | 31 | - | - | - | - | - |
|  | Cement Pointing 1:3 | 1034 | 04 | 21 | - | - | - | - | - |
|  |  | Total | 121 | 711 | 162 | 633 | 14728 | 0.410 | - |
|  |  | Rate | 80.13 | 5117.15 | 3148.34 | 2599.39 | 773.84 | 603.72 |  |
|  |  | Unit | P.beg | % cft | % 0 Nos | % CFT | % cft | P.T | % sft |
|  |  | Amount | 9535 | 37456 | 5101 | 6576 | 11397 | 248 | - |

Total		RS.70313/-

ASSISTANT  ENGINEER			EXECUTIVE  ENGINEER

(BUILDINGS)  SUB-DIVISION		     	       (BUILDINGS) WORKS & SERVICES

KOTDIJI					   DEPARTMENT  KHAIRPUR

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

PART-    WATER SUPPLY

1/-	P/F squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth & floor for pipe connections and making good in

Cement concrete 1:2:4. (A) W.C pan of not less than 23” clear opening between flashing rims and  3 gallons flushing tank. With 4” dia C.I. tape. (S.I.NO.1(A) P.No.1)

Qty: 03 No	@RS: 4846.60	 each	RS: 14540/-

2/-	P/F 22”x16” lavatory basin in white glazed earthen ware complete 	with & i/c the cost of W.I or C.I 	cantilever brackets 6” built into 	wall 	painted white in two coats after a primary coat of red lead 	paint a pair of ½” dia chrome plate pillar traps 1-1/2” dia rubber plug 	& chrome plated brass 	chain 	1-1/4” dia 	malleable iron or 	c.p brass traps malleable iron or brass union & making 	requisite 	number of holes in walls plinth & floor for pipe connection and 	making good in 	cement concrete 	1:2:4 (standard pattern). 	Add extra for labour for providing & fixing of 	earthenware 	pedestal white 	or coloured glazed (Standard pattern) .  	(S.I.NO. 8 P. No.3)

Qty: 01 No	@RS: 4928/-	each	RS: 4928/-

3/-	Providing G.I pipes special & clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (Internal) (i) ½ dia G.I pipe. (S.I.NO.1 (i) P. No.11)

“	“	1 ¼” dia			Qty: 30 Rft	@RS: 188.97	P.Rft	RS: 5669/-

“	“	1” Dia				Qty: 20 Rft	@RS: 128.55	P.Rft	RS: 2571/-

“	“	¾” dia				Qty: 50 Rft	@RS: 95.79	P.Rft	RS: 4790/-

“	“	½” Dia				Qty: 75 Rft	@RS: 73.21	P.Rft	RS: 5491/-

4/-	Add: Extra for labour  G.I Pipe and fitting etc complete (S,I NO: 2 P-12).

Qty: 40 Rft	7.82		P.Rft	RS: 313/-

5/-	P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections & making good in CC 1:2:4. (S.I. No. 20 P. No.5)

Qty: 02 NOs	@RS: 2024.43	each	RS: 4049/-

| Sr | Item of work |  | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

5/-	Supplying and fixing concealed stopcock of superior quality with c.p head ½” dia.(S.I.NO.13 (a) P. No.15)

Qty: 01 No	@RS: 889.46	each	RS: 889/-

6/-	P/F Chrome plated brass towel rail complete with brakes fixing on wooden etc (S.I NO: 1 P-7)

Qty: 01 No	@RS: 1269.95	each	RS: 1270/-

7/-	Providing & fixing 15" x 12" bavelled edge mirror of belgium glass complete with 1/8" thick hard board and c.p screws fixed to wooden pleat.

Qty: 01 No	@RS: 1161.60	each	RS: 1162/-

8/-	Providing & fixing in position nyloon connections  complete with 1/2" dia, brass stop cock with pair of brass nuts and lining joints to nyloon connection.

Qty: 05 Nos	@RS: 447.15	each	RS: 2236/-

9/-	S/F Bib cock superior quality with C.P head  ½” dia (S.I NO: 13 P-19).

Qty: 04 Nos	@RS: 1109.46	each	RS: 4438/-

10/-	S/F Cancelead stop cock of superioir quality (S.I NO: 3 b P-15).

Qty: 02 NOs	@RS: 507.47	each	RS: 1019/-

11/-	S/F handle valve china ¾” Dia (S.I NO: 4 P-17).

Qty: 02 NOs	@RS: 271.92	each	RS: 544/-

12/-	Supplying & fixing Fiber Glass Tank of approved quality and design and wall thickness as specified i/c cost of nuts bolts and fixing in plate form of cement concrete 1:3:6 and making connection for in-let outlet & over flow in-let outlet flow pipes etc. complete. (S.I.No.3 P.No.18) (Rate per gallon Rs: 33/60 for tanks upto capacity of 250 gallons) 200 Gallons

Qty: 01 NO		@RS: 21989.16 each	 RS: 21990/-

13/-	Providing & Fixing R.C.C Pipe Collar glass Band digging the trenches to required depth and fixing in position including cutting , bending & fitting etc complete

6” Dia					Qty: 24  rft		RS: 199.25	P.Rft	RS:4782/-

(one hundred ninty nine rupees twenty five ps:)

14/-	P/F asbestive pipe with collar deed or equivalent i/c digging trencheshes etc (S.I NO: 1 c P23).

Qty: 58 Rft		@RS: 141.11	P.Rft	RS: 8184/-

| Sr | Item of work | No: L:B:D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

15/-	Providing & fixing 6" x 4" C.C gully trap with  4" outlet complete with 4" thick 1: 2: 4 C.C. for bed  & 1/2" thick cement plaster (1:3) to the karb, C.I grating  6" x 6" &  C.I cover  and frame 12" x 12" (inside) etc. complete.

Qty: 03 NOs		@RS: 1193.17	each	RS: 3580/-

=========================================

Total						RS: 92445/-

P.H.S . ITEMS

01/-	Boring for tube well in all water bearing soils from ground level up-to 100 ft or 30.5 meter depth i/c sinking and with drawing of casing pipe. (100 mm 4”) (S.I.No. 01 (b) Page No.36).

Qty: 60 Rft	@RS: 242/-	P.Rft	RS: 14520/-

02/-	Construction of manhole  or inspection chamber  for the required dia

of circular sewer   and 3’-6’ 	(1067 mm ) depth with walls  of 	B.B  in cement mortar  1:3 cement plaster  1:3 ½”  thick in side 1”  	(25 	mm ) thick over  beching  and channel i/c  fixing C.I main 	hole cover with frame of clear 	opening  1 ½”x 1 ½”” 457x457 	mm) of 1.75 cwt . (88.9 kg) embedded in plain c.c 1:2:4) & fixing 	(25 mm) dia M.S steps 6” 150 mm)   white projecting 4” (102 mm) 	from the face of walls at 12” 	(305 mm) c/c duly painted etc 	complete as per standard specification & drawing.(S.I No- P-46).

Qty:02 Nos	@Rs:14748/-			each	RS: 29496/-

03/-	Providing & Fixing PVC  Pipe  Class “D” etc complete

Qty: 40 rft	@RS: 34.			P.Rft	RS: 1360/-

=========================================

Total						RS: 45376/-

E-I ITEm

01/-	Supplying & Fixing water pumping set ½” H.P complete

Qty: 01 No	RS: 3646.59			each	RS: 3647/-

=========================================

Total						RS: 3647

PART-     EXTERNAL DEVELOPMENT

| Sr | Item of work | NO:L.B.D | QTY | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- | --- |

NO==

01/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty: 149 Cft	@RS:3176.25		% Cft	RS:473/-

|  |  |  | Thirty one hundreds seventy six rups twenty five ps: |  |
| --- | --- | --- | --- | --- |

02/-       	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.14)

Qty: 50 Cft	@RS: 8694.95		% Cft	RS:4347/-

|  |  |  | Eighty six hundreds ninety four rups ninety five ps: |  |
| --- | --- | --- | --- | --- |

03/-	Pacca Brick work in foundation / Plint  in cement

cement  & sand  mortar  ratio (1:6)(S.I No:05 (c)P.No:20)

Qty: 127 Cft	@RS: 11948.36  	% Cft	RS:14816/-

|  |  |  | Eleven thousands nine hundreds fourty eight rupes thirty six ps:: |  |
| --- | --- | --- | --- | --- |

04/-	Supplying Sand under floor plugging into wall (S.I NO: 29 P.NO: 26)

Qty: 3498 cft	@RS: 1141.25		% Cft	RS: 43345/-

Eleven hundreds fourty one rupees twenty five ps:

05/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (b) P.No.14)

Qty: 800 cft	@RS: 8122.95		% Cft	RS: 64984/-

Eighty one hundreds twenty two rupees ninty five ps:

06/-	Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing and dividing into panels.  (S.I.No.16 (d) P.No.42).(2” thick).

Qty: 1320 sft	@RS: 3275.50	% Sft	RS: 43236/-

Thirty two hundreds seventy five rupees fifty ps:

07/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. (S.I NO: 13(b)    P: NO:52   )

Qty: 99 Sft	@RS: 2206.60		% Sft	RS: 2185/-

|  |  |  | Twenty two hundreds six rupes sixty ps: |  |
| --- | --- | --- | --- | --- |

08/-	Cement Plaster 1:4 up to 20”-0 heights 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty: 99 Sft	@RS: 2197.52		% Sft	RS: 2176/-

|  |  |  | Twenty One hundreds ninety seven rupes fifty two ps: |  |
| --- | --- | --- | --- | --- |

=========================================

Total						RS:  175562/-

______% Above / Below (Amount to be added as per/deducted)RS:___________

G.Total					RS:

Total Amount in word:________________________________________________

__________________________________________________________________

Note:- 	Quantities ans rates may be changed as per/according to the Techinical Sanction vy the competent authority.

EXECUTIVE  ENGINEER

(BUILDINGS) WORKS & SERVICES

DEPARTMENT  KHAIRPUR

MATERIAL STATEMENT

| S# | NAME OF ITEM | QTY: | CEMENT | H/SAND | BAJRI | STONE | BRICKS | Steel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C.C 1:4:8 | 50 | 04 | 24 | - | 49 | - | -- |
|  | P.B Work 1:6 | 124 | 04 | 32 | - | - | 1674 | - |
|  | C.C 1:6:12 | 800 | 52 | 392 | - | 784 | - | - |
|  | Pavers | 1320 | - | - | - | - | - | - |
|  | Cement Plaster 1:6 | 99 | 01 | 04 | - | - | - | - |
|  | Cement Plaster 1:4 | 99 | 01 | 03 | - | - | - | - |
|  | C.C Topping 2” thick | 1320 | 40 | 98 | 194 | - | - | - |
|  | Extra lead | 6995 | - | - | - | - | - | - |
|  |  | Total | 101 | 616 | 198 | 833 | 1674 | - |
|  |  | Rate | 80.13 | 5117.15 | 3148.34 | 2599.39 | 773.84 |  |
|  |  | Unit | P.beg | % cft | % 0 Nos | % CFT | % cft | P.T |
|  |  | Amount | 8093 | 31521 | 6234 | 21653 | 1295 | - |

Total		RS.68716/-

ASSISTANT  ENGINEER				EXECUTIVE  ENGINEER

(BUILDINGS)  SUB-DIVISION		     	       (BUILDINGS) WORKS & SERVICES

KOT DIJI					   DEPARTMENT  KHAIRPUR

NAME OF WORK:-	CONSTRUCTION   OF   GOVERNMENT DISPENSARY AT VILLAGE FATEH ALI LASHARI TALUKA  KOT DIJI   DISTRICT KHAIRPUR.

GENERAL ABSTRACT

PART-A		MAIN BUILDING		RS: 2688297/-

PART-B		DRY LATERINE		RS: 156619/-

PART-C		COMPOUND WALL		RS: 562105/-

PART-D		W/S & S/F			RS: 141468/-

PART-E		EXTERNAL DEVL:		RS: 236829/-

===============================================

TOTAL							RS: 3785318

Add:    % Contingency					RS: 14682/-

TOTAL							RS: 3800000/-

OR							RS: 3.800	(M)

ASSISTANT  ENGINEER			EXECUTIVE  ENGINEER

(BUILDINGS)  SUB-DIVISION		     	       (BUILDINGS) WORKS & SERVICES

KOT DIJI					   DEPARTMENT  KHAIRPUR

FACE SHEET

FUNDS HEAD:.		__________________________________________

MAJOR HEAD:.		__________________________________________

MINOR HEAH:	CONSTRUCTION   OF   GOVERNMENT DISPENSARY AT VILLAGE FATEH ALI LASHARI TALUKA  KOT DIJI   DISTRICT KHAIRPUR.

This Estimate Has Been Framed  in The Office Of The  Executive Engineer (Buildings) Works & Service Department  Khairpur For Probable  Expenditure That Will Be  Incurred On the Said Works.

Amounting to Rs.:  3.800 (M)