Part (D) Pumping Machinery (At Source)

1.	Supplying and installing in position i/c transportation to site of Improvement Extension & Change Source Water Supply Scheme Hingorno From Mithrao Canal Electric Pumping set of 1450 RPM (Siemens) made 15 BHP Electric motor 3 phase 50 Cycle 400/420 volts with centrifugal pump type ETA-100-33, size 5” x 4” mounted on commons steel frame on base plate (capable of discharge 305 GPM against the required head of 95 ft, i/c installed on cement concrete ratio 1:2:4 Ms nuts bolts of 5/8” dia to be embedded in cement concrete foundation providing fixing MCU star delta starter (Siemens  made) made and providing internal electric wiring for main switch 3-7/100 1” dia PVC insulated wire 660 volts grade (i) buried on the ground at depth of 3.7 m (12 ft) as per Pak PWD specifications and testing pumping set against the required head for 72 hours etc complete (R.A attached).

2.00 Set		@ Rs. 470170/-		P  Set			Rs. 940340/-

2.	Manufacturing and installing 6” dia  M.S pipe made out of M/S sheet (3/16”) thick M.S sheet i/c cutting bending folding to correct curvature as shape (R.A attached).

(i) Suction 6” Dia

42.00 Rft		@ Rs. 749/34			P Rft			Rs. 31472/-

(ii) Delivery 6” Dia

20.00 Rft		@ Rs. 749/39			P Rft			Rs. 14987/-

4.	Manufacturing and installing M.S flanges 3”&4” made out M.S sheet 3/8” thick etc complete.

6” Dia

18.00 Nos		@ Rs. 804/77			Each			Rs. 14486/

5.	C. I sulice valve heavy pattern (test pressure 300 lbs/sq: inch) SOMI No. 2 P-11).

6” Dia

2.00 Nos		@ Rs. 9360/-			Each			Rs. 18720/-

6.	C.I reflex valve heavy pattern (test pressure 300 lbs/square inch (SMOI No. P-11).

6” Dia

2.00 Nos		@ Rs. 4062/50		Each			Rs. 8125/-

7.	C.I foot valve heavy pattern with cone type gate (imported) (SOMI No. 4 P-10).

6” Dia

2.00 Nos		@ Rs. 1381/25		Each			Rs. 2763/-

8.	C. I bend with flanged ends with holes i/c turning and facing of flanges for all size (SOMI No. 8 P-11).

- 6” Dia 900

73.50 Kg		@ Rs. 120/-			P. Kg			Rs.8820/-

9.	C. I tappers flat bottomed or central tapered flanged ends with holes i/c turning and facing of flanges for all sizes (SOMI No. 9 P-11).

21.00 KG		@ Rs. 120/-			P. Kg			Rs. 2520/-

10.	C.I tee with flanged ends with holes i/c turning and facing of flanges for all sizes (SOMI No. 7 P-11).

Tee 6”x6”x6”

21.00 Kg		@ Rs. 120/-			P. Kg			Rs. 2520/-

11.	C.I Short piece (SOMI No. 9 P-15).

6” Dia

1.00 No		@ Rs. 618/75 			Each			Rs. 619/-

12.	Jointing C.I/M.S flanged pipes and specials flange and inside a trench i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joint to the specified pressure etc complete (PHSI No. 1 P-40).

(i) 6” Dia

24.00 Joint		@ Rs. 938/-			P. Joint		Rs. 22512/

13.	Painting New surface

Preparing surface and painting of filled framing skirting pipes gutters similar lincar work not exceeding 4” inch (GSI No. 4 P-69).

62.00 Rft		@ Rs. 686/95			P % Rft		Rs. 426/-

----------------

Total			Rs. 1068310/-

=========